From: tesla-2@emachine.com (Tesla-2)

Subject: Re: whats everyone doing?

Date: Wed, 14 May 1997 01:25:48 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Tesla79@aol.com

Date: Tue, 13 May 1997 21:25:48 -0400 (EDT)

Subject: Re: whats everyone doing?

In a message dated 97-05-13 20:57:55 EDT, you write:

> Hi all

>

>  I noticed the list has been pretty quiet lately.

>  I wonder what everyone is up to?

>

Long time no see Mike!  I still remember you.  You are the one who gave me

invaluable advice about Tesla coil plans.

I haven't been sending e-mail because I was studying for SAT's.  I took the

SAT's May 3.  It went ok..

I'm glad to hear your Tesla coil hobby is going well :)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Safety Chokes

Date: Thu, 15 May 1997 07:47:36 GMT

Organization: The Energy Machine Information System 504-734-0526

From: cave@ldd.net (BRAD BLACKBURN)

Date: Thu, 15 May 1997 02:47:36 -0500

Subject: Safety Chokes

Any suggestions on chokes? I have the specs on my secondary if that helps.

As far as what's going on, I received my poly by UPS the other day so now I

no longer have any excuses. The IRC chat was a good idea, now if we can get

more coilers involved on a nightly basis.

(SAT afternoon is a bad time for me)

Brad Blackburn

cave@ldd.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Schematic questions

Date: Thu, 15 May 1997 17:01:02 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Dave <djb@erie.net>

Date: Thu, 15 May 1997 13:01:02 -0400

Subject: Schematic questions

TESLA COIL SCHEMATIC                                  --------

|      | TC

--------

O

O

SG   SC      FCC     PC   SG        P-COIL     O S-COIL

O---------------OOOOO---------> <-----    O        O

NST  O    |    |              |           |    O        O

O    |    |              |           |    O        O

-----O||O    *   ---            ---          |--->O        O

O||O                                         O        O

-----O||O    *   ---            ---               O        O

110   | O    |    |              |                O        O

Volts | O    |    |              |                O        O

| O---------------OOOOO---------------------|        |

|                                                    |

|                                                    |

Gnd                                                  Gnd

Hi Everyone,

I have a few questions on this simple schematic above. Is this the most

common configuration for small coils...ie less than 1000 watts? Are there

other configurations that might be more advantageous to use, or are there

disadvantages to this setup? Also, what is the purpose of the safety gap? I

can see the chokes would limit the amount of RF kickback, and the safety cap

would act as a short for RF so it doesn't effect the transformer...so why

have a saftey gap, and what would make it fire? And how wide should be the

gap be?

Also could you connect a cap of suitable rating from ground to the bottom of

the secondary before the chokes? I think it would act as an open for 60 hz,

but as a short to ground for RF and protect the power supply, etc. Thanks in

advance for your help.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Newbie...

Date: Thu, 15 May 1997 21:57:28 GMT

Organization: The Energy Machine Information System 504-734-0526

From: RareBear@webtv.net (Scott Staggs)

Date: Thu, 15 May 1997 14:57:28 -0700

Subject: Newbie...

I am new to this list and pretty new to Tesla coils...  I have always

had an interest In Tesla Coils and Nikola Tesla himself..  I have some

Bacground in Electronics, but I am in no way at technician level..  (I

had 2 years of electronics classes in highschool, and did electronics

assembly and light troubleshooting on the job for about a year)

I live in Reno NV, and am 25 years old. I enjoy Photography as a hobby,

as well as model building, Radio (shortwave and scanner) monitoring,

electronics tinkering, And electronics kit-building (mostly radio stuff)

Can somebody point me to a link on the Web that can help me get up to

speed?  I too have more time than money and have to keep my many hobbies

on a tight budget.  I am patient and dont mind taking my time working on

something....

Scott

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Newbie...

Date: Fri, 16 May 1997 00:12:55 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Dave <djb@erie.net>

Date: Thu, 15 May 1997 20:12:55 -0400

Subject: Re: Newbie...

At 09:57 PM 5/15/97 GMT, you wrote:

>From: RareBear@webtv.net (Scott Staggs)

>Date: Thu, 15 May 1997 14:57:28 -0700

>Subject: Newbie...

>

>I am new to this list and pretty new to Tesla coils...  I have always

>had an interest In Tesla Coils and Nikola Tesla himself..  I have some

>Bacground in Electronics, but I am in no way at technician level..  (I

>had 2 years of electronics classes in highschool, and did electronics

>assembly and light troubleshooting on the job for about a year)

>

>I live in Reno NV, and am 25 years old. I enjoy Photography as a hobby,

>as well as model building, Radio (shortwave and scanner) monitoring,

>electronics tinkering, And electronics kit-building (mostly radio stuff)

>

>Can somebody point me to a link on the Web that can help me get up to

>speed?  I too have more time than money and have to keep my many hobbies

>on a tight budget.  I am patient and dont mind taking my time working on

>something....

>

>Scott

Hi Scott,

Welcome to the list. I am new to this list myself. Here are two sites that I

have found informative. Hope they help. Enjoy!

Dave

This page has all sorts of science related items, and among them tesla coils.

http://www.eskimo.com/~billb/

I believe this is the main site where list messages are archived, and

contains various texts, programs, pictures, etc on tesla coils.

http://nic.funet.fi/pub/sci/electrical/tesla/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety Chokes

Cc:

Bcc:

X-Attachments:

At 07:47 AM 5/15/97 GMT, you wrote:

>From: cave@ldd.net (BRAD BLACKBURN)

>Date: Thu, 15 May 1997 02:47:36 -0500

>Subject: Safety Chokes

>

>Any suggestions on chokes? I have the specs on my secondary if that helps.

>As far as what's going on, I received my poly by UPS the other day so now I

>no longer have any excuses. The IRC

>chat was a good idea, now if we can get more coilers involved on a nightly

>basis. (SAT afternoon is a bad time for me)

>

>Brad Blackburn

>cave@ldd.net

>

>

Brad,

The issue of protection chokes is a controversial one

with everybody having their own pet theory and ideas

with little hard evidence one way or another.

That said the idea is to put sufficient inductance

in the HV lines to knock down the level of RF kickbacks

enough to keep the RF from backing up into your neons

and wreaking havoc.

Small chokes will do nothing. To be effective you need

at least 2 millihenries. And I believe that anything

over 10 mh is a waste of wire.

You are not going to completely eliminate the RF level

with series chokes. But you dont need to. Just knock

down the level of the RF so its manageable.

The chokes go in the HV lines between the main gap

and safety gap.

What has worked well for me

is 125 turns of AWG 26 on a large ferrite toroid.

This gives about 2.5 mh and has worked well.

Ferrite rods would work as well.

Lacking ferrite rods or toroids you can try winding

air core chokes on a piece of PVC pipe. About 350

turns of # 28 on a piece of 2 inch PVC will give

about 3 mh of inductance.

Air core chokes can provide their own set of problems.

If wound with many turns of wire they can ring up

like mini Tesla coils and create quite high voltages.

This can cause more problems than it prevents.

The use of a bit of series resistance will kill

the Q of the air core choke and prevent them from

ringing. For the resistance try about 1k ohms

at about 50 watts. Since the current flow through

these is rather low the extra loss caused by the

addition of the resistors is slight.

What we do not want in a RF choke is a high Q low

loss inductor. We want a low Q lossy inefficient

inductor with at least 2 mh of inductance.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: toroidial chokes -Reply

Date: Fri, 16 May 1997 15:33:30 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Kevin Batchelor <KBATCHELOR@arslimited.com>

Date: Fri, 16 May 1997 11:33:30 -0400

Subject: toroidial chokes -Reply

>> I recently got some great looking toroidial core choke coils, but

they're wound with about #16 with about 40-50 turns on them. Will they

have sufficient inductance to keep excessive RF out of my Neon?<<

I'd say put at least two of them in series on each HV run.  I run with one

choke on each run with about 100 turns of #26.  This with my safety gap

seems fine.  I've run my neon hard and opened up my safety quite far.

Kevin

KBATCHELOR@ARSLIMITED.COM

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Copper tubing- How do you... -Reply

Date: Fri, 16 May 1997 15:39:44 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Kevin Batchelor <KBATCHELOR@arslimited.com>

Date: Fri, 16 May 1997 11:39:44 -0400

Subject: Copper tubing- How do you... -Reply

Don't bother covering your tubing.  It's not really necessary as long as

your spacing between turns is over 1/4".  My tubing came in a roll of 50'

so it was already bent into shape ok.  I just had to space it out on my

support forms made of lexan.  You could use wood or plexiglass.  I

found the lexan easier to cut and drill than plexiglass which is quite

brittle.  Either way...coils don't have to be pretty to be functional.

Kevin

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Copper tubing- How do you...

Date: Fri, 16 May 1997 16:45:16 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Fri, 16 May 1997 09:45:16 -0700

Subject: Copper tubing- How do you...

How do you guys get such beautiful smooth coils out of copper tubing?

I've bought a bunch of 1/4" soft copper tubing and I'm having a

terrible time making it bend symmetrically. I need to know how you

guys bend your tubing.

I would also like to run the tubing into some 5/8" aquarium air

hose. to do this I must straighten out the hose and push it through.

then I can bend the copper, but it's still hard to get it to look

pretty, and I know the electrical characteristics are better if the

spiral is smooth and on one plane (for a flat spiral). Give me some

hints!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: toroidial chokes

Date: Fri, 16 May 1997 16:47:10 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Fri, 16 May 1997 09:47:10 -0700

Subject: toroidial chokes

I recently got some great looking toroidial core choke coils, but

they're wound with about #16 with about 40-50 turns on them. Will they

have sufficient inductance to keep excessive RF out of my Neon?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Copper tubing- How do you...

Date: Fri, 16 May 1997 17:42:04 GMT

Organization: The Energy Machine Information System 504-734-0526

From: tesla@america.com (Bob Schumann)

Date: Fri, 16 May 1997 13:42:04 -0400 (EDT)

Subject: Re: Copper tubing- How do you...

>From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

>Date: Fri, 16 May 1997 09:45:16 -0700

>Subject: Copper tubing- How do you...

>

>     How do you guys get such beautiful smooth coils out of copper tubing?

>     I've bought a bunch of 1/4" soft copper tubing and I'm having a

>     terrible time making it bend symmetrically. I need to know how you

>     guys bend your tubing.

>        I would also like to run the tubing into some 5/8" aquarium air

>     hose. to do this I must straighten out the hose and push it through.

>     then I can bend the copper, but it's still hard to get it to look

>     pretty, and I know the electrical characteristics are better if the

>     spiral is smooth and on one plane (for a flat spiral). Give me some

>     hints!

>

I came up with a way that works fine for me and would do it

again and again. If you got a 50 roll of 1/4 inch tubing then

the spiral starts out big, gets smaller toward the center of the

roll and then expands back outward again. It is good to keep this

is mind. Kevin was right in that it does not have to be pretty

to be functional. Here is my method. Keep in mind my primarys are

either flat or inverted cone (mostly inverted cone). Having a

freind around for another set of hands helps.

Find the diameter of your smallest inside turn of your primary.

I found a coffee can that was about THAT size. Place the coffee

can on the floor and place the end of the copper tubing on the

floor up against the coffee can. Using the coffee can as a guide

make your first turn around the coffee can. This first turn is

a little difficult as it is the start of the process. When you

finally get that turn back around to itself then , while keeping

the tubing flat on the floor the whole time, wind the next turns

'ON' the previous turn. This will not only make a continuos

expanding outward wind but will allow you to bend (lets use the

word 'form' - I hate bend) the tubing around the previous turn

and this keeps kinks from happening. The more you work with the

soft copper the more it gets out of whack, so if you just keep

winding the pipe out of the box onto the existing previous turns

you are now just working with it once (the first time). Have a

friend there to hold the previous turns down flat on the floor

as to make the next turn easy. When you get to the center of

the roll and the spiral in the box gets smaller, you will have

to work a little bit to open that small spiral to fit on the

spiral your are forming around the coffee can. When you get done,

you have all of your pipe in a flat spiral on the floor wrapped

around the coffee can and the inner most spiral is about the size

of the inner most turn of your Tesla primary. Now comes the time

to have to work with the copper tubing for the second time. As

you place the tubing in the primary form, from the inside out,

the adjustments you make to the tubing are to make the spiral

just a little bigger to fit. When working with the tubing, it is

MUCH easier to expand the diameter of a turn than it is to make a

turn smaller. Just slowly work your way from the primary center

outward. As your progressive expanding turns occur on your primary,

so do they on the tubing, because you have already pre-formed the

tubing when you wrapped it around the coffee can. I hope I did ok

in trying to explain this.

Bob Schumann

tesla@america.com

http://www.america.com/~tesla

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Copper tubing- How do you... -Reply -Reply

Date: Fri, 16 May 1997 17:57:28 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Kevin Batchelor <KBATCHELOR@arslimited.com>

Date: Fri, 16 May 1997 13:57:28 -0400

Subject: Re: Copper tubing- How do you... -Reply -Reply

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Fri, 16 May 1997 11:01:58 -0700

Subject: Re: Copper tubing- How do you... -Reply

>>     My concern, and my reason for wanting to cover the primary, is to

>>     avoid an Arc to me/arc to primary situation. I con't want to fry my

>>    ass.

>>     60Hz doesn't feel "tickly" after all.... <<

My solution to this is to not touch the primary when your circuit is

energized.  Let me tell you that it might put you on the fast elevator to see

the Big Man!  Arcing to you shouldn't be a problem from the Primary to

you or to itself.  My Primary is spaced about 1/2" apart for each turn and

it doesn't arc.  Plus by having the tubing bare you can tune your primary

quite easily.

Arcing to you should only be an issue if you are really close to the

primary...and if you're this close then you'll probably get struck by the

secondary.  I stay about 5' away and operate my coil from there with a

switch and variac.  I think this is plenty safe.

Good Luck...if you're still not clear send me a note.

Kevin

KBATCHELOR@ARSLIMITED.COM

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Copper tubing- How do you... -Reply

Date: Fri, 16 May 1997 18:01:58 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Fri, 16 May 1997 11:01:58 -0700

Subject: Re: Copper tubing- How do you... -Reply

My concern, and my reason for wanting to cover the primary, is to

avoid an Arc to me/arc to primary situation. I con't want to fry my

ass.

60Hz doesn't feel "tickly" after all....

______________________________ Reply Separator ________________________________

Subject: Copper tubing- How do you... -Reply

Author:  tesla-2@emachine.com (Tesla-2) at Internet

Date:    5/16/97 3:39 PM

From: Kevin Batchelor <KBATCHELOR@arslimited.com>

Date: Fri, 16 May 1997 11:39:44 -0400

Subject: Copper tubing- How do you... -Reply

Don't bother covering your tubing.  It's not really necessary as long as

your spacing between turns is over 1/4".  My tubing came in a roll of 50'

so it was already bent into shape ok.  I just had to space it out on my

support forms made of lexan.  You could use wood or plexiglass.  I

found the lexan easier to cut and drill than plexiglass which is quite

brittle.  Either way...coils don't have to be pretty to be functional.

Kevin

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Copper tubing- How do you... -Reply

Date: Fri, 16 May 1997 20:27:13 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Dave <djb@erie.net>

Date: Fri, 16 May 1997 16:27:13 -0400

Subject: Re: Copper tubing- How do you... -Reply

At 06:53 PM 5/17/97 GMT, you wrote:

>From: Mike Hammer <mhammer@misslink.net>

>Date: Sat, 17 May 1997 13:53:09 -0500

>Subject: Re: Copper tubing- How do you... -Reply

>Insulating your primary probably will not

>guarantee you wont have secondary/primary

>arcing. If you having such arcing you most

>likely are overcoupling. Reducing the coupling by raising

>up the secondary (or lowering the primary) will

>reduce the amount of arcing to a low degree.

>

>If your secondary output from your toroid takes

>to liking to strike your primary insulation

>wont help that much. Thats why a grounded strike ring

>is usually necessary.

>

>This is a copper ring placed just above and slightly outside your

>last primary turn. Its connected to RF ground and should be an open loop.

>

>TTYL!

>Mike Hammer

>mhammer@misslink.net

Hi guys,

What's an easy way of mounting the secondary so you can adjust its height

and still have it firmly attached? And when first building the primary, at

about what height should it be compared to the secondary bottom?

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re[2]: toroidial chokes

Date: Fri, 16 May 1997 20:42:36 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Fri, 16 May 1997 13:42:36 -0700

Subject: Re[2]: toroidial chokes

I think they might be powdered iron. ferrite is usually fairly

lightweight and this stuff isn't. Is the ferrite they use in toroids

the same as used in microwave broadcasting equipment as dummy loads?

______________________________ Reply Separator ________________________________

_

Subject: Re: toroidial chokes

Author:  tesla-2@emachine.com (Tesla-2) at Internet

Date:    5/17/97 6:00 PM

From: Mike Hammer <mhammer@misslink.net>

Date: Sat, 17 May 1997 13:00:39 -0500

Subject: Re: toroidial chokes

At 04:47 PM 5/16/97 GMT, you wrote:

>From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

>Date: Fri, 16 May 1997 09:47:10 -0700

>Subject: toroidial chokes

>

>     I recently got some great looking toroidial core choke coils, but

>     they're wound with about #16 with about 40-50 turns on them. Will they

>     have sufficient inductance to keep excessive RF out of my Neon?

>

>

Number 16 wire is far larger than necessary for RF chokes.

Even at rather high power levels the current flow is low.

I would remove the windings and fit 100 or more turns

on your cores. I used 125 turns of #26 on my toroids.

Be sure to put a couple layers of good vinyl tape on the cores

before putting on the windings. The cores are conductive

enough to cause arcing if you dont insulate them.

Did you get any info with these toroids? Are they ferrite

or powdered iron? Even if you dont know they are probably

worth trying out.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Schematic questions

Cc:

Bcc:

X-Attachments:

At 05:01 PM 5/15/97 GMT, you wrote:

>From: Dave <djb@erie.net>

>Date: Thu, 15 May 1997 13:01:02 -0400

>Subject: Schematic questions

>

>

>    TESLA COIL SCHEMATIC                                  --------

>                                                          |      | TC

>                                                          --------

>                                                              O

>                                                              O

>               SG   SC      FCC     PC   SG        P-COIL     O S-COIL

>           O---------------OOOOO---------> <-----    O        O

>      NST  O    |    |              |           |    O        O

>           O    |    |              |           |    O        O

>   -----O||O    *   ---            ---          |--->O        O

>        O||O                                         O        O

>   -----O||O    *   ---            ---               O        O

>   110   | O    |    |              |                O        O

>   Volts | O    |    |              |                O        O

>         | O---------------OOOOO---------------------|        |

>         |                                                    |

>         |                                                    |

>        Gnd                                                  Gnd

>

>

>Hi Everyone,

>

>I have a few questions on this simple schematic above. Is this the most

>common configuration for small coils...ie less than 1000 watts?

This is a common setup and seen in many older designs.

>Are there other configurations that might be more advantageous to use, or are

>there disadvantages to this setup?

The answer is Yes to both questions.

Lets look at a different setup.

TESLA COIL SCHEMATIC                                ---------

|       | T1

---------

O

X1                                 C1              O

RFC           | |             O  L2

O------------------||||----------| |-----        O

||O       |                  |     | |    |   L1   O

||O       |                  |            |    O   O

-----O||O       * SG               * G1         |--->O   O

O||O--Gnd                                       O   O

-----O||O       *                  *                 O   O

120   ||O       |                  |                 O   O

Volts ||O       |                  |                 O   O

O------------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded center tap (case)

SG = Safety Gap.

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

As you can see the main difference is that the positions of

the main cap and main gap are swapped.

What does this do? Help to protect our fragile neons from damage

due to RF kickbacks.

With the capacitor in parallel across the neon secondary

the neon will "see" that RF across its secondary winding.

With the gap in parallel across the neon output the neon secondary

will be short circuited by the energized gap while all the big

action is taking place in the primary tank circuit.

Will this prevent damage to the neon? No. Not by itself.

But every little bit of protection helps.

>Also, what is the purpose of the safety gap? I

>can see the chokes would limit the amount of RF kickback, and the safety cap

>would act as a short for RF so it doesn't effect the transformer...so why

>have a saftey gap, and what would make it fire? And how wide should be the

>gap be?

RF chokes have only limited success in prventing Rf from getting into

the neon. The chokes help by knocking down the level of the RF.

The safety gap is there as backup to flash over and clamp the voltage

to a safe level when RF sneaks by the chokes.

The safety gap should be just wide enough so it fires every few

seconds. You want it just wider than your neon transformer

can break down on its own.

>Also could you connect a cap of suitable rating from ground to the bottom of

>the secondary before the chokes? I think it would act as an open for 60 hz,

>but as a short to ground for RF and protect the power supply, etc.

Some people have done this with success but I do not recommemd it.

In some neons bypass caps to ground can set up unwanted resonances

inside the transformer and lead to quick destruction.

I totally fried a France 12kv/30 neon in this manner.

If we take all the above steps and combine them all

we have a fairly good level of protection for our

neons. We want the main gap across the HV output.

We want a safety gap across the HV lines directly

from the transformer. And we want of minimum of 2mh

of RF choke inductance in each HV leg in between the safety

gap and main spark gap.

These 3 measures will go a long way in protecting

neons from damage.

As always comments, criticism, flames, and cash donations welcome!

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Primary shape

Date: Sat, 17 May 1997 03:23:53 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Dave <djb@erie.net>

Date: Fri, 16 May 1997 23:23:53 -0400

Subject: Primary shape

<snip>

>The proper position of the secondary in relation to the

>primary depends on a number of factors.

>If you have a flat spiral primary the position of greatest

>coupling occures when the bottom turn of the secondary

>is even with the plane of the primary. This would be a good

>place to start with a flat primary.

>

>This position will likely over couple with a saucer shaped primary

>and most defintely will with a solenoid primary.

>Try elevating the secondary up 1 inch with these primary shapes.

>

>

>When you run the coil if you get primary/ secondary arcing or heavy

>corona the raise up the secondary a bit until it stops.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

How do you decide what shape to make the primary? What are the pros and cons

of each primary shape?

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: toroidial chokes

Cc:

Bcc:

X-Attachments:

At 04:47 PM 5/16/97 GMT, you wrote:

>From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

>Date: Fri, 16 May 1997 09:47:10 -0700

>Subject: toroidial chokes

>

>     I recently got some great looking toroidial core choke coils, but

>     they're wound with about #16 with about 40-50 turns on them. Will they

>     have sufficient inductance to keep excessive RF out of my Neon?

>

>

Number 16 wire is far larger than necessary for RF chokes.

Even at rather high power levels the current flow is low.

I would remove the windings and fit 100 or more turns

on your cores. I used 125 turns of #26 on my toroids.

Be sure to put a couple layers of good vinyl tape on the cores

before putting on the windings. The cores are conductive

enough to cause arcing if you dont insulate them.

Did you get any info with these toroids? Are they ferrite

or powdered iron? Even if you dont know they are probably

worth trying out.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Copper tubing- How do you... -Reply

Cc:

Bcc:

X-Attachments:

At 06:01 PM 5/16/97 GMT, you wrote:

>From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

>Date: Fri, 16 May 1997 11:01:58 -0700

>Subject: Re: Copper tubing- How do you... -Reply

>

>

>     My concern, and my reason for wanting to cover the primary, is to

>     avoid an Arc to me/arc to primary situation. I con't want to fry my

>     ass.

>

>     60Hz doesn't feel "tickly" after all....

>

>

Insulating your primary probably will not

guarantee you wont have secondary/primary

arcing. If you having such arcing you most

likely are overcoupling. Reducing the coupling by raising

up the secondary (or lowering the primary) will

reduce the amount of arcing to a low degree.

If your secondary output from your toroid takes

to liking to strike your primary insulation

wont help that much. Thats why a grounded strike ring

is usually necessary.

This is a copper ring placed just above and slightly outside your

last primary turn. Its connected to RF ground and should be an open loop.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Insulated Primary/Elevating Secondary

Date: Sat, 17 May 1997 20:14:26 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bob Schumann <tesla@america.com>

Date: Sat, 17 May 1997 16:14:26 -0400 (EDT)

Subject: Insulated Primary/Elevating Secondary

As for covering the primary with the tubing, I am against it.

In endeavoring in this hobby and learning what there is to

know, one on the most basic taught concepts is that touching

the primary coil can kill you. The lesson is to respect the primary

coil. Covering it with tubing does little when the high frequency

voltages of the secondary are 'swinging back' into the primary.

The primary's construction should be that of space allowance

between turns to avoid flashover. It is feasible to have a totally

insulated primary in which the connections are at each end of

the primary coil, and have the system work. Greater care and

mostly - greater thought will have to go into the system since

your variable in the resonant frequency formula will now be

your primary circuit capacitor and not the primary inductance.

A feat I do not want to attempt. I would never insulate any

part of my primary due to it's main function - to allow me to

find resonance with the secondary circuit - which could be

anywhere (tapped) on the primary coil. Your system will work

best when it has attained resonant frequency.

For the secondary elevation, I came up with an idea that was

still based on the PVC idea like most mocern day Tesla parts

are made. I took my secondary to a Scottys store and found

an adapter in which the small end slipped inside of the secondary,

or would with little sanding, and the big end of the adapter was

a little bigger than the outside diameter of the secodary. I then

found a PVC piece that was just a 3 inch long tube that had the

same diameters as the piece on the adapter that slid inside the

secondary. When I go home I glued the 2 pieces together and

tada done. I had a base that would stand up with a shaft I could

slide the secondary on. Coupling from this point is just a matter

of sliding the secondary up and down on the shaft. In fact, I

made measurements and marked on the shaft lines than indicated

the coupling coefficients like .1, .12, .18. .2

Which later made it easy in other calculations when bring the

whole system into best performance.

Hope this helps,

Bob Schumann

tesla@america.com

http://www.america.com/~tesla

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Copper tubing- How do you... -Reply

Cc:

Bcc:

X-Attachments:

>Hi guys,

>

>What's an easy way of mounting the secondary so you can adjust its height

>and still have it firmly attached? And when first building the primary, at

>about what height should it be compared to the secondary bottom?

>

>Dave

>

I made a foot for my secondary from wood. The foot has a smaller

disk of wood on top of it to fit up inside the secondary form.

I just set the secondary inside the primary and put whatever

spacers I need to elevate the secondary underneath the foot.

I'm sure there are better methods but this works for me.

The proper position of the secondary in relation to the

primary depends on a number of factors.

If you have a flat spiral primary the position of greatest

coupling occures when the bottom turn of the secondary

is even with the plane of the primary. This would be a good

place to start with a flat primary.

This position will likely over couple with a saucer shaped primary

and most defintely will with a solenoid primary.

Try elevating the secondary up 1 inch with these primary shapes.

When you run the coil if you get primary/ secondary arcing or heavy

corona the raise up the secondary a bit until it stops.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Re[2]: toroidial chokes

Cc:

Bcc:

X-Attachments:

At 08:42 PM 5/16/97 GMT, you wrote:

>From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

>Date: Fri, 16 May 1997 13:42:36 -0700

>Subject: Re[2]: toroidial chokes

>

>

>

>     I think they might be powdered iron. ferrite is usually fairly

>     lightweight and this stuff isn't. Is the ferrite they use in toroids

>     the same as used in microwave broadcasting equipment as dummy loads?

>

>

>

Nathan,

Both ferrite and powder iron cores come in a variety of flavors

for use at various frequency ranges.

Generally speaking ferrites are meant for lower frequency operation

than powdered iron.

Not knowing what your cores are made from is a bit of a gamble.

But thats the risk we take when buying surplus.

The ferrite toroids I have are 2.5 inches in diameter and an inch

thick and are fairly heavy.

When you use your toroids check them for temperature.

If they get really hot they are probably saturating and may not

be suitable. I would bet though they will work just fine.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Primary shape

Cc:

Bcc:

X-Attachments:

At 03:23 AM 5/17/97 GMT, you wrote:

>From: Dave <djb@erie.net>

>Date: Fri, 16 May 1997 23:23:53 -0400

>Subject: Primary shape

>

>

>How do you decide what shape to make the primary? What are the pros and cons

>of each primary shape?

>

>Dave

>

Good question Dave.

There are 3 main primary coil shapes.

Solenoid (like secondary coil)

saucer

flat spiral.

The solenoid form of primary is usually twice the diameter of the

secondary or a bit larger. This primary shape often overcouples

forcing you to raise the secondary up out of the primary

field to reduce coupling. Its easy to wind and works well

if you can cope with the high degree of coupling.

If you have many turns the height of this primary can be a

disadvantage as it becomes a favorite target for your streamers.

The flat spiral is easy to wind as the boxes of soft copper

tubing come nearly already formed in this shape. The height

of this primary shape is essentially zero so is not likely to

be hit often with secondary streamers. One disadvantage is this

form will possibly undercouple especially at lower power levels.

The saucer shaped or conical shaped primary is a comprimise

between the flat spiral and solenoid shapes. It combines

the best features of both. The slope of the turns is usually

30 degrees but there is nothing sacred about that value.

The flatter you make the primary the less coupling it will provide.

This type is a little more difficult to construct.

So what type to use? Probably doesn't make that much difference.

Inductance is inductance. They will probably all provide equivalent

performance after making allowance for the difference in coupling.

Solenoid type primaries work well on small coils.

On higher power coils where over coupling may become

a problem a saucer or flat primary may be called for.

For high power coils the flat spiral is definitely the best choice.

Gary Weaver recently performed tests on a number of different

primary coils and tested them on the same secondary. He has determined that

tightly wound (small turn to turn spacing) saucer shaped

primaries with a 15 degree slope performed the best for him.

He also found that primaries wound with 1/4 inch copper tubing

with 1/4 inch turn to turn spacing gave best results.

comments and criticism always welcome

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Insulated Primary/Elevating Secondary

Date: Sun, 18 May 1997 11:12:39 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bob Schumann <tesla@america.com>

Date: Sun, 18 May 1997 07:12:39 -0400 (EDT)

Subject: Re: Insulated Primary/Elevating Secondary

At 10:38 PM 5/18/97 GMT, you wrote:

>From: Mike Hammer <mhammer@misslink.net>

>Date: Sun, 18 May 1997 17:38:53 -0500

>Subject: Re: Insulated Primary/Elevating Secondary

>

>At 08:14 PM 5/17/97 GMT, you wrote:

>>From: Bob Schumann <tesla@america.com>

>>Date: Sat, 17 May 1997 16:14:26 -0400 (EDT)

>>Subject: Insulated Primary/Elevating Secondary

>

><SNIP>

>

>>For the secondary elevation, I came up with an idea that was

>>still based on the PVC idea like most mocern day Tesla parts

>>are made. I took my secondary to a Scottys store and found

>>an adapter in which the small end slipped inside of the secondary,

>>or would with little sanding, and the big end of the adapter was

>>a little bigger than the outside diameter of the secodary. I then

>>found a PVC piece that was just a 3 inch long tube that had the

>>same diameters as the piece on the adapter that slid inside the

>>secondary. When I got home I glued the 2 pieces together and

>>tada done. I had a base that would stand up with a shaft I could

>>slide the secondary on. Coupling from this point is just a matter

>>of sliding the secondary up and down on the shaft. In fact, I

>>made measurements and marked on the shaft lines than indicated

>>the coupling coefficients like .1, .12, .18. .2

>>Which later made it easy in other calculations when bring the

>>whole system into best performance.

>>

>>Hope this helps,

>>

>>Bob Schumann

>>

>

>Bob what a great idea!. Gee wish I had thought of that.

>

>How do you fix the secondary in position once you have it

>at the height you want?

>

>TTYL!

>Mike Hammer

>mhammer@misslink.net

Mike,

I would find an adaptor whose one diameter fit the secondary's

inside diameter snug or would require little sanding. Since it is

a snug fit, the secondary 'telescopes' on the shaft. To move the

secondary, I would 'twist' the secondary back and forth and in

the direction I wanted the secondary to go. I once did

have a shaft that was not snug enough so I took some tape

and wrapped just the top of the shaft and the secondary held

in place by that point.

Bob

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Insulated Primary/Elevating Secondary

Cc:

Bcc:

X-Attachments:

At 08:14 PM 5/17/97 GMT, you wrote:

>From: Bob Schumann <tesla@america.com>

>Date: Sat, 17 May 1997 16:14:26 -0400 (EDT)

>Subject: Insulated Primary/Elevating Secondary

<SNIP>

>For the secondary elevation, I came up with an idea that was

>still based on the PVC idea like most mocern day Tesla parts

>are made. I took my secondary to a Scottys store and found

>an adapter in which the small end slipped inside of the secondary,

>or would with little sanding, and the big end of the adapter was

>a little bigger than the outside diameter of the secodary. I then

>found a PVC piece that was just a 3 inch long tube that had the

>same diameters as the piece on the adapter that slid inside the

>secondary. When I got home I glued the 2 pieces together and

>tada done. I had a base that would stand up with a shaft I could

>slide the secondary on. Coupling from this point is just a matter

>of sliding the secondary up and down on the shaft. In fact, I

>made measurements and marked on the shaft lines than indicated

>the coupling coefficients like .1, .12, .18. .2

>Which later made it easy in other calculations when bring the

>whole system into best performance.

>

>Hope this helps,

>

>Bob Schumann

>

Bob what a great idea!. Gee wish I had thought of that.

How do you fix the secondary in position once you have it

at the height you want?

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Copper tubing- How do you...

Date: Mon, 19 May 1997 00:21:23 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sun, 18 May 1997 20:21:23 -0400

Subject: Re: Copper tubing- How do you...

> From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

> Date: Fri, 16 May 1997 09:45:16 -0700

> Subject: Copper tubing- How do you...

>

>      How do you guys get such beautiful smooth coils out of copper tubing?

>      I've bought a bunch of 1/4" soft copper tubing and I'm having

a

>      terrible time making it bend symmetrically. I need to know how

you

>      guys bend your tubing.

>         I would also like to run the tubing into some 5/8" aquarium

air

>      hose. to do this I must straighten out the hose and push it

through.

>      then I can bend the copper, but it's still hard to get it to

look

>      pretty, and I know the electrical characteristics are better

if the

>      spiral is smooth and on one plane (for a flat spiral). Give me

some

>      hints!

>

Nathan,

Forget the plastic aquarium tubing! It will be a major pain in the

you-know-what to get it over 50 feet of copper tubing. Once on, it

will make the copper tubing a real bear to handle, especially if you

have to do any more forming with it. Attaching to the proper tune

point will not be easy through the plastic. If a discharge wants to

hit the primary, the plastic tubing will not REALLY help you much

(use a strike rail instead). Once a discharge DOES ram its way

through the plastic tubing, THAT will become the *preferred* entrance

point for all subsequent discharges. This will result in

carbonization of the plastic. It will now conduct just wonderfully,

thank-you, and will char itself into an ugly mass of garbage that

just sits there spewing noxious fumes into the room. You will then

curse madly as you try to *remove* the plastic that you so

laboriously installed.

Totally wasted human effort. Messes with your mind. Upsets your

delicate balance. You have better things to do with your physical and

emotional energy.

At first it seems like a good idea. But in reality, it really isn't

worth the aggravation.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: new cap

Date: Sat, 24 May 1997 01:46:38 GMT

Organization: The Energy Machine Information System 504-734-0526

From: kcravens <kcravens@sound.net>

Date: Fri, 23 May 1997 20:46:38 -0500 (CDT)

Subject: Re: new cap

At 05:25 PM 5/24/97 GMT, you wrote:

>From: Mike Hammer <mhammer@misslink.net>

>Date: Sat, 24 May 1997 12:25:42 -0500

>Subject: new cap

>

>Hello all,

>

>As some of you may already know my last attempt at cap

>building was less than successful.

(SNIP)

>After letting the new cap sit in oil for 24 hours I fired it

>up and it has performed splendidly. I have had solid 36 inch

>arcs using my small 7 gap RQ gap and 12kv/90ma neon.

>Mike Hammer

>mhammer@misslink.net

Hey Mike!

Glad ta hear your cap was well worth the effort! 36" sounds good!

I think I'll try some stacked plate caps again. The last ones I made were

of glass and foil, with air for di-elec. They were really just 10 or so single

plates in a line --> ||||||||  and I paralled 'em  about an inch apart. Had a

12K 30ma neon then... now I use 9K 60ma's cause they're more common here in KC.

They were so "lossy" that they kinda added to the "show", whenever I ran it. ;o

)

I was gettin' maybe 18" of spark then. (my first attempt coil) But enough

cap glow

to read by ;op~   When I get time I'll hook up the glass caps to the 4 - 9 60's

and see how they do.

Your coil sounds pretty efficient! 36" lightnin'. My current "best" is about

6' usin rolled caps--.006 -- and four 9 60's.  But mine's all "temporary"

hookups and looong wires to connections... I just wanted to see a really

crude set up make sparks, and now each little improvement makes it better,

and better. I like that :o)  Well, I hope this post ain't all chopped up

'n stuff, I hadda edit it some, and I'm afraid I have that "onewordonaline"

email virus thingy ;o)

Coil ON!

Ken Cravens

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: new cap

Date: Sat, 24 May 1997 16:39:53 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sat, 24 May 1997 09:39:53 -0700

Subject: Re: new cap

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sat, 24 May 1997 12:25:42 -0500

> Subject: new cap

>

> Hello all,

>

> As some of you may already know my last attempt at cap

> building was less than successful.

>

> It was a plate type cap with 14  10 X 10 inch plates

> of aluminum flashing. The dielectric was 10 layers

> of 6 mil poly between each plate. The poly sheets were cut

> to 12 X 12 inches. Measured capacitance was .0087 mfd.

>

> I rushed this cap into operation and its performance was poor.

> I kept pouring on the power and it blew up.

>

> The arc did not go through the poly layer so the 60 mils

> of poly held up to my 12kv neon. The arc traveled from the edge

> of one plate along the surface of the poly layer to the edge of

> the poly sheet and down onto the plate below it.

>

> Upon disassembly of this blown cap it was apparent the reason for

> the poor performance was trapped air. Even after 4 days in oil

> there were many very large air bubbles over an inch in diameter.

>

> I figure the breakdown was due to one of these air bubbles

> ionizing and becoming conductive.

>

> So I rebuilt the cap using the same design. I added two additional

> plates to bring the capacity up to a measured .0099 mfd.

>

> I built this cap in oil. I dipped each poly sheet in mineral

> before adding it to the stack and then working the air bubbles

> out with my fingers. It was a messy job and I was up to my

> elbows in mineral oil for two days.

>

> After letting the new cap sit in oil for 24 hours I fired it

> up and it has performed splendidly. I have had solid 36 inch

> arcs using my small 7 gap RQ gap and 12kv/90ma neon.

>

> I will try this weekend to break my old record of 40 inches

> with this cap and using my 8 gap vacuum gap.

>

> Moral of this story is that trapped air is an enemy of

> homemade caps. Due what it takes to get the air out.

> If you have access to a vacuum pump by all means use it.

> This is especially true of rolled caps which tend to trap even more air.

>

> If you're like me and dont have access to a vacuum pump then build

> plate caps and take measures to eliminate the air bubbles. Small plates

> will help as will putting a layer of kraft paper between the poly

> layers to improve the wicking action. The paper will also

> cut down the capacitance obtained.

>

> Building the cap in oil while messy does seem to be

> a viable method. It worked for me.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

>

> * Send list Replies to: Tesla-2@emachine.com

>

> * To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

>   in the body of your message to: Listserv@Emachine.com

>

> -----

> Processed with Listserv v2.93 for Wildcat v4

Mike,

Sounds like a messy job... but it also sounds like it worked!!

Have a great 3-day weekend!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: new cap

Cc:

Bcc:

X-Attachments:

Hello all,

As some of you may already know my last attempt at cap

building was less than successful.

It was a plate type cap with 14  10 X 10 inch plates

of aluminum flashing. The dielectric was 10 layers

of 6 mil poly between each plate. The poly sheets were cut

to 12 X 12 inches. Measured capacitance was .0087 mfd.

I rushed this cap into operation and its performance was poor.

I kept pouring on the power and it blew up.

The arc did not go through the poly layer so the 60 mils

of poly held up to my 12kv neon. The arc traveled from the edge

of one plate along the surface of the poly layer to the edge of

the poly sheet and down onto the plate below it.

Upon disassembly of this blown cap it was apparent the reason for

the poor performance was trapped air. Even after 4 days in oil

there were many very large air bubbles over an inch in diameter.

I figure the breakdown was due to one of these air bubbles

ionizing and becoming conductive.

So I rebuilt the cap using the same design. I added two additional

plates to bring the capacity up to a measured .0099 mfd.

I built this cap in oil. I dipped each poly sheet in mineral

before adding it to the stack and then working the air bubbles

out with my fingers. It was a messy job and I was up to my

elbows in mineral oil for two days.

After letting the new cap sit in oil for 24 hours I fired it

up and it has performed splendidly. I have had solid 36 inch

arcs using my small 7 gap RQ gap and 12kv/90ma neon.

I will try this weekend to break my old record of 40 inches

with this cap and using my 8 gap vacuum gap.

Moral of this story is that trapped air is an enemy of

homemade caps. Due what it takes to get the air out.

If you have access to a vacuum pump by all means use it.

This is especially true of rolled caps which tend to trap even more air.

If you're like me and dont have access to a vacuum pump then build

plate caps and take measures to eliminate the air bubbles. Small plates

will help as will putting a layer of kraft paper between the poly

layers to improve the wicking action. The paper will also

cut down the capacitance obtained.

Building the cap in oil while messy does seem to be

a viable method. It worked for me.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: new cap

Cc:

Bcc:

X-Attachments:

At 01:46 AM 5/24/97 GMT, you wrote:

>From: kcravens <kcravens@sound.net>

>Date: Fri, 23 May 1997 20:46:38 -0500 (CDT)

>Subject: Re: new cap

>

>

>Hey Mike!

>Glad ta hear your cap was well worth the effort! 36" sounds good!

>I think I'll try some stacked plate caps again. The last ones I made were

>of glass and foil, with air for di-elec.

Boo-Hiss  :)

>They were really just 10 or so single

>plates in a line --> ||||||||  and I paralled 'em  about an inch apart. Had a

>12K 30ma neon then... now I use 9K 60ma's cause they're more common here in KC.

>They were so "lossy" that they kinda added to the "show", whenever I ran it.

>>I was gettin' maybe 18" of spark then. (my first attempt coil) But enough

>cap glow to read by. When I get time I'll hook up the glass caps to the 4 - 9 60's

>and see how they do.

Generate lots of heat, corona and ozone I bet.

>

>Your coil sounds pretty efficient! 36" lightnin'. My current "best" is about

>6' usin rolled caps--.006 -- and four 9 60's.

Is that 6 inches or 6 feet? I would hope you meant feet. If you

are running 9kv /240ma I would hope for better than 6 inches!

What size secondary you have and what you using for a gap?

I've been using the same 12kv/90 neon for power for some time

and been trying to improve the efficiency and get the most spark

from this one neon. My best record is 40 inches using a 19 X 6

toroid with a vacuum gap.

My goal for this coil is 2X the secondary winding length. That would

be 44 inches. I'm hoping my new .01 cap and maybe a little bigger toroid

will get me there. Seems like the gains are coming an inch at a time.

>But mine's all "temporary" hookups and looong wires to connections...

No such things as a permanent hookup. Not with Tesla coils

anyway.

>I just wanted to see a really

>crude set up make sparks, and now each little improvement makes it better,

>and better. I like that :o)  Well, I hope this post ain't all chopped up

>'n stuff, I hadda edit it some, and I'm afraid I have that "onewordonaline"

> email virus thingy ;o)

email virus? Is this something I havn't heard of yet?

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: stacked plate cap

Date: Sun, 25 May 1997 07:40:00 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Ralph Down <rjdown@fan.net.au>

Date: Sun, 25 May 1997 17:40:00 +1000

Subject: stacked plate cap

I recently used one of the Tesla helper programs (Teslac- I think) to design

a flat stacked cap using .1" thick  solid polystyrene and .01" thick

aluminium litho plates (used in printing). These are usually recycled, but

if you approach a small printing firm, they will often part with 10 to 20

sheets.

The design for .01 uF required 20 plates 10" X 12", separated by the .1"

polystyrene.

I assembled the stack dry to test the capacitance using a cheap multimeter

with a capacitance range and found it to be exactly as predicted (.01 uF).

Now here is the interesting bit.  When I finally assembled the unit, I

constructed a plexiglass container just slightly larger than the assembed

stack so that I could submerge the unit in oil.  Having built a similar cap

previously using aluminium foil and then  trying to get rid of the trapped

air (unsuccessfully), I decided to assemble the unit in the enclosure one

plate at a time and brush each plate with oil as it was stacked so as to

eliminate any air or dry spots. When all the plates were stacked, I removed

the completed unit from the enclosure and used two 2 1/2" brass bolts with

.1" washers between each aluminium plate pushed through previously drilled

holes in the plates and tightened the whole stack with a brass nut.(This

then became one terminal with a wing nut to secure the cable).  I then

lowered the assembled unit into the enclosure & filled it with SUNISO

refrigeration oil (bought from local refrigeration repair shop for $30 for 4

litres.) I was very surprised when I again measured the capacity of the

completed unit - it was now .02 uF - a 100% increase in capacity due to the

addition of the high dielectrc constant of the oil.  This cap has been

working for several months using 4 x 10kV 30 mA neon trannies in parallel

with no problems whatsoever.

Hope this is useful to someone.

Regards

Ralph Down

Today is the tomorrow you worried about yesterday....... and everything is O.K.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tesla Coil on the News

Date: Mon, 26 May 1997 06:10:14 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Jeremy Bair <pwac@flinet.com>

Date: Mon, 26 May 1997 02:10:14 -0400 (EDT)

Subject: Tesla Coil on the News

Two days ago a friend woke me up at around 11:30 to tell me that he saw

some sort of Tesla Coil coming up on the news. So I got up and stuck a

blank tape in the VCR :)

When I woke up the next day and played the story, it started out informing

us of power company struggles with customers, one customer in particular,

his name was Phillip Cambell of Tallahassee Florida (I don't expect any

other coilers here have seen the report, it was probably only run in

Florida, but I would like to be aware of anyone who saw it in or out of

Florida *please reply*).

Basically what happened was FP&L (Florida Power And Light) cut this guys

power off because he wasn't paying his bills, so he decided to live

without consumer electricity. They never mentioned how he was currently

giving his house juice, but I take it he had some sort gas powered of

generator.

The whole news report was stupid (in my opinion of course), they were

hinting (Although not directly implying) that he was trying to develop a

way to transport electricity wireless to move the power from land lines to

the air. Using what they called "Age old methods". This report was a

crock, they never even mentioned the name Nikola Tesla throughout the

whole report.  After recently reading the biography of Nikola Tesla

(Wizard: The Life and Times of Nikola Tesla - Marc J. Seifer *very good

book*) I became very pissed off at this report.

I called up the station and they told me it was from an affiliate station

and they thought it looked "cool" so they decided to take up the rest of

the time with it.

It looks like this guy *may* be trying to take credit to reinvent Nikola

Tesla's system of wireless transmission, all I got to say to him is good

luck. But I doubt he will get anything accomplished. He looked so poor I

doubt he is going to get any support from anyone concerning this, and I

bet he is going to have some REAL fun with the FCC if he tries something

like transmitting electricity.

On Tuesday I'm going to try to get in touch with the news channel that

did the report in Tallahassee and see if I can get anymore info about who

this guy is and if they know how he was powering his house.

Oh, and by the way. The coil he was demonstrating with was REAL poor, you

sound have seen it, the secondary was rapped on a piece of PVC pipe that

looks like he cut it from some construction site, it wasn't coated or

sanded (You could read the writing on it), and he was using ALLOT of black

electrical tape to hold his coil together.

The primary was even worse, it was about 4-5 turns of copper way below the

secondary, and his connections were maxed out at the end of copper (I

guess thats all he could rip out of his old frig).

Guess how he was tuning his coil? By placing items like tools and spark

plugs on top of his secondary! I'm sure that looked real nice to display

in front of a camera, a corona covered spark plug...

Anyhow, thats enough from me, let me conclude that the lady I talked to at

the news channel (the program director) could not even understand what the

story was about. I think she ran it because of the pretty blue sparks :)

--------------------------------------------------------------------------

|Jeremy Bair IRC:DeadBatry chat.talkcity.com #Tesla

|Email: mailto:pwac@flinet.com mailto:deadbatry@hotmail.com

|Web page: http://www.flinet.com/~pwac

|Phone: 561-585-4165

|

|

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: stacked plate cap

Cc:

Bcc:

X-Attachments:

At 07:40 AM 5/25/97 GMT, you wrote:

>From: Ralph Down <rjdown@fan.net.au>

>Date: Sun, 25 May 1997 17:40:00 +1000

>Subject: stacked plate cap

>

>I recently used one of the Tesla helper programs (Teslac- I think) to design

>a flat stacked cap using .1" thick  solid polystyrene and .01" thick

>aluminium litho plates (used in printing). These are usually recycled, but

>if you approach a small printing firm, they will often part with 10 to 20

>sheets.

>The design for .01 uF required 20 plates 10" X 12", separated by the .1"

>polystyrene.

>I assembled the stack dry to test the capacitance using a cheap multimeter

>with a capacitance range and found it to be exactly as predicted (.01 uF).

>Now here is the interesting bit.  When I finally assembled the unit, I

>constructed a plexiglass container just slightly larger than the assembed

>stack so that I could submerge the unit in oil.  Having built a similar cap

>previously using aluminium foil and then  trying to get rid of the trapped

>air (unsuccessfully), I decided to assemble the unit in the enclosure one

>plate at a time and brush each plate with oil as it was stacked so as to

>eliminate any air or dry spots. When all the plates were stacked, I removed

>the completed unit from the enclosure and used two 2 1/2" brass bolts with

>.1" washers between each aluminium plate pushed through previously drilled

>holes in the plates and tightened the whole stack with a brass nut.(This

>then became one terminal with a wing nut to secure the cable).  I then

>lowered the assembled unit into the enclosure & filled it with SUNISO

>refrigeration oil (bought from local refrigeration repair shop for $30 for 4

>litres.) I was very surprised when I again measured the capacity of the

>completed unit - it was now .02 uF - a 100% increase in capacity due to the

>addition of the high dielectrc constant of the oil.  This cap has been

>working for several months using 4 x 10kV 30 mA neon trannies in parallel

>with no problems whatsoever.

>Hope this is useful to someone.

>Regards

>Ralph Down

Ralph,

The construction method you used on your plate

cap is very similar to what I did. I used 4 bolts to put mine

together. One in each corner. These did double duty as terminal

connections like you did in yours.

Polystyrene is OK as a dielectric. It has low dissipation

but it has considerably more loss than polyethylene.

I dont believe it offers any advantage over poly unless

you have a cheap source of it and you dont have any poly.

Then it would be great. :)

Your increase in capacitance when you put in oil may be due

to trapped air being replaced by oil and the refrigeration

oil having a high dielectric constant. Its probably a silicone

based oil.

One very good oil but expensive is castor oil. It has good

electrical properties and a dielectric constant of 4.7.

Twice that of mineral oil or polyethylene. I've always

wanted to try it but couldn't find a source of it cheap

enough to be practical.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Primary coils...

Date: Tue, 27 May 1997 06:26:13 GMT

Organization: The Energy Machine Information System 504-734-0526

From: elite@rain.org

Date: Mon, 26 May 1997 23:26:13 -0700

Subject: Primary coils...

Organization: Elite Web Productions

I have been building a Tesla Coil on and off for a couple of weeks now

and have run into multiple problems.  I think that i have solved all of

them except for the one involving the primary coil.  The secondary coil

that i am using is a 2ft long 1.5in PVC pipe wrapped with 22awg magnet

wire.  I have two problems/questions.  They are as follows:

1.  How would a primary coil be constructed to work in conjunction with

the secondary coil.

2.  Where would the primary coil be placed?  In many of the schematics

that i have seen... it seems like the primary is next to the secondary,

but it seems most logical that it sould be basically wrapped around the

secondary.

oh yeah... one other thing...

how do you measure the capacitance the capacitors that i am using?  (i

am using a four bottle configuration)  what is involved with this and

why do i need to know this??)

Thanks for your time..

Nick Savage

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Primary coils...

Date: Tue, 27 May 1997 11:18:14 GMT

Organization: The Energy Machine Information System 504-734-0526

From: tesla@america.com (Bob Schumann)

Date: Tue, 27 May 1997 07:18:14 -0400 (EDT)

Subject: Re: Primary coils...

>From: elite@rain.org

>Date: Mon, 26 May 1997 23:26:13 -0700

>Subject: Primary coils...

>Organization: Elite Web Productions

>

>I have been building a Tesla Coil on and off for a couple of weeks now

>and have run into multiple problems.  I think that i have solved all of

>them except for the one involving the primary coil.  The secondary coil

>that i am using is a 2ft long 1.5in PVC pipe wrapped with 22awg magnet

>wire.  I have two problems/questions.  They are as follows:

>

>1.  How would a primary coil be constructed to work in conjunction with

>the secondary coil.

There are basically 3 types of primary coil configurations that coilers

use.

1) Archemedian Spiral Primary  (Flat Pancake)

This primary is most often used for systems that provide

a high power output. Systems greater that 2 - 3kw employ

this primary configuration.

2) Helical Primary

This primary is a rising spiral and is parallel to the

secondary coil which is within it. Not to many coilers

use this configuration because a potential occurs at the

top winds of this primary that can easily arc over to the

secondary. The logical answer would be to increase the

inside diameter large enough so that arc over ceases but

at the same time you are diminishing the field for the

secondary. The only time I used this primary was when I

was messing around with a Model-T coil and it was the

best coupling I could get.

3) Hybrid Primary (Inverted Cone)

This seems to be the most popular primary and is basically

a marriage of the above 2 primary types. It is a saucer

shaped spiral which rises on an angle of about 30 degrees.

It is used in small to medium power systems. This will

probably be the primary you would want to make. There are

pictures of this primary in the small coil construction

section of my homepage.

In all the primary configurations, a form(s) is made to support

the winds the make up the secondary. Learn from my mistakes. My

first forms were made all based on PI. It was not until I got

them finished did the reality slap me in the face that there are

no perfect circles in a primary. It is an expanding spiral and

the forms must be engineered  accordingly.

>2.  Where would the primary coil be placed?  In many of the schematics

>that i have seen... it seems like the primary is next to the secondary,

>but it seems most logical that it sould be basically wrapped around the

>secondary.

The secondary coil is placed within the primary coil. Though it is

possible to have a secondary coil 'work' outside of a primary, it

would not have as good energy transfer as if the secondary were

placed directly in the field that the primary creates. The symbols

used in schematics depict unique components and is a representation

of a ciruit in 2 dimensions (on paper). The schematics you see are

just to show components and  connections and don't always account for

the 3 dimensional world we live in. The secondary is placed in the

center of the primary.

>oh yeah... one other thing...

>how do you measure the capacitance the capacitors that i am using?  (i

>am using a four bottle configuration)  what is involved with this and

>why do i need to know this??)

I'm lazy and did not want to do the math and use a signal generator

to determine capacitance. I bought a LCR meter.

(L) Inductance

(C) Capacitance

(R) Resistance

So it is just a matter of hooking up a component and reading the

measurement. If I remember correctly, bottle caps have a value

in the picofarads, which is real small. Many bottles would have

to be connected in parallel to achieve a high capacitance.

Why is knowing the capacitance important? Suppose I told you

to turn on your FM radio and tune to the station at 110.3 mhz

You go to your radio and find that your radio only goes up

as far as 108 mhz. You cant get 110.3 mhz because you do not

have the ability to 'tune' it in. The FM 'band' is lower than

the 110.3 mhz. I say 'too bad' because if you could tune to

110.3 mhz you would get sparks. This is just an example. The

combination of your primary's capacitance (PRIMARY CAPACITOR) and

the primary's inductance (PRIMARY COIL) is going to determine

your dial range and just what 'station' range you can select.

If the 'station' that your secondary circuit is, cannot be 'tuned'

in with the range that your primary is capable of, you will

never get any spark. Someone asked me what is the most important

part of a Tesla coil. And though I almost said 'spark gap'

because it is truly the heart of the Tesla coil system, I instead

said 'resonance' because without it you can have the best spark

gap in the world and you will still have no spark.

Bob Schumann

tesla@america.com

http://www.america.com/~tesla

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tesla Coil on the News

Cc:

Bcc:

X-Attachments:

At 06:10 AM 5/26/97 GMT, you wrote:

>From: Jeremy Bair <pwac@flinet.com>

>Date: Mon, 26 May 1997 02:10:14 -0400 (EDT)

>Subject: Tesla Coil on the News

>

>

>Two days ago a friend woke me up at around 11:30 to tell me that he saw

>some sort of Tesla Coil coming up on the news. So I got up and stuck a

>blank tape in the VCR :)

<SNIP>

Jeremy

Interesting but very typical of news organizations trying

to cover something they don't understand.

I had an interesting go around with a local newspaper

that printed a photo that someone took of a "UFO"

flying over Galesburg, Il.

Just so happens I saw the exact same thing and it was the planet

Venus distorted by some lenticular clouds.

They put the "UFO" photo on the front page and never did

print my explanation of what it really was.

But I was able to get them to print some pictures

I took of comets Hyakutake and Hale Bopp.

Since you have a contact now at the TV station why not

invite them over for a demo of a real coil when you get yours

up and running and write then up a little piece on Tesla.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Rotary gap

Date: Fri, 30 May 1997 07:45:44 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Dave <djb@erie.net>

Date: Fri, 30 May 1997 03:45:44 -0400

Subject: Rotary gap

Why shouldn't a rotary gap be used in conjunction with a neon transformer?

Does this also go along with OBIT's?

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: progress report

Cc:

Bcc:

X-Attachments:

Hello all,

I finally got around to doing some work

with my coil the past couple of days. (The rain

and cold weather here in Illinois helped)

My new .01 mfd plate cap is performing well.

I began pushing it a bit harder and am trying to

push my distance record for streamer length out farther.

This is the same cap I blew up a couple weeks ago.

Now rebuilt with the air removed it is performing

splendidly.

Using my vacuum gap with 7 gaps @ .025 inch and

19 X 6 toroid I got very strong 40 inch streamers.

This ties my old record using my older .008 mfd plate

cap but the streamers definitely stronger and brighter

with the new cap.

Opening up the gap to a full 8 gaps @ .025 inch got

me 42 inch streamers which is a new record but still

2 inches short of my goal of 44 inches. The goal is

to get 2X my 22 inch secondary winding length.

Power for these runs is a single 12kv/90ma neon.

Vacuum gap is modified RQ style cylinder gap with

airflow provide by vacuum cleaner motor.

The bigger cap has definitely increased primary circuit

energy levels and I notice now some heating in my primary

tap arrangement. I need to beef this area up a bit

as there seems to be some loss involved there.

Always seems there is something to rebuild or modify.

For some of the newer list members who may not know

what the specifics of my coil are I will list the specs

here.

secondary   4 inch PVC with 756 turns AWG 22.

measured resonant frequency 475 KHZ

Primary     17 turns AWG 6 solid copper. Tapered solenoid

8 inch diameter at bottom   12 inches at top

tapped at turn 15 for 19 X 6 toroid

capacitor  .01 mfd (9960 pf measured)  16 aluminum plates

10 X 10 inches with 10 layers 6 mil poly dielectric

in mineral oil

power       Jefferson 12kv/60ma neon rebuilt with 5 shunts removed

giving 12kv/90 ma measured

Terminal    19 X 6 inch toroid made from aluminum dryer duct

and pie pans  brings resonant frequency down to 295 KHZ

Gap         modified RQ cylinder gap with vacuum cleaner motor

supplying airflow. 8 gaps @ .025 for .200 inch total gap

safety gap  set at .4375 (7/16) inch

RF chokes   2.25 MH  125 turns AWG 26 wound on large ferrite toroids

RF ground   two 4 foot ground rods in parallel located 6 feet from

base of coil. Ground lead is short 6 foot straight run

of AWG 8 ground cable.

I would like to hear from some of you with operating coils

about what you are doing and what results you've had.

I also want to know what you are using for a primary tap since

I need to improve this. I was using a large alligator clip

with some copper plates soldered to the jaws to increase the

surface area. This has worked well but needs replacement.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Rotary gap

Cc:

Bcc:

X-Attachments:

At 07:45 AM 5/30/97 GMT, you wrote:

>From: Dave <djb@erie.net>

>Date: Fri, 30 May 1997 03:45:44 -0400

>Subject: Rotary gap

>

>Why shouldn't a rotary gap be used in conjunction with a neon transformer?

>Does this also go along with OBIT's?

>

>Dave

>

This seems to be one of those questions that

keeps coming back again and again. It seems that

all new coilers eventually come to this point.

They all see a rotary gap as something magical that

will give extraordinary performance.

Neon transformers and OBITs (oil burner ignition transformers)

do not need a rotary gap as they perform very well with static

gaps and can even be damaged by the use of a rotary.

How can a rotary gap damage a neon?

Well, consider this.

Think of a static gap. Its electrodes are fixed and its breakdown

voltage doesnt vary. When used on a coil its sitting there waiting

for the voltage to rise enough to reach its breakdown point.

Once the breakdown voltage is reached the gap fires and the stored

energy in the primary cap is released. This breakdown voltage

will be the same for each firing and the primary circuit

voltage wont rise above this value. (for the most part)

Now consider a rotary gap. Its not just sitting and waiting.

Its rotating. Its breakdown voltage is varying all the time.

From a few KV to hundreds of KV. When the cap voltage is peaking

the rotary gap may not be in firing position just yet. So

we have to wait a bit more and the voltage rises still more.

The primary circuit voltages can rise to quite high voltages.

To the peak value of the transformer rating. And more.

Peak value of a 15kv neon would be  1.414 X 15kv = 21210 volts.

This greater voltage rise will wear away at the transformer

insulation and place extra stress on your capacitor.

Neons have very little safety margin above their plate rating

and can and do fail even when run at their rating.

Also the higher pulse rate the rotary gap provides places

our caps and transformer under a torture test.

Only the best quality components survive.

A rotary gap can and is used to good advantage with

pole transformers and other heavy duty transformers.

These units are able to withstand the punishment

but our poor fragile neons wont hack it.

Prevent neon abuse!

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: catastrophe

Cc:

Bcc:

X-Attachments:

Hello all,

I may not be around much for the next few days

due to a major computer crash. It was a simple

upgrade to increase my machines memory from 12 to

16 megs RAM.

Adding the new simm trashed my Win95 installation

so bad I could not recover it. Even installing it

again over the old installation proved fruitless.

I was only able to get the machine up by installing Win95

fresh in a new directory and try to salvage my programs

and data on a one by one basis.

I have the machine up and working again (back at 12 meg RAM)

but do not have Internet access reconfigured yet.

My only Internet access for the time being will be my machine

at work.

So if I havnt replied to email that some of you may have sent

thats why. I hope to be back up to speed in a couple more days.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: catastrophe

Date: Wed,  4 Jun 1997 00:55:54 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Shane Randle <randlec@scnc.addison.k12.mi.us>

Date: Tue, 03 Jun 1997 20:55:54 -0400

Subject: Re: catastrophe

Mike,

Sorry to hear about your crash, I know what that's like.  Is it possible

that you may have caused some kind of short that interferes with the

hard drive operation?  I've never heard of a case like the one you

described.

I wish to unsubscribe to the Tesla Coil mailing list.  The articles are

great, but my knowledge of electronics is not quite at that level yet.

I'd like to find a group with more basic, introductory-type electronics,

if you know of any.  I would appreciate any insights you might have on

this.  Good luck restoring your system, I have experienced somewhat

similar crashes before, so I know how "fun" restoring a system can be.

Thanks in advance for any info you might have.

-Shane M. Randle

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: re: catastrophe

Cc:

Bcc:

X-Attachments:

Hello all,

Well I have my computer mostly all back

together and working well. Fortunately

I had backups of most critical stuff

but of course a few things went into the bit bucket.

Thanks to all who sent notes of encouragement

and sympathy.

Needless to say I have not done much Tesla coiling

the past few days but plan to return to some

experiments this coming weekend.

Planned experiments include use of a new 24 X 6 toroid

and paralleling my old .008 mfd cap and the new .01 mfd cap.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject:

Date: Tue, 10 Jun 1997 10:09:10 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Tue, 10 Jun 1997 12:09:10 +0200 (SAT)

Subject:

Dear Sir

I want to know if anyone knows about The Magnifier Coil and that can help me

construct a capacitor.

My transformer is 15000 VAC and 30mA

Please send me your email address if you can help me.

Thanks

Richard Smit

email: esmit@ilink.nis.za

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Help

Date: Wed, 11 Jun 1997 07:28:56 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Wed, 11 Jun 1997 09:28:56 +0200 (SAT)

Subject: Help

Hi, you all.

Does anyone know a formula to calculate the coeeficient of coupling.

Will someone please check if I can make my coil perform better.

Here is my coil specs

Transformer: It's a 15000 VAC rated 30 milli-amps transformer.

Secondary Coil:  It has about 723 windings of 21 AWG wire.

--------------   The diameter of the coil is 4.4 inch.

The height of my coil is 24 inch.

Primary Coil:    It has 16 turns but the resonant point is at about the

------------     eighth turn.

The diameter is 8.8 inch.

The height is 6 inches high.

1/4 inch copper pipe.

Capacitor:      It is a glass foil capacitor and I think about .0053 uF

----------      The glass size is 12 inch x 16 inch x .16 inch

The aluminium foils is 8 inch x 12 inch.

Spark gap:     It's made with iron nails put in series at a very close

---------      spacing about .08 inch apart, it's about 16 nails.

It looks like this:              /-

|--------------------------/|

|                         / |

|   ------------------\  /  |

|  |                   \/   |

|  |                    |   |

|  |                   /\   |

|  -------------------/  \  |

|                         \ |

|--------------------------\|

\+

PS! I hope you can make the picture out.

Anyone who has a tip can send it to me please.

Anyone who had seen a magnifier in work please tell me.

Thanks

Richard Smit

EMAIL: esmit@ilink.nis.za

South Africa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Free Resonator List

Date: Wed, 11 Jun 1997 07:32:03 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Wed, 11 Jun 1997 09:32:03 +0200 (SAT)

Subject: Free Resonator List

Hi all

Can anyone tell me how to join the Free Resonator List.

Thanks

Richard

Richard Smit

EMAIL: esmit@ilink.nis.za

South Africa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Free Resonator List

Date: Wed, 11 Jun 1997 13:55:11 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 11 Jun 1997 06:55:11 -0700

Subject: Re: Free Resonator List

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Richard <esmit@ilink.nis.za>

> Date: Wed, 11 Jun 1997 09:32:03 +0200 (SAT)

> Subject:

>

> Hi all

>

> Can anyone tell me how to join the Free Resonator List.

>

> Thanks

> Richard

> Richard Smit

> EMAIL: esmit@ilink.nis.za

> South Africa

>

Richard,

I don't believe it exists any more... Sorry!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Help

Date: Wed, 11 Jun 1997 14:00:18 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 11 Jun 1997 07:00:18 -0700

Subject: Re: Help

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Richard <esmit@ilink.nis.za>

> Date: Wed, 11 Jun 1997 09:28:56 +0200 (SAT)

> Subject: Help

>

> Hi, you all.

>

> Does anyone know a formula to calculate the coeficient of coupling.

>

> Will someone please check if I can make my coil perform better.

> Here is my coil specs

>  Transformer: It's a 15000 VAC rated 30 milli-amps transformer.

>

>

>         Secondary Coil:  It has about 723 windings of 21 AWG wire.

>         --------------   The diameter of the coil is 4.4 inch.

>                          The height of my coil is 24 inch.

>

>         Primary Coil:    It has 16 turns but the resonant point is at about the

>         ------------     eighth turn.

>                          The diameter is 8.8 inch.

>                          The height is 6 inches high.

>                          1/4 inch copper pipe.

>

>         Capacitor:       It is a glass foil capacitor and I think about .0053 uF

>         ----------       The glass size is 12 inch x 16 inch x .16 inch

>                          The aluminium foils is 8 inch x 12 inch.

>

>         Spark gap:       It's made with iron nails put in series at a very close

>         ---------       spacing of .08 inch apart, it's about 16 nails.

>

>            It looks like this:              /-

>                 |--------------------------/|

>                 |                         / |

>                 |   ------------------\  /  |

>                 |  |                   \/   |

>                 |  |                    |   |

>                 |  |                   /\   |

>                 |  -------------------/  \  |

>                 |                         \ |

>                 |--------------------------\|

>                                             \+

>         PS! I hope you can make the picture out.

> Anyone who has a tip can send it to me please.

> Anyone who had seen a magnifier in work please tell me.

>

> Thanks

> Richard Smit

> EMAIL: esmit@ilink.nis.za

> South Africa

>

Richard,

Unlike finding the inductance of primaries or secondaries, there's no

[simple] formula for calculating the mutual inductance or coupling

coefficients. You can increase k by raising the primary versus the

secondary, but you need to be carefull that you don't raise it so high

that you get primary to secondary flashovers. There are ways of

measuring it indirectly if you have a signal generator, but I don't

think that coupling is your problem...

Your coil's performance will be more governed by the quality of the tank

capacitor and the spark gap. Both of these are the weak links in your

system. Glass is a very lossy dielectric - even more so if simply run in

air. Corona and dielectric losses combine to rob you of power that would

otherwise go into the secondary discharges. You may get some improvement

by immersing your glass cap under mineral oil. However, I'd recommend

that you look into building an oil-immersed polyethylene cap to fix the

lossiness of the dielectric itself.

The second problem is that you need to have gap electrodes that won't

heat up - nails won't do it!. Sections of 1" (or larger) diameter pieces

of copper pipe with high-velocity air flowing through the gaps between

them will work much better. Copper is a much better electrode material

since it conducts heat (and electricity) so well.

Descriptions of how to make better caps and gaps can be found at the

Tesla site at:

ftp://ftp.funet.fi/pub/sci/electrical/tesla

Safe coilin' to you, Richard!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Small Cap?

Date: Wed, 11 Jun 1997 14:43:17 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Michael Huff)

Date: Wed, 11 Jun 1997 10:43:17 -0400

Subject: Small Cap?

I'm looking to build a small .01uF cap for a small self-contained coil

(I want something that I can take places).  I was thinking of building a

rolled polyethylene cap using 3" PVC as the case and painters drop cloth

(4 mil folded several times).  Has anyone ever used this material for

capacitor dielectric?  I'm worried about puncturing it with 10kV (two

5kV 60mA neons).  The plates will just be common aluminum foil.

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: IQ

Date: Wed, 11 Jun 1997 15:07:17 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Wed, 11 Jun 1997 17:07:17 +0200 (SAT)

Subject: IQ

Hi there guys.

I have constructed a Tesla coil and it's giving 6 inch sparks and I want to

create longer sparks. I just want to know what is the maximum sparks a

person can get from a 15000 VAC 30mA transformer. You see, I have only been

interested for about a 3/4 year and now I want to make the best of it and

learn alot from you guys. If you have questions you can ask me maybe I can

tell you something that I know about coils.

Questions:

----------

1. Do anyone of you know how to create ball lightning?

2. Do anyone of you know the work Tesla did in Colorado Springs?

3. Have anyone of you seen a magnifier coil in action?

4. Will those who have files on Tesla coils please send it to me?

5. Will you who know what experiments to do with Tesla coils let me know.

6. Does anyone have a file on how a Magnifier Coil works incase you have

will you please send it to me.

7. If there's someone who has joined the TCBA and don't want their old

magazines    tell me if you don't want them. If you don't want to give them

away is their    a way of you scanning it and sending it to me. I would be

very greatfull. The    reason I'm asking you all is because I live in South

Africa and to buy things    from Amerika is very expensive.

8. Anyone who knows an experimenter with the name Steve, he also work with

magnifiers please let me know.

9. Those who has all the formulas for Tesla coils please send them to me.

I made a decision that is going to be great. The decision I made was that I

want to become a great expert on Tesla coils just like Richard Hull and

those other famous coilers. For this task I need all of you to help me, if

you would like that. Would those who would like to help me send all info on

Tesla coils for example about capacitors, magnifiers, experiments and

everything about Tesla coils. I would be greatfull if anyone of you or all

of you will help me. Thanks!

!!! If it weren't for TESLA we wouldn't have been such great coilers !!!

Thanks

Richard

EMAIL: esmit@ilink.nis.za

South Africa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Robust Capacitors

Date: Wed, 11 Jun 1997 17:22:13 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 11 Jun 1997 13:22:13 -0400

Subject: Robust Capacitors

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Small Cap?

> Date: Wednesday, June 11, 1997 10:43 AM

>

> From: msnock@theonramp.net (Micheal Huff)

> Date: Wed, 11 Jun 1997 10:43:17 -0400

> Subject: Small Cap?

>

> I'm looking to build a small .01uF cap for a small self-contained coil

> (I want something that I can take places).  I was thinking of building a

> rolled polyethylene cap using 3" PVC as the case and painters drop cloth

> (4 mil folded several times).  Has anyone ever used this material for

> capacitor dielectric?  I'm worried about puncturing it with 10kV

> (two 5kV 60mA neons).  The plates will just be common aluminum foil.

>

> Mike

Mike,

You will need about 60 mils total thickness if you want to build a

robust capacitor. 4 mil stuff is awfully thin. Look around in places

that sell general building supplies and you should be able to find at

least 6 mil poly.

I would recommend building two rolled caps with 5 sheets of 6 mil

poly as the dielectric. You would have to make TWO .02 mfd capacitors

and then connect them in series. That would give you .01 mfd as you

requested. I think you will find that 5 layers is a lot easier to

handle than 10 layers!

An alternative is to build a ten stage serial capacitor with each

stage using just a single thickness of dielectric. The tradeoff in

building a ten stage serial capacitor is that it ends up being a

single unit and only uses single thicknesses of dielectric. The

downside is that it will have TEN stages, and each stage will be

effectively a .1 mfd at 1,000 volt capacitor. The stages are serially

connected DURING construction. In case you are interested in this

form of construction I am appending at the end of this post a post I

sent to the Tesla List detailing how the multi-stage serial rolled

capacitor is built.

Fr. Tom McGahee

Subject: Multi-Rolled Cap

Date:  Tue, 13 May 1997 15:41:01 -0400

From:  "Thomas McGahee" <tom_mcgahee@sigmais.com>

To:  <tesla@pupman.com>

Fellow Coilers,

Fr. Tom here with what I hope will be a useful addition to our set of

techniques for making capacitors.

I note that a number of coilers are attempting to use thinner than

usual polyethelene to make rolled capacitors. It is difficult to roll

too many layers at once! But you don't really HAVE to. Here is a

method for making multi-section ROLLED capacitors that reduces the

number of poly sheets that must be handled.

*Note: wherever the text refers to a sheet of poly, you MAY in fact

be using two or more sheets. For example, your "sheet" may be four

layers of 6 mil poly.

First the facts: multi-section caps are actually more robust than a

single section cap. The high voltage rating is attained by having

many lower voltage sections connected in *SERIES*.

You can make a "rolled" multi-section capacitor that contains two or

MORE sections in a SINGLE roll. When completed it will have only TWO

connections. I will attempt to explain the construction of such a

capacitor, as I have never heard anyone mention the construction

details of such a capacitor on the Tesla list.

Let's assume you wanted to make a FOUR section capacitor using one

foot wide sections of 30 mil sheet polyethelene. Let's further assume

that you have two long sections of 1 foot wide polyethelene already

cut and loosely rolled up, and two rolls of aluminum foil that you

have already cut down to a width of 10 inches.

Let's wind the capacitor on a piece of 1" diameter copper pipe that

is 1 foot 2 inches long. First TWIST the end of a bare copper wire

(at least size #10) that is about ten inches long around one end of

the copper pipe, about 1/4" from the edge. SOLDER the wire down well,

making the solder flow as evenly as you can ALL AROUND the circle of

wire so that you have a connection with good surface area. The free

end of the wire will eventually be trimmed to a shorter length and

used to interconnect the finished capacitor to the tank circuit. The

copper pipe is going to provide us with a first class electrical

connection to one side of the capacitor.

To make it easier to manipulate the capacitor roll as we assemble it,

shove a broomstick or wooden or metal pole or pipe up inside the

copper pipe and then use wedges, shims, or whatever it takes to get

the pole temporarily firmly attached to the copper pipe. This will

give us a "handle" that we can use so that the turning of the roll

can be done more easily.

Use scotch brite or steel wool or sandpaper to clean the outside of

the copper tube so that it can make a GOOD electrical connection all

along its outer surface.

Take one of the aluminum foil ends and center it on the copper pipe.

Wrap several turns of foil TIGHTLY around the pipe. This gives us the

good electrical connection between this first foil sheet and the

copper pipe. Use a strip or two of 14" long duct tape to tape the

entire width down. We will call this piece of foil FOIL 1. Tape the

end of one of the polyethelene rolls ON TOP of Foil 1 using a piece

of 14" duct tape. This will be poly roll 1. Tape the end of poly roll

2 even with the end of poly roll 1. ROLL the assembly slowly around

THREE full turns.

Now insert the end of foil 2 *in between* poly roll 1 and poly roll

2. Use a 10" strip of tape to hold it in place. You NOW have a

sandwich that consists of foil 1, poly 1, foil 2, and poly 2. Slowly

roll the assembly, making sure that the foil sheets stay centered,

and the poly sheets are rolling up smoothly and evenly. Apply a

slight tension to the assembly as you roll it, and smooth out any

wrinkles if they begin to develop.

Ocassionally check the capacitance between the copper pipe and foil 2.

When the FIRST section reaches a value that is FOUR TIMES the desired

final value, pause. (4/1=4)

*Note that to get a valid value for the capacitance rolled so far you

must have the remaining foil in two rolls that are separated by about

a foot.

Since section #1 began with foil 1, carefully cut foil 1 free. Foil 2

remains intact at this time. Roll around at LEAST 1 complete turn of

poly 1, foil 2, poly 2. Now re-insert foil 1 (use tape if needed),

and continue to roll the capacitor.

Ocassionally check the capacitance between the copper pipe and foil

1.

When the combined FIRST and SECOND sections reach a value that is TWO

TIMES the desired final value, pause. (4/2=2)

Since section #2 began with foil 2, carefully cut foil 2 free. Foil 1

remains intact at this time. Roll around at LEAST 1 complete turn of

poly 1, poly 2, foil 2. Now re-apply foil 2 (use tape if needed), and

continue to roll the capacitor.

Ocassionally check the capacitance between the copper pipe and foil 2.

When the combined FIRST, SECOND and THIRD sections reach a value that

is FOUR-THIRDS TIMES the desired final value, pause. (4/3=four

thirds)

Since section #3 began with foil 1, carefully cut foil 1 free. Foil 2

remains intact at this time. Roll around at LEAST 1 complete turn of

poly 1, foil 2, poly 2. Now re-insert foil 1 (use tape if needed),

and continue to roll the capacitor.

Ocassionally check the capacitance between the copper pipe and foil 1.

When the comined sections reaches a value that is the SAME as the

desired final value, STOP.

Since section #4 began with foil 2, carefully cut foil 2 free. Foil 1

remains intact at this time. Roll around at LEAST *TWO* complete

turns of poly 1, poly 2, foil 2. The extra is insurance. If you are

using particularly thin poly, you may want to increase the number of

outer layers.

You now need to attach a connection to the remaining intact foil. (In

this case Foil 1). This will form the second capacitor lead. One good

method of getting a low impedance connection is to use a strip of

flashing that is about 1" wide and as long as needed. Extend the

remaining foil a couple of extra inches and then *tightly* roll the

flashing inside the extra aluminum foil. I like to have a piece of

the flashing extending below that I bend over after wrapping. This

prevents the flashing from pulling free accidentally. Use duct tape

to hold everything together. Wrap AROUND the entire assembly. Several

layers here makes a lot of sense. Cut away any remaining poly. You

might want to wrap a protective layer using a piece of thicker poly,

if available. Use more duct tape to hold the outer layer together

firmly.

You now have a single rolled capacitor that is actually FOUR

series-connected sections.

You can extend the basic idea for more or less sections. The

dimensions given are just for the sake of clarity.

If you are rolling along and you come to the end of a poly roll, you

can carefully overlap by AT LEAST 1/2 turn of poly, and then continue

as usual.

It is higly recommended that the capacitor be immersed in oil, as is

usual, and vacuumed down to help remove bubbles. But feel free to

modify the rolled cap as needed. What this capacitor assembly method

seeks to do is make the job of making multisection capacitors a bit

easier by reducing the number of layers of poly that are being

rolled. The finished cap will require the usual break-in procedure

before you attempt to run it full power.

Please note that if you do not have a capacitance meter, you can use

the actual LENGTH of each section's FOIL to determine the approximate

capacitance. Not as accurate, but close enough.

Hope this concept helps somebody.

Fr. Tom McGahee

********

Here's another post I wrote on how to make robust FLATE PLATE

capacitors.

********

Subject:  Idea for Capacitor

Date:   Tue, 10 Jun 1997 19:13:11 -0400

From:  "Thomas McGahee" <tom_mcgahee@sigmais.com>

To:  <tesla@pupman.com>

Fellow Coilers,

Fr. Tom here again with some more food for thought.

As I mentioned in one of my previous posts, I am going to be away

from my usual residence (I live in a school building in Paterson, NJ)

for the summer. I *hope* to be able to get online at my summer

assignment (giving five one-week retreats to groups of sisters), but

I cannot be sure about getting online.

So, I want to pass along an idea I have been working on, but will

definitely *not* be able to do any final work on until after the

summer. I am passing it on now in the hopes that someone else out

there who has the time will try the idea out and report back to the

rest of us.

We all know that the major failure mechanism in our Homemade High

Voltage Capacitors for Tesla Coil use is those stinking air bubbles

that get trapped between the plates. As you may remember from

previous posts, I have had good success with using paper between the

plates so as to actively wick oil in between the plates. In

multi-layer dielectrics I also use paper between the dielectric

layers, and not just between the plates and the dielectric.

I have noticed that one of the major frustrations for many coilers is

the long wait they have after building a capacitor as they wait for

the bubbles to subside.

I think I may have a solution that will work for flat plate

capacitors at least. What we want to do is build a fairly regular

flat plate capacitor, but with three special features.

Feature One: The use of oil-soaked paper placed between all capacitor

surfaces, whether metal to dielectric, or dielectric to dielectric.

Pre-soaking of these pieces of paper may be just a slight bit on the

messy side, but will allow us to ensure an initial starting condition

that is already oil-rich exactly where we need the oil the most.

Paper pieces should be smooth, absorbent, clean paper. Kraft paper

used to wrap packages works well. Paper should be soaked for at least

an hour before use. Before soaking in oil, cut the paper so that it

is AT LEAST as large as the polyethelene sheets. A slight overhang

along all edges is actually useful. I would recommend 1/4" of

overhang on LARGE size plates, and at least 1/8" overhang on smaller

size plates.

Feature Two: Build up the flat plate capacitor on a COMPRESSION

FRAME! Imagine a flat plate capacitor sandwiched between two very

rigid non-conducting plates such as 3/8" plexiglass or 1/4"

fiberglass board. In a pinch you could use 3/8 or 1/2" plywood even,

although the thought makes me want to barf. I would envision using

something rigid which would NOT soak up oil.

Note: Don't make the compression plates too thin. Remember that they

are there to allow us to apply fairly decent pressures to the

capacitor. Skimping here can cause the material to crack. Oh yes, do

not put holes too close to the edge of the plexiglass or the pressure

make cause cracking from the holes to the edge. I planned to use a

metal washer of decent size anywhere a bolt head or nut was pushing

against the plexiglass.

This frame would have long bolts holding the two frame pieces

together, with the capacitor sandwiched in between. I would envision

using bolts every inch or so along the WIDTH of the compression

frame. My own design plans call for me to use the bottom compression

frame as the building support for the actual capacitor.

Feature Three: The inside face of *the Top* compression plate should

*NOT* be flat! To force the stinking air bubbles *OUTWARD* and away

from the center of the capacitor can only be accomplished if we have

MORE pressure along the center of the capacitor.

We can accomplish this by making the center of *one* facing

compression plate thicker than the ends. An easy way to do this would

be to cut out about five pieces of poster board or thin vinyl or

WHATEVER for each compression plate. The thicker the capacitor, the

thicker this flexible material has to be. I suggest something that is

impervious to the oil, but in a pinch posterboard is OK.

The largest should be about the same size (length and width) as a

sheet of dielectric. (By width I mean the shorter of the two

dimensions)

The next sheet should be about 20% smaller along the LARGEST

dimension of the capacitor, and the same width as the smallest

dimension of the capacitor. As each smaller piece of poster board is

cut, keep the width constant, and the length reducing by about an

additional 20% with each new sheet. Obviously you could also use

thinner sheets, more of them, and therefore make the size reduction

per sheet smaller, such as by 10%. The numbers used in this example

are just for illustration purposes. Once you understand the basics of

what we are proposing, modify as necessary!

Note: If the above technique is applied to BOTH compression plates

instead of just the top one, then the material can be thinner, as it

will be basically doubled by being made in two sets. (It will be more

work, but the end result will be slightly better, as forces will be

more evenly distributed.)

Before attaching the top compression plate, lay the cut strips down

on the top of the capacitor. They should all be the same width as the

width of the capacitor. The SMALLEST one goes down first. Center it.

Place the next larger strip centered over the first strip. The widths

should all be the same, and lined up with the width of the dielectric

sheets. Keep adding the larger pieces until all are centered and in place.

After the capacitor itself is built up, the compression frame top

would be put on and all the bolts tightened a little bit at a time. I

would begin by finger-tightening only and adjusting until  the frame

was levelled out with equal distances on each bolt being used. Then I

would use a wrench or nut driver to tighten each bolt in turn, by

just a quarter turn at a time. I would continue this until the whole

thing was level and extremely tight. Note that at this time most of

the oil will have been squeezed out. That is exactly what we want.

Because along with most of the oil, almost ALL of the stupid air

bubbles would have been compressed out.

Because of the extra pieces of flexible material, the pressure

applied at the center of the capacitor will be greatest. Thus, as you

tighten the assembly, air and oil are first driven away from the

center and outward. As compression continues the bubbles are forcibly

removed, but the absorbent paper will ensure that oil will still be

available between each sheet of dielectric.

******

The BEST thing to do with this assembly is to immerse it in oil. Note

that it will be instantly ready to be used. It should not require any

waiting, since all the air has been forcibly removed.

By the way, if the final stages of tightening are done while the unit

is immersed in oil, then you will have an even better final product.

This is, however, just a tad messier. The capacitor can remain flat.

It does not need to be placed sideways. It can be mounted in ANY

orientation.

Taking care of it would mean just swishing the oil around once in a

while to disperse surface bubbles. If you wanted, you could loosen

all the bolts, wait a minute or two, and then re-tighten them. This

would cause fresh oil to circulate and replace the old oil.

This capacitor would *NOT* need any form of vacuum to make it

bullet-proof.

Because of the compression frame, the capacitor would be robust

enough that it could be moved and handled fairly roughly. It is also

interesting to note that for transporting such a capacitor, you could

drain the free oil out and into another container. Once you got the

capacitor to its destination you would just pour the oil back in,

wait a minute or two, swish the tank to allow surface bubbles to

break free, and fire up your coil almost immediately!

*****

I hope that someone out there builds a capacitor like this and gets

back to the rest of us on their experience with it.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Info on Oil

Date: Wed, 11 Jun 1997 20:14:20 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 11 Jun 1997 16:14:20 -0400

Subject: Info on Oil

Fellow Coilers,

Certain questions tend to come up again and again on this list, and

the topic of oil for capacitors is one of them. In the interest of

gathering and presenting as much information as I could find on this

topic, I present this post in the hope that it will be found useful

by some. It includes parts of posts by Richard Quick, Mike Hammer,

Ralph Down, and many other coilers on the Tesla List (including

myself). I have not marked down who is responsible for what here,

simply because my clip file of data from which this was assembled in

many cases has only the data, and not the name of the person who

posted it. I hope no one is offended by having their name

inadvertantly left out.

If you have additional comments to add to this modest compendium,

please e-mail me at tom_mcgahee@sigmais.com and I will try to keep

this compendium as up-to-date as I can. If I get any significant

e-mails I will repost the updated version when appropriate.

Please note that large sections of this post were taken from posts

originally sent to the Tesla List by many different people. So if

some of the paragraphs sound awfully familiar, it may just be because

it is based on sometheing *you* sent to the Tesla List.

As I have said before, the really good stuff gets recycled again and

again.

Beginning of Discussion on Oil

*******

Coilers use oil to reduce corona and increase High Voltage insulation

in such applications as Capacitors and Transformers.

Mineral oil is a rather generic term given to a whole class

of light oils. There are literally dozens of types.

Veterinary grade mineral oil can be had rather cheaply

in agricultural areas. Sold in 2 and 5 gallon pails.

Pure USP or veterinary grade mineral oil will work fine. The price

varies based on purity and quantity. You can get USP mineral oil at

any pharmacy for less than $2 a pint. While that seems reasonable, it

is much better to buy whatever you use by the gallon or five gallon

container if you want to get a really reasonable price.

Some coilers have even used mineral oil that had a fragrance added.

In *general* try to stay away from all forms of additives, because

you just don't know what properties they bring when used for High

Voltage insulation.

Buy the Transformer Oils from oil distributors. Just tell them you

need a good transformer oil that is relatively inexpensive. Many

prefer transformer oil over mineral oil because it is specifically

made for insulating high voltage, has ALL water rigorously removed,

and is a very THIN oil, so it penetrates MUCH better than mineral

oil.

Whenever possible, go for the Transformer Oil rather than the generic

Mineral Oil. The thinner the oil the better, as thick oil makes

bubble removal harder.

Texaco, Mobil, Conoco, and many others also make transformer oil, but

I don't have any specific info on these products in my files (yet).

Some coilers have used refrigeration oil and silicone oils. Ralph

Down used SUNISO refrigeration oil (bought from local refrigeration

repair shop for $30 for 4 litres.) He was pleasantly surprised to

find that the high dielectric constant of this oil actually DOUBLED

the capacitance value of his homemade capacitor.

Coilers have used drugstore grade mineral oil and animal grade

mineral oil. Animal grade oil is sold as livestock laxative.

Those in agricultural areas can find it easily enough, but

city types might have a problem.

For what it's worth, I built *my* capacitors with Exxon Univolt 61. 5

Gallons cost me $34.50. That turned out to be a most reasonable price

for an EXCELLENT true transformer oil. The dielectric constant of

this stuff was almost exactly the same as the polypropylene that I

was using. By the way, when I did my yellow page searches, I was

quoted prices ranging from $34.50 to $48.00 for the exact same

product. So check out multiple sources before buying!

As to using stuff like motor oil, well, it has been done. Read the

label. Go for the stuff that has the LEAST amount of additives. Most

motor oils are too thick. Go for the lightest weight oil you can get.

Remember, you want the stuff to infiltrate every nook and cranny of

your capacitor. Something the consistency of molasses isn't going to

do you much real good!

In general, stay away from food oils. They tend to go rancid and have

too much water content.

But never be afraid to experiment on a *small* scale. Make up a small

flat plate capacitor consisting of just two plates of relatively

small size and EXPERIMENT. Determine the breakdown voltage yourself,

if you have to. The bottom line is not whether all the bigwigs would

use it, but does it work for *you*? And if you find a good reasonably

priced source for Castor Oil, please let the rest of us know about

it! (Castor Oil has a nice high dielectric constant, more than double

the value of transformer/mineral oil).

All of the following oils have been used successfully by coilers on

the Tesla List.

Mineral Oils:

Shell MVI-65

Tulco Snaplube 100

USP Mineral Oil (Laxative) (available at any pharmacy).

Veterinary Grade Mineral Oil (Laxative)

Commercial transformer oils are excellent! Examples would be:

Shell Diala-X or Shell Diala-AX

Esso VoltEsso 35

Exxon Univolt N60

Exxon Univolt N61

Exxon Univolt 33

r-Temp (Paraffinic - Best for capacitors, but more expensive)

Refrigeration Oil:

SUNISO

There are many other brands of silicone oil

Motor Oils:

Some coilers have used LIGHT GRADES of Motor Oil successfully

Use ones with least number of additives.

Those of you who are scroungers at heart might want to try your local

power utility. Some people have had success getting a couple of

gallons of transformer oil from their local power company cheaply or

even free.

When looking to locate a source for *any* item, the FIRST thing is to

use the yellow pages of your phone book. In this case you would look

under OIL. Find the advertisements that specifically mention

transformer oil. Try these first. Call at a time of day when the

person working there is not going to be too busy. That way they will

be more inclined to spend some time talking to you on the phone.

Some people on the other end of the phone will be very knowledgeable

and helpful. Other persons won't have much of an idea of exactly what

you want unless you give them specific information such as "do you

carry Exxon Univolt 61 transformer oil?" Remember, sometimes the

person at the other end of the phone knows more than *you* about the

topic you are discussing (which at the moment is transformer oil, not

Tesla Coils). If it seems like the person is pretty knowledgeable,

then you might divulge the purpose for which you want to use the

transformer oil... you might be surprised and find that he can give

you something even better than what you were asking for.

Besides checking out the Oil Suppliers and Distributers, you might

also discretely try to find out what KIND of customers the Oil

Suppliers are supplying all this transformer oil to. You can

sometimes get one of these customers to sell you something less than

a full five gallon container at their cost (which is usually going to

be LESS than what the supplier quoted *you*, because many customers

buy the stuff 100 gallons or more at a time.)

It is often quite useful to tell people what you are going to use the

oil for. This is especially true if you go some place in person. Take

along some pictures of a Tesla Coil in operation. If you don't have

any good pictures of one of your own coils, download one of somebody

else's magnificent coils and use *that*. Depending on a great many

factors, you can often get the person you are talking to to either

sell you the stuff at cost, or give you a small amount for free.

Below is a copy of a table produced by Richard Quick some time ago.

It includes the dielectric constants and breakdown voltages

per mil of a number of substances, including several kinds of oil.

I could have cut this table down to just include oils, but the

table will have greater utility for all of us if it is kept intact.

=====================================================================

Dielectric strength   Puncture Voltage

Material                       K           per Mil(0.001 inch)

---------------------------------------------------------------------

Miscellaneous

-------------

Vacuum                            1.0

Paper, bond                       3.0            200

Paper, Royal Grey                 3.0            200

*   Paper, telephone, treated         2.5 - 4        200 - 250

Paper, Parafin Coated             2 -3.5

Paper, Kraft                      2.2

Oil, Castor                       4.67

Oil, Mineral, Squibb              2.7            200

Oil, Mineral                      2.2

*   Oil, Transformer                  2.1 - 2.5      75

Rubber                            3.0

*   Rubber, Hard                      3.0            160 - 500

1   Rubber, Vulcanized                3.2 - 3.9

Fibre                             5.0 - 7.5      150 - 180

Fibre, Red                        5.0

Mica                              4.5 - 8.0      3800 - 5600

Mica, Ruby                        5.4            3800 - 5600

Quartz                            3.8 - 5.0      1000

*   Quartz (Fused)                    4.2            150 - 200

Shellac                           2.5 - 4.0      200 - 400

Spar Varnish                      4.8 - 5.5

Steatite, low loss                5.8            150 - 315

Steatites (Magnesium silicate,etc)5.5 - 7.5      200 - 300

2   Cambric (Varnished)               4.0

Alsimag 196                       5.7

3   Gutta Percha                      4.0

4   Amber                             3.0 - 7.0

Resin                             2.48 - 2.57

Enamel                            5.1            450

Mycalex                           7.4            250

Silicone RTV                      3.6            550

Wood                              2.0 - 5.2

*   Wax (Parafin)                     2.1 - 2.5      250 - 450

Beeswax                           2.9 - 3.0

5   Slate                             7.0

Barium titanate(25 C)             1200

Bariam titanate                   6000

Titanium dioxide                  125

Cellulose acetate                 3.3 - 3.9      250 - 600

6   Casein, Moulded                   6.4

A   Polytetraflourethylene            2.0

Aluminum oxide                    8.7

Tantalum pentoxide                22

Glass

-----

Glass                             4.8 - 10       300

Plate Glass                       6.8 - 8.4

Pyrex Glass                       4.8 - 10       335

Window Glass                      7.6 - 7.8      200 - 250

Ceramics

--------

Cordierite ceramics               5.0 - 5.5      100

Magnesium titanate ceramic        12 - 18        150

Porcelain                         5.1 - 7.5      40 - 280

Titanium dioxide ceramic          70 - 90        100

Titanium-zirconium dioxide ceramic40 - 60        150

Plastics

--------

Bakelite                          4.4 - 5.8      300

Bakelite, Mica filled             4.7            325 - 375

Epoxy Circuit Board               5.2            700

7   Formica                           4.6 - 4.9      450

Nylon (lowest values of 3 types)  3.2            407

PVC (rigid type)                  2.95           725

Plexiglass                        2.8            450 - 990

Polyethylene                      2.2 - 2.3      450 - 1200

Polycarbonate (Lexan)             2.96           400

Polyethylene Terphthalate (Mylar) 3.0 - 3.1      7500

Polystyrene                       2.5 - 2.6      500 - 700

Teflon                            2.1            1000 - 2000

Gases

--------

Air (dry air at 1 atm)            1.0006         30 - 70

*   Air (20 atm, 19 deg. C)           1.0108         500

*   Carbon dioxide ( 1 atm, 0 deg. C) 1.000985       36

Carbon dioxide (20 atm, 15 deg. C)1.020

*   Hydrogen (1 atm,0 deg.C)          1.000264       26.1

Liquids

--------

Ammonia (liquid)                  22

Benzene                           2.28

8   Carbon tetrachloride              2.24

9   Chlorinated diphenyl              6.5

Ethyl Alcohol (O C)               28.4

Ethyl alchohol (20 deg.C)         25.8

Methyl alchohol                   33.1

Water (distilled)                 80 - 81

LEGEND:

* = Measured in Kilovolts per centimeter. All others are Volts

per mil (.001 inch) unless otherwise stated.

1 = Vulcanized means it has been melted, or heated in some

way.

2 = Cambric is a finely woven white linen or cotton fabric.

3 = Gutta Percha is a rubbery substance made from the latex of

tropical trees and is used in insulation, waterproofing,

and dentist use it in thin sheets sometimes when working

on teeth.

4 = Amber is a hard, translucent, yellow, orange, or brownish yellow

fossil resin, used in making ornamental objects like jewelry.

5 = Slate is a fine grained metamorphic rock that splits into

thin smooth faced layers. Black Boards for writting are

made of this. Also used as roofing material in some areas.

6 = Casein is a white, tasteless, oderless milk and cheese protein

used in the manufacture of plastics, glues, paints, and food. The

word 'moulded' means it has been shaped by a mold.

7 = Formica is a trademark for any of various high-pressure laminated

plastic sheets of melamine and phenolic materials used for

chemical and heat-resistant surfaces.

8 = Carbon tetrachloride is a very toxic substance. It has also been

shown to cause cancer in lab animals. It is banned in most labs.

It is a liquid that was used as a strong solvent.

9 = Chlorinated diphenyl is a liquid dielectric that is used to

impregnate Kraft paper in small A.C. capacitors. This is a PCB

and may cause cancer, handle carefully.

A = Polytetraflourethylene films retain good properties even at 200

degrees Celsius (200 C).

atm = atmospheres (pressure of air at sea level is 1 atm).

deg.C = degrees Celsius.

Warning - Some liquids and gases listed may be explosive under the

right conditions. Many solids can catch fire and burn. Use

CAUTION and GOOD COMMON SENSE.

Note: Some books gave very different values for each substance so I

have given you the highest and lowest values reported. The

values will depend on the purity of the substance your using.

If you know your substance is very pure then use the higher

value reported. If you know your substance is contaminated or

of poor quality then use the lower values reported. If you have

unknown purity then use the average of the values given.

**** End of Richard Quick's Table

If you have anything to add to this compendium, please e-mail it to:

tom_mcgahee@sigmais.com

Hope this helps someone out there. File it away for later.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Small Cap?

Cc:

Bcc:

X-Attachments:

At 02:43 PM 6/11/97 GMT, you wrote:

>From: msnock@theonramp.net (Micheal Huff)

>Date: Wed, 11 Jun 1997 10:43:17 -0400

>Subject: Small Cap?

>

>I'm looking to build a small .01uF cap for a small self-contained coil

>(I want something that I can take places).  I was thinking of building a

>rolled polyethylene cap using 3" PVC as the case and painters drop cloth

>(4 mil folded several times).  Has anyone ever used this material for

>capacitor dielectric?  I'm worried about puncturing it with 10kV (two

>5kV 60mA neons).  The plates will just be common aluminum foil.

>

>Mike

>--

>

Mike I didnt notice this until it was brought to my attention

but you mention above using two 5kv neons in series to get 10kv.

While this can be done its not a recommended procedure. The cases

will no longer be at ground potential and will be hot.

This could lead to early failure and is an unsafe condition.

If you do this be quite careful and insulate the the neon cases

from ground and you.

TTYL!

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Small Cap?

Date: Wed, 11 Jun 1997 20:42:01 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Wed, 11 Jun 1997 16:42:01 -0400

Subject: Re: Small Cap?

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Thu, 12 Jun 1997 11:45:35 -0500

> Subject: Re: Small Cap?

>

> At 02:43 PM 6/11/97 GMT, you wrote:

> >From: msnock@theonramp.net (Micheal Huff)

> >Date: Wed, 11 Jun 1997 10:43:17 -0400

> >Subject: Small Cap?

> >

> >I'm looking to build a small .01uF cap for a small self-contained coil

> >(I want something that I can take places).  I was thinking of building a

> >rolled polyethylene cap using 3" PVC as the case and painters drop cloth

> >(4 mil folded several times).  Has anyone ever used this material for

> >capacitor dielectric?  I'm worried about puncturing it with 10kV (two

> >5kV 60mA neons).  The plates will just be common aluminum foil.

> >

> >Mike

> >--

>

> Mike,

> First of all a .01 mfd cap can hardly be classified as small.

> My .01 mfd plate style cap is in a tank that measures 13 X 17 X 3

> inches.

Ok.  Those dimensions sound good.  I am assuming it is a vertical stack

since you don't have a vacuum pump (neither do I).  Flat plate caps are

good since their is virually no inductance which would limit the size of

the cap.  I just want a cap that will fit under a 18" stand

(primary/secondary will be on top and all the guts ie. my two neons,

cap, and protection circuitry will be in the middle.)

> A rolled cap may be a bit more compact.

They are naturally more compact, but they also require two sheets of

dielectric.

> I used 4 mil poly in my first plate style cap. I used 2 layers for 8 mils

> and constructed 6 caps which I connected in series to give an equivalent

> of 48 mils of dielectric.

> You can use your 4 mil poly in a rolled cap but you would

> need from 48-60 mils total dielectric thickness to stand

> up to 10KV well.

> I would consider making 2 identical .02mfd caps using 24 mils

> (6 layers of 4 mil) of poly and using them in series.

> This will give you the equivalent of 48 mils and .01 mfd and each

> cap only has to handle half the voltage.

Good idea.  I think I'll try it.

> If you can find some 6 mil poly use it. This will cut down

> on the number of layers you need.

>

> The biggest problem is going to be trapped air in the many layers of

> poly. If you have access to a vacuum pump then make your tanks

> air tight and pull a vacuum on it to remove the air.

> If your like me and dont have access to a vacuum pump then you will

> have to let the air come out on its own. This can take quite a while

> for rolled caps.

I think I'll go with your idea of a stacked plate cap.  Could you run

down the specs on your cap?  Plate dimensions, dielectric dimensions,

plate material, etc.

> The use of regular transformer oil instead of mineral oil will help

> as transformer oil is very thin and will allow the air bubbles

> to come out easier.

And besides, transformer oil is terribly cheap if bought from the power

company.  Got a gallon once for about $3.00

> Building high quality caps is a major project. I've spent

> more time and money building caps than all the rest of my

> coil components combined.

I know what you are saying.  My bipolar right now is running on Leyden

jars made from those huge iced tea jars.  Capacitor design and

construction is probably the most annoying and troublesome aspect of

building a good coil.

Thanks,

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Small Cap?

Date: Wed, 11 Jun 1997 21:19:55 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Wed, 11 Jun 1997 17:19:55 -0400

Subject: Re: Small Cap?

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Thu, 12 Jun 1997 14:21:23 -0500

> Subject: Re: Small Cap?

>

> At 02:43 PM 6/11/97 GMT, you wrote:

> >From: msnock@theonramp.net (Micheal Huff)

> >Date: Wed, 11 Jun 1997 10:43:17 -0400

> >Subject: Small Cap?

> >

> >I'm looking to build a small .01uF cap for a small self-contained coil

> >(I want something that I can take places).  I was thinking of building a

> >rolled polyethylene cap using 3" PVC as the case and painters drop cloth

> >(4 mil folded several times).  Has anyone ever used this material for

> >capacitor dielectric?  I'm worried about puncturing it with 10kV (two

> >5kV 60mA neons).  The plates will just be common aluminum foil.

> >

> >Mike

> >--

> >

>

> Mike I didnt notice this until it was brought to my attention

> but you mention above using two 5kv neons in series to get 10kv.

>

> While this can be done its not a recommended procedure. The cases

> will no longer be at ground potential and will be hot.

>

> This could lead to early failure and is an unsafe condition.

>

> If you do this be quite careful and insulate the the neon cases

> from ground and you.

I have already came across this problem.  The transformers will be

mounted in the coil's stand which will be enclosed (except for holes in

the bottom to allow air flow into the spark gap).  Besides, the center

connection can be grounded like I was using just one transformer.  Of

course the cases will also be isolated.

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Robust Capacitors

Date: Thu, 12 Jun 1997 12:05:45 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Thu, 12 Jun 1997 08:05:45 -0400

Subject: Re: Robust Capacitors

> From: Mike Hammer <mhammer@misslink.net>

> Date: Thu, 12 Jun 1997 14:37:23 -0500

> Subject: Re: Robust Capacitors

>

> At 05:22 PM 6/11/97 GMT, you wrote:

> >From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> >Date: Wed, 11 Jun 1997 13:22:13 -0400

> >Subject: Robust Capacitors

> >

> >

> <major snippage>

.

> >

> >Feature Three: The inside face of *the Top* compression plate should

> >*NOT* be flat! To force the stinking air bubbles *OUTWARD* and away

> >from the center of the capacitor can only be accomplished if we have

> >MORE pressure along the center of the capacitor.

> >

> <final snippage>

>

> Fr. Tom,

>

> Aha! I thought someone was sneaking around here when I was building my

> last plate cap. A little coiling espionage?

>

I keep my nose in everything, in case you haven't noticed! But my

master plan is to ferret out every idea that is useful to coilers and

get it posted for all to see. Actually, this is just another example

of how many people will tend to think along the same lines. My hope

is that others will see the ideas and make use of them in their own

coiling efforts. And hopefully they will share the RESULTS back with

the rest of us so we can *all* move forward to bigger and better arcs

and sparks!!

> This is pretty much the way I built my last plate cap.

> I built it between two sheets of 1/4 inch plexiglass

> to compress the layers together and used bolts through the whole

> assembly so it could be cranked down tight.

>

> The bolts also do double duty as the electrical busses connecting

> together the plates.

>

> I did not use paper layers to help with air removal. If you are going to

> go to the trouble of soaking sheets of paper in oil and putting

> the oil soaked sheets in the stack why not just dip the poly sheets in

> oil and forget about the paper.  Either way is equally messy.

>

Putting oil on the poly and leaving out the paper will *not* give

equal results. Remember that the wicking action will draw transformer

oil up into the paper even when it is under a fair amount of

compression. This oil will help to dissolve minute air bubbles that

will ALWAYS be present between the poly unless you use excessive

pressures.

However, even without the paper the compression method will eliminate

the vast majority of air bubbles... enough to make it a useable and

quite viable technique. The paper just adds another level of

insurance.

> When the bolts are tightened down the excess oil and air

> bubbles are squeezed out. Plop it in a tank of oil and its ready

> for big sparks right off the bat.

>

> This method does work and results in a good performing cap.

> Just wish I had thought of it. :)

>

By the way, an alternative to implementing Feature Three is to just

adjust the compression in stages and always have more compression

applied at end "A" and less at end "B" right up until maximum

compression is reached. This makes the pressure greater on one side

and preferentially causes trapped air to be expelled in ONE

direction. It is sort of like a wedge. Once the unit is almost fully

compressed, then the bolts can all be tightened equally.

By the way, you will notice that if using paper, the paper fibers

themselves will always give the air an escape path. This is *not*

true if using bare poly. In fact, if the poly is really thin, then

the compression may force a bubble actually through a thin sheet of

poly, rupturing it. (This will not happen if you use the wedge mode

of compression.)

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

>

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject:

Date: Thu, 12 Jun 1997 15:05:14 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Thu, 12 Jun 1997 17:05:14 +0200 (SAT)

Subject:

Hi there.

Well today I have constructed a small rolled capacitor of paper and foil.

The paper was made one inch thick(it's about 6 pieces of A4 paper put on top

of each other and dipped in oil and then I left it in oil) I have made 4 of

these and put them in series.

Can anyone give me info on what the dielectric of paper is is and how much

voltage a paper can handle. I see that the capacitor I build was quite small

and it gave .5 inch spark so maybe if I make it just the right size then I

will be equel to your Tesla coil's sparks.

Yesterday, while I was working with my coil.(I have putten another coil on

top of the other to have a longer secondary coil so that it doesn't arc over

to the primary. While I was working I found that the saw that was hanging on

the wall on a board began to move. You should try it. It is as if all that

magnetic or electric field or whatever it is can create motion. It's really

cool. You should try that.

Thanks

Richard

EMAIL: esmit@ilink.nis.za

South Africa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Robust Capacitors

Date: Thu, 12 Jun 1997 16:35:07 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Thu, 12 Jun 1997 12:35:07 -0400

Subject: Re: Robust Capacitors

> By the way, an alternative to implementing Feature Three is to just

> adjust the compression in stages and always have more compression

> applied at end "A" and less at end "B" right up until maximum

> compression is reached. This makes the pressure greater on one side

> and preferentially causes trapped air to be expelled in ONE

> direction. It is sort of like a wedge. Once the unit is almost fully

> compressed, then the bolts can all be tightened equally.

>

> By the way, you will notice that if using paper, the paper fibers

> themselves will always give the air an escape path. This is *not*

> true if using bare poly. In fact, if the poly is really thin, then

> the compression may force a bubble actually through a thin sheet of

> poly, rupturing it. (This will not happen if you use the wedge mode

> of compression.)

I think I'll try this.  If the air bubbles can be removed by hand rather

than letting them float to the top, then I think instead of using oil

I'll try some hot paraffin, that way after you're done you can forget

about leaks and air bubbles.

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Small Cap?

Cc:

Bcc:

X-Attachments:

At 02:43 PM 6/11/97 GMT, you wrote:

>From: msnock@theonramp.net (Micheal Huff)

>Date: Wed, 11 Jun 1997 10:43:17 -0400

>Subject: Small Cap?

>

>I'm looking to build a small .01uF cap for a small self-contained coil

>(I want something that I can take places).  I was thinking of building a

>rolled polyethylene cap using 3" PVC as the case and painters drop cloth

>(4 mil folded several times).  Has anyone ever used this material for

>capacitor dielectric?  I'm worried about puncturing it with 10kV (two

>5kV 60mA neons).  The plates will just be common aluminum foil.

>

>Mike

>--

Mike,

First of all a .01 mfd cap can hardly be classified as small.

My .01 mfd plate style cap is in a tank that measures 13 X 17 X 3

inches.

A rolled cap may be a bit more compact.

I used 4 mil poly in my first plate style cap. I used 2 layers for 8 mils

and constructed 6 caps which I connected in series to give an equivalent

of 48 mils of dielectric.

You can use your 4 mil poly in a rolled cap but you would

need from 48-60 mils total dielectric thickness to stand

up to 10KV well.

I would consider making 2 identical .02mfd caps using 24 mils

(6 layers of 4 mil) of poly and using them in series.

This will give you the equivalent of 48 mils and .01 mfd and each

cap only has to handle half the voltage.

If you can find some 6 mil poly use it. This will cut down

on the number of layers you need.

The biggest problem is going to be trapped air in the many layers of

poly. If you have access to a vacuum pump then make your tanks

air tight and pull a vacuum on it to remove the air.

If your like me and dont have access to a vacuum pump then you will

have to let the air come out on its own. This can take quite a while

for rolled caps.

The use of regular transformer oil instead of mineral oil will help

as transformer oil is very thin and will allow the air bubbles

to come out easier.

Building high quality caps is a major project. I've spent

more time and money building caps than all the rest of my

coil components combined.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: IQ

Cc:

Bcc:

X-Attachments:

At 03:07 PM 6/11/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Wed, 11 Jun 1997 17:07:17 +0200 (SAT)

>Subject: IQ

>

>Hi there guys.

>

>I have constructed a Tesla coil and it's giving 6 inch sparks and I want to

>create longer sparks. I just want to know what is the maximum sparks a

>person can get from a 15000 VAC 30mA transformer. You see, I have only been

>interested for about a 3/4 year and now I want to make the best of it and

>learn alot from you guys. If you have questions you can ask me maybe I can

>tell you something that I know about coils.

Richard,

My notes show me the best output I got from a single 12kv/30ma

neon was 21 inches. This was with a 4 inch diameter secondary using

a 15 X 3 inch toroid. So I would expect you COULD realize 20-24 inches

froma 15kv/30. The limiting factors are going to be the quality of you

capacitor and the efficiency of your gap.

>

>Questions:

>----------

>1. Do anyone of you know how to create ball lightning?

No and I doubt anyone else does either. Tesla was rumored

to have been able to do this but no one to my knowledge

has been able to demonstrate it with a Tesla coil.

>2. Do anyone of you know the work Tesla did in Colorado Springs?

Purchase Tesla's lab notes from Colorado Springs

"Colorado Springs Notes 1899-1900" and read it for yourself.

>3. Have anyone of you seen a magnifier coil in action?

Nope. A few pictures only.

>4. Will those who have files on Tesla coils please send it to me?

Go to ftp.funet.fi/pub/sci/electrical/tesla

and download all the files you can stand.

>5. Will you who know what experiments to do with Tesla coils let me know.

Do whatever comes to mind. Its great fun to place various metallic and

non-metallic objects on top of the coil and see the effect.

Try various light bulbs. The clear glass bulbs are best.

Always wanted to try out the neighbors cat. :)

>6. Does anyone have a file on how a Magnifier Coil works in case you have

>will you please send it to me.

Stick to the traditional two coil Tesla coil until you have mastered it.

There is years worth of experimenting to do before you would want

to mess with a magnifier.

>7. If there's someone who has joined the TCBA and don't want their old

>magazine  tell me if you don't want them. If you don't want to give them

>away is their  a way of you scanning it and sending it to me. I would be

>very greatfull. The reason I'm asking you all is because I live in South

>Africa and to buy things  from America is very expensive.

Forget it. Prized posessions.

>8. Anyone who knows an experimenter with the name Steve, he also work with

>magnifiers please let me know.

>9. Those who has all the formulas for Tesla coils please send them to me.

All the formulas? Come on now. There are hundreds or even thousands

of different formulas that can apply to Tesla coils in one fashion

or another. Do some research. Get some books on electronics and radio.

Take advantage of the info available on the funet ftp site and the dozens

of Tesla web sites out there.

No one is going to hand you everything you need to know

about Tesla coils on a silver platter. Do a little digging

and research. Build some coils and experiment. Ask questions

and use your brain.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: More Capacitor Ideas

Date: Thu, 12 Jun 1997 18:10:20 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Thu, 12 Jun 1997 14:10:20 -0400

Subject: More Capacitor Ideas

Fellow Coilers,

Fr. Tom here again.

As regards my post about using a compression plate for eliminating

bubbles from capacitors:

Malcolm mentioned that plexiglass is too easily bent to allow a lot

of compression. My solution would be to get something like a sheet of

1/4 inch fiberglass, and to use lengths of aluminum angle bar across

the width. Thus every set of holes along the side would have an angle

bar connecting across the width. We would need these on both the top

and the bottom compression plates. Angle bar is extruded aluminum (or

iron or whatever) in the shape of the letter "L". It is extremely

strong for its weight. It is available at building supply stores.

In a pinch you could use almost ANY material for the compression

plates then, as the actual FORCE is being supplied by the angle bars,

and the compression plates just sort of hold everything together and

average out the forces. Material should be flat on the side that is

up against the capacitor. Just make sure the material is at least 1/4

inch thick and won't crack under the pressure. You would probably

want to avoid acrylic and go with something like fiberboard or

fiberglass. (fiberboard is like pegboard, but without the holes).

Heck, even plywood would work with angle bars!!

*****

Some more thoughts:

I am sure that most of the ideas I come up with have also been

thought about by others like Malcolm Watts and Alfred Skrocki and

Mike Hammer, just to mention a few. Mike even asked me if I had been

spying on him :) Nope. I gues that it's just that great minds often

think along the same lines. Different actual thoughts, but remarkably

similar in many respects. When I get a really good idea I like to

work out all the details, verify it, and then share it. Sometimes I

don't have the time to do the actual full-scale build-it part before

I find it adviseable to begin sharing the thought part. Like many of

you, I am a sort of hands-on experimenter type and I love to actually

try out my ideas. I have done *enough* initial experimentation on

this idea to know that it has an excellent chance of taking care of

one of our biggest problems with capacitors: the stupid stinking air

bubbles.

Alfred is right: Assembly in oil is actually the preferred method

here. Mike Hammer can tell you that while assembly in oil is indeed a

messy proposition, it DOES have the positive good point of allowing

you to make really good Tesla service capacitors.

I am fairly confident that if originally assembled in a container

under oil, that the assembled unit could then be lifted out and

transferred to a smaller container without much chance of air getting

back inside the assembly. In its new home it should ALSO be

oil-immersed. This is important to make sure that we don't get

corona/sparks around the edges of the poly, which is protected only

by the oil.

*****

Thinking Even Bigger Thoughts...

Hmmmm. Thinking about it, *IF* the extra poly around the edges (all

four edges) were made about an inch larger than normal, and if the

assembly was assembled under oil or at least with a brushing of oil

on *ALL* surfaces (even poly-to-poly surfaces) and really bolted down

super-tight you *might* be able to get by without needing continued

oil immersion. But it seems a bit too risky for me.

But I think there is a safer way to accomplish almost the same thing.

See what you think of THIS idea:

A *DRY* TESLA COIL CAPACITOR

First assemble the whole capacitor DRY with no oil whatsoever. The

initial dry assembly would be to make sure all parts fit perfectly

and smoothly together so there are no surprises. Then disassemble and

prepare for final assembly. The final assembly will have to be done

fairly rapidly as I propose having a container handy that has a

gallon of Spar Varnish poured into it. The container needs to be

bigger than a sheet of the poly or the capacitor plates being used

for the capacitor.

(Wherever I say Spar Varnish, replace with your own favorite paint-on

insulation...)

(Wherever I say dunk the stuff into the Spar Varnish, you can decide

to paint Spar Varnish on instead, but it takes longer. This assembly

MUST be done all in ONE sitting!!)

It is important that all materials (poly, plates, bolts, plastic,

aluminum angle bars, any electrical connecting hardware) have been

cleaned and de-greased and are fully dry. Try not to handle any parts

with your bare hands during this final assembly, as body oils may

create problems. I would say use latex gloves that have likewise been

cleaned and de-greased. Have a big clean paintbrush ready, and lots

of newspaper and rags for cleanup.

The assembly is going to be just a bit messy, so do it in a plastic

container if possible. Or just be sloppy and get your wife upset.

Once everything is ready, begin the assembly by dunking each poly and

aluminum flashing element into the spar varnish one at a time, as we

need them. Coat completely, let drip for a few seconds, and then

transfer to the assembly bed and affix in place. Work as quickly as

you can, as we do not want the varnish drying before we do the final

clamp-down.

(The electrical connections will be OK as soon as the bolts are

tightened down. But if that makes you uneasy, then skip

dunking/painting the section of the plate or part that will

electrically connect to the bolts.)

Assemble all capacitor elements as mentioned above, and then apply

the top compression plate. Tighten it down quickly and methodically.

Remember that you want to initially apply MORE pressure to the center

so that all air and varnish are expelled AWAY from the center.

Continue tightening until the center is at maximum compression. Then

tighten all other bolts until the whole thing is equally tight.

Attach whatever Electrical Hardware needs attaching to the plates. If

varnish gets on the electrical connection bolts you may have problems

later. Use common sense!!

Now we want to let the entire assembly dry. To make sure it doesn't

stick to the newspaper, you can lay it down on some old wood blocks

so very little of it is touching anything. Some may want to place the

assembly sideways others may prefer a parallel to earth orientation.

Do whatever turns you on.

EXCESS Spar Varnish can be removed using the paintbrush. By *excess*

I mean *REALLY* EXCESS. We actually *WANT* a *LOT* of the Spar

Varnish all over every available capacitor surface area, but

especially the edges. I would try to keep varnish off of the NUTS

used to tighten the bolts, as otherwise future disassembly would be

extremely difficult.

AVOID the tendency to want to speed up the drying process by baking

the poor thing. To do so might introduce bubbles, which is what we

are trying mightily to AVOID here! (It also might deform any

plastic). Let it AIR DRY for at least 24 hours, and then apply

additional Spar Varnish anywhere where it looks a little thin. Pay

extra special attention to the sides of the capacitor, as this is

where the greatest danger of future corona breakout would be.

Let dry COMPLETELY now. This may take several days. Be patient!!!

You should now have a bubble-free capacitor that is totally sealed

against moisture. It is a DRY capacitor that does NOT NEED OIL!

This baby can be mounted in ANY orientation, and should be PERFECT as

a capacitor for portable applications

Common sense says you should place it into some kind of enclosure,

but in a pinch the compression mechanism will give it a fair amount

of structural protection.

As an aside: After the capacitor is fully assembled and compressed,

there should actually be almost no Spar Varnish in between the

plates. But anywhere where there is even the slightest dimple should

have Spar Varnish there rather than an air bubble.

Further Thoughts: After a compression Capacitor has been fully dried,

it *might* be possible to remove the compression apparatus and still

have the thing stay intact *IF* you have put sheets of waxed paper in

between the capacitor and the compression plates during assembly, and

IF the removal of compression doesn't cause the capacitor plates to

come apart. Once removed, the unit should be wrapped tightly with one

or more layers of strong cloth tape. Then paint liberally with (you

guessed it) Spar Varnish.  Let dry. You could then put several of

these capacitor units into a single insulating enclosure (use poly or

acrylic as insulation between units), wire them up in series or

parallel, and use them for all kinds of Tesla Coils.

Note that all the thoughts about a DRY Tesla Coil Capacitor are just

that: thoughts. I have *not* actually built such a beast. But the

more I think about it, the more I like the idea!

Hope the thoughts are useful and trigger even bigger and better

thoughts!

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: More Capacitor Ideas

Date: Thu, 12 Jun 1997 18:48:36 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Thu, 12 Jun 1997 14:48:36 -0400

Subject: Re: More Capacitor Ideas

> Note that all the thoughts about a DRY Tesla Coil Capacitor are just

> that: thoughts. I have *not* actually built such a beast. But the

> more I think about it, the more I like the idea!

>

> Hope the thoughts are useful and trigger even bigger and better

> thoughts!

> Fr. Tom McGahee

Here's a thought.  Get some paraffin, melt it, dunk all

plates/dielectric material in to cover it then assemble the cap.  After

you have worked all the air bubbles out of the dielectric and plates you

can then reimmerse the whole assembly backinto hot paraffin and let it

harden.  The result should be a completely dry, but well insulated

(paraffin is an excellent corona suppressor/insulator) block which would

be your dry Tesla cap.

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Robust Capacitors

Cc:

Bcc:

X-Attachments:

At 05:22 PM 6/11/97 GMT, you wrote:

>From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

>Date: Wed, 11 Jun 1997 13:22:13 -0400

>Subject: Robust Capacitors

>

>

<major snippage>

>********

>Here's another post I wrote on how to make robust FLATE PLATE

>capacitors.

>********

>Subject:  Idea for Capacitor

>  Date:   Tue, 10 Jun 1997 19:13:11 -0400

>  From:  "Thomas McGahee" <tom_mcgahee@sigmais.com>

>    To:  <tesla@pupman.com>

>

>

>Fellow Coilers,

>

>Fr. Tom here again with some more food for thought.

>

<even more snippage>

>

>I think I may have a solution that will work for flat plate

>capacitors at least. What we want to do is build a fairly regular

>flat plate capacitor, but with three special features.

>

>Feature One: The use of oil-soaked paper placed between all capacitor

>surfaces, whether metal to dielectric, or dielectric to dielectric.

>Pre-soaking of these pieces of paper may be just a slight bit on the

>messy side, but will allow us to ensure an initial starting condition

>that is already oil-rich exactly where we need the oil the most.

>

>Paper pieces should be smooth, absorbent, clean paper. Kraft paper

>used to wrap packages works well. Paper should be soaked for at least

>an hour before use. Before soaking in oil, cut the paper so that it

>is AT LEAST as large as the polyethelene sheets. A slight overhang

>along all edges is actually useful. I would recommend 1/4" of

>overhang on LARGE size plates, and at least 1/8" overhang on smaller

>size plates.

>

>Feature Two: Build up the flat plate capacitor on a COMPRESSION

>FRAME! Imagine a flat plate capacitor sandwiched between two very

>rigid non-conducting plates such as 3/8" plexiglass or 1/4"

>fiberglass board. In a pinch you could use 3/8 or 1/2" plywood even,

>although the thought makes me want to barf. I would envision using

>something rigid which would NOT soak up oil.

>

>Note: Don't make the compression plates too thin. Remember that they

>are there to allow us to apply fairly decent pressures to the

>capacitor. Skimping here can cause the material to crack. Oh yes, do

>not put holes too close to the edge of the plexiglass or the pressure

>make cause cracking from the holes to the edge. I planned to use a

>metal washer of decent size anywhere a bolt head or nut was pushing

>against the plexiglass.

>

>This frame would have long bolts holding the two frame pieces

>together, with the capacitor sandwiched in between. I would envision

>using bolts every inch or so along the WIDTH of the compression

>frame. My own design plans call for me to use the bottom compression

>frame as the building support for the actual capacitor.

>

>Feature Three: The inside face of *the Top* compression plate should

>*NOT* be flat! To force the stinking air bubbles *OUTWARD* and away

>from the center of the capacitor can only be accomplished if we have

>MORE pressure along the center of the capacitor.

>

<final snippage>

Fr. Tom,

Aha! I thought someone was sneaking around here when I was building my

last plate cap. A little coiling espionage?

This is pretty much the way I built my last plate cap.

I built it between two sheets of 1/4 inch plexiglass

to compress the layers together and used bolts through the whole

assembly so it could be cranked down tight.

The bolts also do double duty as the electrical busses connecting

together the plates.

I did not use paper layers to help with air removal. If you are going to

go to the trouble of soaking sheets of paper in oil and putting

the oil soaked sheets in the stack why not just dip the poly sheets in

oil and forget about the paper. Either way is equally messy.

When the bolts are tightened down the excess oil and air

bubbles are squeezed out. Plop it in a tank of oil and its ready

for big sparks right off the bat.

This method does work and results in a good performing cap.

Just wish I had thought of it. :)

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Small Cap?

Cc:

Bcc:

X-Attachments:

At 08:42 PM 6/11/97 GMT, you wrote:

>From: msnock@theonramp.net (Micheal Huff)

>Date: Wed, 11 Jun 1997 16:42:01 -0400

>Subject: Re: Small Cap?

>

<SNIP>

>

>I think I'll go with your idea of a stacked plate cap.  Could you run

>down the specs on your cap?  Plate dimensions, dielectric dimensions,

>plate material, etc.

>

Mike,

Well I have made two quite successful vertical plate

caps of completely different design.

The first one contains 6 individidual caps wired in series.

Each cap has a value of .048 mfd and contains 24  6 X 6 inch

plates of aluminum foil seperated by 2 layers of 4 mils poly.

The poly squares are 7 X 7 inches and the foil plates

have a 1 inch by 2 extended tab for connections.

Each cap is sandwiched between two 7 X 7 sheets of plexiglass.

The tabs for each section are bundled together and the connections

between caps are put together with small brass bolts and washers.

This is housed in a homemade tank made from plywood and lined

with plexiglass. RTV was used to seal the seams. This is

in about 1.25 gallons of veternairy grade mineral oil.

This cap has served me well using 12kv neons and up to 120ma

current. Best output with this cap and my 4 inch secondary

was 36 inches.

My newer plate cap was an experiment. I wanted to try larger plates

and make it one single cap instead of a series string. This was

done to try and minimize the number of plates and poly squares needed.

And while I was at it I designed for a higher capacitance of .01 mfd

so I could accomodate a larger toroid on my coil.

I used 14 plates of aluminum flashing cut 10 X 12 inches.

The overlapping area of the plates is 10 X 10. So 2 inches

hangs off each end. The dielectric is 10 layers of 6 mil poly

for 60 mil thickness.

The poly sheets are cut 12 X 12.

I drilled two holes in one end of each plate. 3/16 inch bolts

are used in these holes to connect like plates togther and act

as electrical busses.

The stack is sandwiched between two sheets of 1/4 inch plexiglass

cut 12 X 14.

The whole thing is solidly bolted together and rigid.

It is in another plywood tank lined with plexiglass and RTV

and I used mineral oil from a local pharmacy.

I let it soak in oil for 4 days and fired it up.

Results were poor and disappointing. It was much inferior

to my previous cap. After trying to push some power through it

it blew up. After disassembly I found the failure was due

to trapped air. The arc traveled from the edge of one plate

along the surface of the poly out to the edge and down onto the plate below.

The total arc distance was about 1 1/4 inches. This wouldnt of

happened through oil.

I rebuilt the cap just the same way but this time I dipped each

poly sheet in oil before adding to the stack. I worked the bubbles

out to the edge with my fingers. It was very messy and took 2 days.

But once it was all back together and in oil I fired it right up

and it works splendidly.

It was obviously the trapped air causing the poor performance.

I have now got 42 inch sparks using this cap and a 19 X 6 toroid.

If any of this isnt clear or you have further questions

feel free to ask.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject:

Date: Fri, 13 Jun 1997 15:38:33 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Fri, 13 Jun 1997 17:38:33 +0200 (SAT)

Subject:

Hi all

Does anyone know if 15000 VAC will go through 90 mil paper submerged in oil.

Thanks

Richard

EMAIL: esmit@ilink.nis.za

South Africa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject:

Date: Fri, 13 Jun 1997 15:42:19 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Fri, 13 Jun 1997 17:42:19 +0200 (SAT)

Subject:

Hi all

Can anyone tell me how to read file from ftp.funet.fi The file is like this:

example

teslacal.exe.zip

EMAIL: esmit@ilink.nis.za

South Africa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re:paper caps

Cc:

Bcc:

X-Attachments:

At 03:05 PM 6/12/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Thu, 12 Jun 1997 17:05:14 +0200 (SAT)

>Subject:

>

>Hi there.

>

>Well today I have constructed a small rolled capacitor of paper and foil.

>The paper was made one inch thick(it's about 6 pieces of A4 paper put on top

>of each other and dipped in oil and then I left it in oil) I have made 4 of

>these and put them in series.

>Can anyone give me info on what the dielectric of paper is is and how much

>voltage a paper can handle. I see that the capacitor I build was quite small

>and it gave .5 inch spark so maybe if I make it just the right size then I

>will be equel to your Tesla coil's sparks.

Just so happens that Fr.McGahee posted this info

in his interesting post on oils yesterday. I will paste in the

part on paper here.

Dielectric strength   Puncture Voltage

Material                       K           per Mil(0.001 inch)

---------------------------------------------------------------------

Miscellaneous

-------------

Vacuum                            1.0

Paper, bond                       3.0            200

Paper, Royal Grey                 3.0            200

Paper, telephone, treated         2.5 - 4        200 - 250

Paper, Parafin Coated             2 -3.5

Paper, Kraft                      2.2

Paper can be used as a dielectric but it is by

far inferior to polyethylene or polypropylene. Thats why

you only see commercial paper caps used at DC or audio frequencies

not RF.

Still for experimental caps it would be worthwhile to play with.

>

>Yesterday, while I was working with my coil. (I have put another coil on

>top of the other to have a longer secondary coil so that it doesn't arc over

>to the primary.

If you have primary secondary arcing then the problem is most likely

overcoupling. By adding another coil on top of your existing one

you have drastically reduced its resonant frequency. Probably

outside the range your primary circuit can tune.

Try raising your secondary up away from your primary a 1/2 inch at a

time until the arcing stops.

>While I was working I found that the saw that was hanging on

>the wall on a board began to move. You should try it. It is as if all that

>magnetic or electric field or whatever it is can create motion. It's really

>cool. You should try that.

Electrostatic effects like this are not uncommon.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Re:

Date: Fri, 13 Jun 1997 17:06:08 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Fri, 13 Jun 1997 13:06:08 -0400

Subject: Re:

Tesla-2 wrote:

>

> From: Richard <esmit@ilink.nis.za>

> Date: Fri, 13 Jun 1997 17:38:33 +0200 (SAT)

> Subject:

>

> Hi all

>

> Does anyone know if 15000 VAC will go through 90 mil paper submerged in oil.

It shouldn't, especially when submerged in oil as the paper will soak up

oil providing extra insulation. Just remember that 15kVAC is about

21210V peak.  I have tested about 90-100mils thick of paper at around

40kVDC and it didn't breakdown, but I personally wouldn't use it since I

imagine it would rot away, poly won't do that, but of course paper is

much cheaper and available.  You might also try some wax paper because

it has a higher dielectric constant, "k". (That's the stuff you set

cookies on after baking them.)

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Robust Capacitors

Cc:

Bcc:

X-Attachments:

At 04:35 PM 6/12/97 GMT, you wrote:

>From: msnock@theonramp.net (Micheal Huff)

>Date: Thu, 12 Jun 1997 12:35:07 -0400

>Subject: Re: Robust Capacitors

>

>> By the way, an alternative to implementing Feature Three is to just

>> adjust the compression in stages and always have more compression

>> applied at end "A" and less at end "B" right up until maximum

>> compression is reached. This makes the pressure greater on one side

>> and preferentially causes trapped air to be expelled in ONE

>> direction. It is sort of like a wedge. Once the unit is almost fully

>> compressed, then the bolts can all be tightened equally.

>>

>> By the way, you will notice that if using paper, the paper fibers

>> themselves will always give the air an escape path. This is *not*

>> true if using bare poly. In fact, if the poly is really thin, then

>> the compression may force a bubble actually through a thin sheet of

>> poly, rupturing it. (This will not happen if you use the wedge mode

>> of compression.)

>

>

>I think I'll try this.  If the air bubbles can be removed by hand rather

>than letting them float to the top, then I think instead of using oil

>I'll try some hot paraffin, that way after you're done you can forget

>about leaks and air bubbles.

>

>Mike

While I suppose paraffin would work in this

manner I'm against the idea.

If any air bubbles remained trapped (and there will be)

they will be forever imbedded in the solid wax.

It would also make your cap much more

difficult to repair in case of breakdown and it

eliminates one of the most desirable features of plate style

caps. That is the ability to add additional plates at any

time to increase capacitance.

I did this twice to my original plate cap. I started with 16 plates

per cap and later added 4 more for a total of 20 per cap.

Later on I added 4 more per cap to go to 24.

I had planned on increasing it to 30 plates per cap but decided

to build a whole new cap instead.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: More Capacitor Ideas

Cc:

Bcc:

X-Attachments:

At 06:10 PM 6/12/97 GMT, you wrote:

>From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

>Date: Thu, 12 Jun 1997 14:10:20 -0400

>Subject: More Capacitor Ideas

>

>Fellow Coilers,

>

>Fr. Tom here again.

>

>As regards my post about using a compression plate for eliminating

>bubbles from capacitors:

>

>Malcolm mentioned that plexiglass is too easily bent to allow a lot

>of compression. My solution would be to get something like a sheet of

>1/4 inch fiberglass, and to use lengths of aluminum angle bar across

>the width. Thus every set of holes along the side would have an angle

>bar connecting across the width. We would need these on both the top

>and the bottom compression plates. Angle bar is extruded aluminum (or

>iron or whatever) in the shape of the letter "L". It is extremely

>strong for its weight. It is available at building supply stores.

>

I used 1/4 inch plexiglass and it does bend quite a bit.

I have thought about using something to stiffen it up

a bit and thought about epoxying a couple extra layers of plexiglass

across the sheet as ribs. But the angle iron idea seems to be a good one.

I'll save that idea for my next cap. (you never have enough capacitance)

<snip>

>Thinking Even Bigger Thoughts...

>

>Hmmmm. Thinking about it, *IF* the extra poly around the edges (all

>four edges) were made about an inch larger than normal, and if the

>assembly was assembled under oil or at least with a brushing of oil

>on *ALL* surfaces (even poly-to-poly surfaces) and really bolted down

>super-tight you *might* be able to get by without needing continued

>oil immersion. But it seems a bit too risky for me.

Maybe if the cap was laid down flat but it will run

out slowly if stood on edge.

>

>But I think there is a safer way to accomplish almost the same thing.

>See what you think of THIS idea:

>

>A *DRY* TESLA COIL CAPACITOR

>

>

>First assemble the whole capacitor DRY with no oil whatsoever. The

>initial dry assembly would be to make sure all parts fit perfectly

>and smoothly together so there are no surprises. Then disassemble and

>prepare for final assembly. The final assembly will have to be done

>fairly rapidly as I propose having a container handy that has a

>gallon of Spar Varnish poured into it. The container needs to be

>bigger than a sheet of the poly or the capacitor plates being used

>for the capacitor.

>

<giant snip>

Ugh!

Well I suppose it would work and might work well.

But let me tell you from my experince with building a cap

under oil that I had mineral oil up to my elbows, on my clothes,

on the floor, in my hair, on the dog, etc.

Now just imagine doing that with varnish. Yikes.

For you neat and clean types it may work out well

but for the disaster types like myself I would think twice. :)

Still though the idea is interesting. Perhaps just a small one.

And just a small disaster.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: secondary size

Date: Fri, 13 Jun 1997 19:57:05 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Dave <djb@erie.net>

Date: Fri, 13 Jun 1997 15:57:05 -0400

Subject: secondary size

Hello,

I have a question about the secondary coil diameter which has been bugging

me for awhile now. I can see that increasing the diameter of the secondary

would increase the inductance of it while lowering it's Q, and I know we

want a high inductance and a Q of at least 100, but what I don't understand

is what size diameter of PVC to use for different power levels. For example,

my intuition tells me you would not want to use a neon transformer to power

an 18 inch secondary, or a 'pole pig' to power a 4 inch secondary...but I

cannot explain the physics of why you wouldn't want to do that for each case.

Also, I have been sick the last week, and am just wondering if there are

daily discussions still going on in the tesla irc channel?

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

To: tesla-2@emachine.com

From: Mike Hammer <mhammer@misslink.net>

Subject: primary taps

Cc:

Bcc:

X-Attachments:

Hello all,

Since I will be taking some time off

the next couple weeks I hope to get some

serious coiling done.

One thing I need to work on is my primary tap.

After raising my primary capacitance I noticed

that my primary tap gets quite warm.

After several extended runs this piece was too hot

to touch. So obviously my arrangement of a large

alligator clip for a primary tap has some amount

of resistive loss. I had attached 1/2 sections of

small copper tubing to the jaws to increase the contact area.

My question is to those of you with operating coils.

What are you using for a primary tap?

Just looking for a few ideas to lower the loss in this area.

TTYL!

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: secondary size

Date: Fri, 13 Jun 1997 22:35:56 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Shane Randle <randlec@scnc.addison.k12.mi.us>

Date: Fri, 13 Jun 1997 18:35:56 -0400

Subject: Re: secondary size

I am starting to build my first coil this summer, and I was wondering

what the secondary:primary ratio is for the ideal height of the primary

coil.  My secondary will be about 14 inches tall, but I have not yet

constructed my primary because I am unsure how tall it should be.

-Shane Randle

randlec@scnc.addison.k12.mi.us

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary taps

Date: Sat, 14 Jun 1997 00:52:38 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Fri, 13 Jun 1997 20:52:38 -0400

Subject: Re: primary taps

A little cross-pollenation from the Tesla-2 List

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: primary taps

> Date: Saturday, June 14, 1997 2:36 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sat, 14 Jun 1997 13:36:13 -0500

> Subject: primary taps

>

> Hello all,

>

> Since I will be taking some time off

> the next couple weeks I hope to get some

> serious coiling done.

>

> One thing I need to work on is my primary tap.

>

> After raising my primary capacitance I noticed

> that my primary tap gets quite warm.

>

> After several extended runs this piece was too hot

> to touch. So obviously my arrangement of a large

> alligator clip for a primary tap has some amount

> of resistive loss. I had attached 1/2 sections of

> small copper tubing to the jaws to increase the contact area.

>

> My question is to those of you with operating coils.

> What are you using for a primary tap?

> Just looking for a few ideas to lower the loss in this area.

>

> TTYL!

>

>

> Mike Hammer

> mhammer@misslink.net

Mike,

As you have noticed, the resistive losses from the primary TAP

mechanism can be excessive. The losses are from several factors.

1) The currents are just obscenely high. But that's what we WANT, so

we will NOT try to reduce *that* in any way :)

2) With extreme high RF currents you need a large surface area to get

the current transferred. The stupid little teeth on even the best

'gator clips are pathetic for RF use. The points just intensify the

problem.

So, get rid of the stinking teeth and increase the surface area.

Grind the jaw structure so that a maximum surface area come in

contact.

Add a more robust surface area by soldering on copper pieces shaped

so as to maximize surface contact (not too easy since the shape of

the spiral changes from turn-to-turn). Easier for you solenoid guys,

as the pitch and radius are constant in that form of primary.

Use a MUSHY metallic material to make the contact. Think metallic

Braid. Think of the outside metallic braid on coax cable. Yeah, I

know that everyone tells us not to use braid. But they are talking

about the main power runs, my friend! HERE is a place where the braid

has a quality that overcomes its defect. The great MUSHINESS of the

braid makes it ideal to create a larger surface area contact.

Try *this* with your existing clip lead setup: wrap two or three

turns of braid **tightly** around the desired tap point. Let the clip

lead sink its old 'gator teeth into the braid. Not perfect, but a

definite improvement! (And ridiculously EASY to implement, for all

you lazy bones coilers out there). How many wraps you can tolerate

depends on the size of your 'gator, and the spacing between primary

turns.

All right, so now that I have convinced you of the joys of braid,

consider attaching the braid right to the inside of the jaws of the

'gator itself. You can file away the teeth and that will give you

more room for the braid. Solder it only OUTSIDE the mouth of the

'gator clip. We want the stuff between the jaws to remain MUSHY. Be

careful, as the braid LOVES to wick up solder. TIN the outside of the

jaws and lightly tin the end of the braid. Heat up the jaw until the

solder melts and then touch the tinned end of the braid into the

solder on the jaw . Hold the braid in a needle nose pliers jaw,

tightly and near the end. This will restrict the solder wicking by

drawing away the heat. It will also prevent your fingers from having

the permanent imprint of wire braid branded into them.

After soldering one end, wrap the braid along the inside of the jaw

and solder the other end on the other side. What you want is a run of

mushy flat braid across the inside of the jaw. Do both the top and

bottom jaws.

An alternative to going across the width of the jaw is to run it down

from the top of the nose of the jaw, loop it INSIDE the jaw, and

around such that it then comes back out across the bottom of the

bottom jaw. Soldered on the outside, of course.

There are MANY variations on the theme. Pick one. Make a mushy jawed

'gator clip. Amaze yourself. Impress the neighbors.

3) With really huge currents the TYPE of conductor becomes very

important. Heavy duty solid COPPER is best. It is also one of the few

metals that really solders well. So if you are going to modify a

'gator clip, I suggest beginning with a solid copper one if you can

get one. Trying to solder aluminum 'gator clips is an exercise in

frustration. Save your energy and your sanity. Buy copper 'gator

clips.

Now, some of you are using those dumb little Radio Shack mini 'gator

clip leads with all the cool little colored booties on them. Doomed

to failure from the word go. Not enough surface area to conduct our

obscene RF currents. These babies get hot enough for the solder to

melt and the plastic booties begin to drip off. Forsake the wimpy

mini gator clip and go for the Macho 'gator clip. Think Surface Area.

4) 'Gator clips are made from two sections. In a high current RF

environment this is bad news, as the second section has only a few

minor points of contact with the main section. This means a high RF

resistance for half of the clip. This in turn means lots of heating

WITHIN the 'gator clip itself. Solder coax braid from the wide top to

the wide bottom. Keep it short and WIDE. Really industrious little

coilers will find ways to do this work from the INSIDE as well as

from the outside. Think Skin Effect.

5) And the final 'gator clip modification: Throw out the stinking

gator clip. Build you own Compression Fitting Primary Tap. This is a

tap that is designed so that it is primarily a PERMANENT TAP, but in

a Temporary sort of way. A 'gator clip is temporary and makes a lousy

permanent tap. Soldering and other such methods are permanent and you

have to live with it.

A compression fitting can be fabricated in many ingenious ways, but

its main feature is that you can move it around, but once you are at

the right place, you can tighten this sucker down and leave it there

forever if you want. The down side is that it takes a bit longer to

move it around. But it makes an EXCELLENT connection.

I have seen small hose clamps used. Properly trimmed they aren't all

that bad. These work best on larger diameter tubing.

The compression fitting has SOME means to clamp the metal tightly

around the primary coil. The metal that gets clamped should have some

flexibility, but most importantly it should make an extremely tight

connection with a good sized surface area.

The material should be something that can take a certain amount of

flexing without breaking. I have used copper strapping to good

advantage. You can make your own from sheet copper, or actually buy

pieces that are about 1/2 to 1" wide at building supply stores. You

can also use wide flat braid as the flexible conductor part and use

brass plates to do the compression part.

The compression part consists of some means to pull the strap tightly

around the copper tubing and secure it. I often use brass plates that

I have drilled and tapped so that one plate has holes larger than the

screws and the other has been tapped so that the screw can be secured

by that plate. Also connected to this brass plate is the supply lead.

It may be soldered or also compression clamped. Think surface area.

Sometimes I attach more than one feed lead per clamp. That increases

the surface area of the feed lead while keeping it relatively

flexible.

I normally have the flexible flat copper/brass or braid section

soldered to the two afore-mentioned plates. Solder them well, as the

solder is going to be providing physical strength to the assembly.

Another method is to use another plate to clamp the flexible metal

band to the master plate.

If using braid, consider first rounding the edges of the plates that

are where the braid passes by. Otherwise the sharp edges will bite

through.

However you decide to do it, get the spacings adjusted so that when

the flexible piece is wrapped around the tubing and you tighten down

the screw(s) the unit is firmly attached with a decent surface area.

Hope the information is of use to someone.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Quenching

Date: Sat, 14 Jun 1997 13:15:07 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Sat, 14 Jun 1997 15:15:07 +0200 (SAT)

Subject: Quenching

Hi,

I was wondering if anyone can tell me how I can make my Tesla coils spark

gap quench very fast without using a rotary spark gap.

Thanks.

Richard

EMAIL: esmit@ilink.nis.za

South Africa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re:paper caps

Cc:

Bcc:

X-Attachments:

At 03:38 PM 6/13/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Fri, 13 Jun 1997 17:38:33 +0200 (SAT)

>Subject:

>

>Hi all

>

>Does anyone know if 15000 VAC will go through 90 mil paper submerged in oil.

>

Richard,

My first reaction is to say forget paper as a dielectric.

But not knowing what the availability of proper materials

are like in your location perhaps you have no better choice.

And I'm a use what I got kind of guy.

Paper is not a good insulator and it has high RF dissiaption factor.

Which means that lots of RF energy will be converted to heat.

This heat will break down and degrade the paper until a failure

occurs. And this represents lost energy that will not be

contributing to spark output.

Now that I got that out of my system lets look at the possibility.

The voltage capability would depend greatly on the type of

paper used and the type of oil.

The list Fr. Tom posted recently lists bond paper

as have a rating of 200 volts per mil. (.001 inch)

The ratings are DC values so need to be cut in half for

AC. Thats gets us down to 100 volts AC per mil. Because

we are not subjecting Tesla caps to just plain old calm AC

but to nasty ragged pulses of RF you would want to derate this further

by another 25 percent. So that gives us 75 volts per mil.

So your 90 mil thick bond paper will be good for around 6750

volts. You would need three such units in series to handle

your 15 kv neon reliably. The voltage rating would most likely

improve some by impregnating the paper with oil but how much

would be hard to determine.

Remember if you are connecting caps in series they should be identical

and the resulting capacitance will be less than the value

of the individuals by a factor equal to the number of units.

So if you need .005 mfd and are using 3 caps in series each

unit must have 3 times that value or .015 mfd.

If you do build such a cap let us know how it works out.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: funet files

Cc:

Bcc:

X-Attachments:

At 03:42 PM 6/13/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Fri, 13 Jun 1997 17:42:19 +0200 (SAT)

>Subject:

>

>Hi all

>

>Can anyone tell me how to read file from ftp.funet.fi The file is like this:

>example

>        teslacal.exe.zip

>

Richard,

I took a quick look at funet .fi and the closest I

come to what you said above was teslacal.bas.z.

This is a Qbasic program compressed with the UNIX GZIP

program. Many of the files and archives on that site are GZIPPED.

You can find a version of GZIP to run on your machiine

or if you are running Windows get the latest version

of WinZip.

WinZip handles GZIP as well as regular ZIP files.

It also handles Base64 and UUENCODED files.

A program well worth having.

TTYL!

------

To: tesla-2@emachine.com

From: Mike Hammer <mhammer@misslink.net>

Subject: Tesla coil tuning

Cc:

Bcc:

X-Attachments:

Hello all,

Several people have asked me via email and on the

IRC Tesla channel about how to go about tuning

a coil and the steps to take.

So I thought I would share some info on the subject with

the list.

Once a Tesla coil is set up and ready to fire, certain steps

should be taken before power is applied. Don't expect to throw

full power into a new coil or a newly rearranged system. If you do

you may damage components. The coil must be brought into tune

and up to full power gradually.

The use of a variac is highly recommended for controlling

the input voltage to whatever step up transformer you

may be using. (neon, obit) Never throw full power to a coil

until you are sure the system is in proper tune.

Set the primary circuit up first. Double check all of your

connections. Run your grounds and double check those. Never take

any connection for granted. Set the secondary into the

primary and attach the RF ground. Use a small discharge terminal

at first. A small spherical terminal works well at this point.

A smaller terminal will break down and produce spark

at a lower power level than a larger terminal. This allows

you to search for the proper tune with less stress placed on

components. If you dont have a smaller terminal and only have

a large toroid place a point discharger on the toroid. Sparks

will easily break out from such a point. Often times a well placed

thumbtack will work well.

CLOSE YOUR SPARK GAPS DOWN!

I can't overemphasize this point. Don't run a lot of gap.

This will limit the peak voltages in the primary to low

levels while things are out of tune.

As things are brought closer to resonance the gap can be opened

a little at a time to increase the primary energy level.

Apply a bit of power until the spark gap fires. If no spark

breakout is seen from the discharge terminal don't worry. Use a

flourescent bulb as an indicator. As the coil gets closer to resonance

the farther away and brighter the bulb will glow. Turn off the coil

and move the tap connection on the primary coil a half turn and

check the bulb brightness. Then move it a full turn in the

opposite direction and note the brightness of the bulb again.

This way you can determine the direction the tap needs to be

moved to locate the proper tune.

Open the main and safety gaps up in steps. Apply the same voltage

input (low power) and check for discharge. Once spark has begun

to break out of the discharge terminal use it as a visual

indicator of tune.

If you find the best tune point is with very few primary

turns then your primary cap is too large. Adding a larger toroid

to your secondary will lower its resonant frequency and allow

you to tap in more primary turns also.

If you get tuned all the way out to your last primary turn

and you still havnt found the resonant point then your primary

cap may be too small or your toroid too large.

This often happens with beginner coils wound with far too

many turns of small guage wire. The large number of secondary turns

causes the secondary resonant frequency to be very low and beyond the range

of your primary circuit. Cutting down the secondary to no more than 900-1000

turns will help here.

A system in proper tune should not break down the safety gap often. The

safety gap once set properly should not be too active. Once the coil

is in good tune the safety gap electrodes may be opened to the point

where it only fires occasionally.

If your safety gap is firing continuously then you may be out of tune

or you may be running too wide a main gap. Try closing down your main gap

slightly and retuning for best output.

Hope this is helpful.

As always comments, criticism and large sums of money welcome.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: secondary size

Cc:

Bcc:

X-Attachments:

At 07:57 PM 6/13/97 GMT, you wrote:

>From: Dave <djb@erie.net>

>Date: Fri, 13 Jun 1997 15:57:05 -0400

>Subject: secondary size

>

>Hello,

>

>I have a question about the secondary coil diameter which has been bugging

>me for awhile now. I can see that increasing the diameter of the secondary

>would increase the inductance of it while lowering it's Q, and I know we

>want a high inductance and a Q of at least 100, but what I don't understand

>is what size diameter of PVC to use for different power levels. For example,

>my intuition tells me you would not want to use a neon transformer to power

>an 18 inch secondary, or a 'pole pig' to power a 4 inch secondary...but I

>cannot explain the physics of why you wouldn't want to do that for each case.

There really arent any rules set in stone. It just generally

follows that larger coils need more power to develop

peak output.

Likewise small coils have a definite power handling limit.

That limit depends on lots of factors including the secondary form,

type of insulation on the wire, and type and thickness of any overcoating

on the secondary. Also other factors such as gap efficiency

and degree of coupling determine how much power is actually

transferred to your secondary.

My own 4 inch secondary has suffered several breakdowns

as I tried to push more and more power through it with

more efficient gaps, tight coupling and large toroids.

But I am only using a single 12kv/90ma neon. I am trying

to squeeze all the spark I can out of this one neon transformer.

Record output stands at 42 inches.

My own thoughts based on nothing but what I have observed

with my own coils and results reported by others is that a 3 inch

diameter secondary is good to about 700 VA or so and a 4 incher

maybe up to 1.5 KVA. A 6 inch coil could handle 4 or 5 KVA.

These levels I'm sure can be exceeded with care in construction

and operation.

So as I see it its more of a situtation of going to a bigger coil

when your power level starts to exceed the capabilities of your

present secondary.

>

>Also, I have been sick the last week, and am just wondering if there are

>daily discussions still going on in the tesla irc channel?

Yes there are. It is kind of impromptu and mainly from 6-8 PM CDT.

Later hours during the weekend. Take a peek once in a while

and see if anyone is around.

If anyone else is interested in coming on IRC to talk coils

the server address is chat.talkcity.com  port 6667

The channel name is #Tesla and my nickname in the channel is XTOL.

Use an IRC program like MIRC or if using Netscape 3 go to

www.talkcity.com.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Leyden Jar Caps

Date: Sat, 14 Jun 1997 22:12:20 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Shane Randle <randlec@scnc.addison.k12.mi.us>

Date: Sat, 14 Jun 1997 18:12:20 -0400

Subject: Leyden Jar Caps

I am about to begin building the capacitor for my first Tesla coil this

summer, and, since it is my first homemade capacitor, I decided to use a

simple glass jar/salt water design.

So far, I am planning to fill a glass jar with salt water solution, and

place this into a larger container also filled with salt water.  Both

containers will have a steel bolt in them which will serve as the

terminals.  The glass in the jar is the dielectric.

I was wondering if there was anything I am forgetting.  Is there

anything else I need to add for the spark gap, or will the terminals

themselves form the gap?

I am sure you all have more experience at this than me, and I would

appreciate any insight or advice you all might have in this matter.

Thank you very much in advance.

-Shane M. Randle

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Leyden Jar Caps

Date: Sun, 15 Jun 1997 01:26:32 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Michael Huff)

Date: Sat, 14 Jun 1997 21:26:32 -0400

Subject: Re: Leyden Jar Caps

Tesla-2 wrote:

>

> From: Shane Randle <randlec@scnc.addison.k12.mi.us>

> Date: Sat, 14 Jun 1997 18:12:20 -0400

> Subject: Leyden Jar Caps

>

> I am about to begin building the capacitor for my first Tesla coil this

> summer, and, since it is my first homemade capacitor, I decided to use a

> simple glass jar/salt water design.

> So far, I am planning to fill a glass jar with salt water solution, and

> place this into a larger container also filled with salt water.  Both

> containers will have a steel bolt in them which will serve as the

> terminals.  The glass in the jar is the dielectric.

> I was wondering if there was anything I am forgetting.  Is there

> anything else I need to add for the spark gap, or will the terminals

> themselves form the gap?

> I am sure you all have more experience at this than me, and I would

> appreciate any insight or advice you all might have in this matter.

> Thank you very much in advance.

> -Shane M. Randle

I used saltwater caps at one time too.  They work well even though you

are using glass as a dielectric.  My caps where more portable though.  I

covered the outside of the bottles with aluminum foil and filled the

insides with saltwater.  Covering the outside of the bottle makes them

very clean and easy to use.  Rubber cement works well for securing the

foil.  After the foil has been secured to the bottle, wrap the foil with

electrical tape.  This will prevent rips to the aluminum foil and

suppress corona. Be sure to top of the saltwater with some vegetable oil

(the quality of oil need not be high as it is used just to supress

corona, not as an insulator) or even motor oil, but this can get real

messy with motor oil.  Last night one ruptured along the neck of the

bottle and oil oozed out all over the floor.  Have fun with your coil.

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Quenching

Cc:

Bcc:

X-Attachments:

At 01:15 PM 6/14/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Sat, 14 Jun 1997 15:15:07 +0200 (SAT)

>Subject: Quenching

>

>Hi,

>

>I was wondering if anyone can tell me how I can make my Tesla coils spark

>gap quench very fast without using a rotary spark gap.

>

Very fast quenching is not really needed with neon

transformers and loosely coupled 2 coil tesla coils.

The repetition rate is normally 120 pps with static gaps.

Thats two firings per AC cycle.

With a large transformer and a good gap it is possible to get more

than 2 firings per cycle.

What is important is being able to blow out the arc

quickly enough so the cap can begin charging again.

The best way to insure adequate quenching is to break up

the gap into a series string of smaller gaps. These smaller

gaps run cooler and more efficiently than a single gap of the

same total spacing.

A good airflow through the gap will help with cooling

and the removal of hot ions that might cause the gap

to remain lit. On the cylinder type gap a good strong

muffin fan is usually more than adequate.

Higher speed air will help improve the quenching

by physically blowing out the arc. This can be overdone.

I built a cylinder type gap with airflow supplied by

a high speed vacuum cleaner motor. At maximum RPMs

it causes less secondary output. The high speed air

is probably inhibiting the initial firing of the gap.

By varying the motor speed with a variac I can find

just the right amount of airflow.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Leyden Jar Caps

Cc:

Bcc:

X-Attachments:

At 10:12 PM 6/14/97 GMT, you wrote:

>From: Shane Randle <randlec@scnc.addison.k12.mi.us>

>Date: Sat, 14 Jun 1997 18:12:20 -0400

>Subject: Leyden Jar Caps

>

>I am about to begin building the capacitor for my first Tesla coil this

>summer, and, since it is my first homemade capacitor, I decided to use a

>simple glass jar/salt water design.

I also used a salt cap for my first cap and it worked

out rather well. I got 14 inches of spark with a 3 inch diameter

secondary and a 12kv/30ma neon.

Its cheap, it works and gets you sparking.

>So far, I am planning to fill a glass jar with salt water solution, and

>place this into a larger container also filled with salt water.  Both

>containers will have a steel bolt in them which will serve as the

>terminals.  The glass in the jar is the dielectric.

This sounds fine. I used beer bottles and got about

700pf of capacitance per bottle.

I had best results with 6 bottles or about .0042 mfd.

Brown glass beer bottles stood up well to 12kv neons.

I only had one bottle fail and that was a clear glass Miller bottle.

Consider laying a sheet of aluminum foil in the bottom

and up one side of your larger outer container.

The set your beer bottles or whatever on top of the

foil and add your saltwater. Use some good alligator

clips to attach to the foil. The foil/saltwater is one

plate connection and the carriage bolt/saltwater inside

the beer bottles is the other connection.

>I was wondering if there was anything I am forgetting.  Is there

>anything else I need to add for the spark gap, or will the terminals

>themselves form the gap?

Spark gap is totally seperate. Consider building a RQ style

cylinder gap. Well worth the trouble.

>I am sure you all have more experience at this than me, and I would

>appreciate any insight or advice you all might have in this matter.

>Thank you very much in advance.

>-Shane M. Randle

>

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: GE Caps

Date: Mon, 16 Jun 1997 11:57:20 GMT

Organization: The Energy Machine Information System 504-734-0526

From: rjb@pchem.chem.umass.edu (Rick Bishop)

Date: Mon, 16 Jun 1997 07:57:20 -0400

Subject: GE Caps

Hello,

I've just managed to come across some GE caps, rated 4.8uF 12500VDC. I have a

few questions about these before I hook them up to my coil.

I'm running a small coil powered by a 12kV 30ma neon sign transformer. I gather

that if I place two of the caps in series, this will be equivalent to a 2.4uF

25kV, right? This should be able to stand up to my neon ok? Also, is it safe to

use such a high capacitance on a small coil? I imagine that it will discharge

through the gap before it is anywhere near fully charged anyway. Or would it be

better to hook up more in series to reduce the capacitance (I can get up to 6

of them, but they are quite large.)

In case it's needed, coil specs are:

12kV 30ma Neon Xmfr

Primary: 14 turns 12awg copper diameter 10"

Sec: Approx 950 turns 24awg on 4" treated PVC

Cap: currently 8 saltwater caps

Gap: cylinder

Thanks for any help,

Rich

--

Richard Bishop          | Tel   (413) 545-2888

Department of Chemistry |

Box 34510               | Email rjb@chemistry.umass.edu

Umass                   | WWW   http://pchem.chem.umass.edu/~rjb/index.html

Amherst MA 01003-4510   |

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Ignition TC

Date: Mon, 16 Jun 1997 17:11:44 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Mon, 16 Jun 1997 19:11:44 +0200 (SAT)

Subject: Ignition TC

Hi,

I was wondering if a person can build a Tesla coil that has a transformer

made of say two ignition coils. If it is possible and someone knows how to

build such a Tesla coil please send me info on how. If you also know the

amps of a 30 kV please let me know.

Thanks

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1997                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: GE Caps

Date: Mon, 16 Jun 1997 19:13:24 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 16 Jun 1997 12:13:24 -0700

Subject: Re: GE Caps

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: rjb@pchem.chem.umass.edu (Rick Bishop)

> Date: Mon, 16 Jun 1997 07:57:20 -0400

> Subject: GE Caps

>

> Hello,

>

> I've just managed to come across some GE caps, rated 4.8uF 12500VDC. I have a

> few questions about these before I hook them up to my coil.

>

> I'm running a small coil powered by a 12kV 30ma neon sign transformer. I gather

> that if I place two of the caps in series, this will be equivalent to a 2.4uF

> 25kV, right? This should be able to stand up to my neon ok? Also, is it safe to

> use such a high capacitance on a small coil? I imagine that it will discharge

> through the gap before it is anywhere near fully charged anyway. Or would it be

> better to hook up more in series to reduce the capacitance (I can get up to 6

> of them, but they are quite large.)

>

> In case it's needed, coil specs are:

>

> 12kV 30ma Neon Xmfr

> Primary: 14 turns 12awg copper diameter 10"

> Sec: Approx 950 turns 24awg on 4" treated PVC

> Cap: currently 8 saltwater caps

> Gap: cylinder

>

> Thanks for any help,

>

> Rich

>

> --

>

> Richard Bishop          | Tel   (413) 545-2888

> Department of Chemistry |

> Box 34510               | Email rjb@chemistry.umass.edu

> Umass                   | WWW   http://pchem.chem.umass.edu/~rjb/index.html

> Amherst MA 01003-4510   |

>

> -----

> Processed with Listserv v2.93 for Wildcat v4

Richard,

These caps are too large to be of use in your coil. The neon

transformers will not be able to charge them up to a high enough voltage

to even fire your gap. Also, even if you drove them from a higher

capacity power source, you'd need a MUCH larger secondary coil and

toroid in order to even begin coming close to being in tune and to be

able to handle to power. Even VERY large coils seldom use tank caps

greater than 0.5 uF, and most large coils top out at about 0.2uF. Now 6

of these in series would get you in the right range for a very large

coil, powered off a potential transformer or a pole-pig.

There may be a couple of other problems as well - these sound like they

might be power filter or power factor correction caps, not rated for

pulse-discharge Tesla Coil work. If they are paper or Mylar dielectric,

their performance may still leave much to be desired due to the lossy

dielectric at RF frequencies. These caps might be usable for making a

can crusher however. You might want to check out:

http://hibp.ecse.rpi.edu/~chowm/can_crusher.html

Watch out - these caps pack energy levels that make them potential

killers - you'll not get any second chances if you accidently get across

one of these!!

_Safe_ coilin' to you!

-- Bert H --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: This thing isn't working!!

Date: Tue, 17 Jun 1997 01:58:26 GMT

Organization: The Energy Machine Information System 504-734-0526

From: RSNV02D@prodigy.com ( MATT   MCARTHUR)

Date: Mon, 16 Jun 1997 20:58:26, -0500

Subject: This thing isn't working!!

Hello all,

I have just been signed on to this list and have

several questions to ask about my first coil.

I keep melting the transformers.  They are 10KV

@ 23mA Oil burners and I thought they were almost

invincible (key words, were and almost)

My current specs are .0065uF salt water caps,

7" diameter, 6" tall solenoid style primary, 4 static

gaps @ .025" each, .5" safety gap, and a 3.5" diam.,

12.25" long secondary wound with 500 turns of #23 wire

and a 5" sphere on the top.

All was working as it was after I put a new tranny in it,

then it putzd out and hummed. It still drew the same

amount of current, so my guess is shorted secondary

windings.

If anyone can help me keep from killing all of these poor,

innocent, obit's, I would greatly appreciate it.

Thanks

--Matt

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: This thing isn't working!!

Date: Tue, 17 Jun 1997 04:33:06 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 16 Jun 1997 21:33:06 -0700

Subject: Re: This thing isn't working!!

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: RSNV02D@prodigy.com ( MATT   MCARTHUR)

> Date: Mon, 16 Jun 1997 20:58:26, -0500

> Subject: This thing isn't working!!

>

> Hello all,

>

>   I have just been signed on to this list and have

> several questions to ask about my first coil.

> I keep melting the transformers.  They are 10KV

> @ 23mA Oil burners and I thougth they were almost

> invincible (key words, were and almost)

>   My current specs are .0065uF salt water caps,

> 7" diameter, 6" tall solenoid style primary, 4 static

> gaps @ .025" each, .5" safety gap, and a 3.5" diam.,

> 12.25" long secondary wound with 500 turns of #23 wire

> and a 5" sphere on the top.

>   All was working as it was after I put a new tranny in it,

> then it putzd out and hummed. It still drew the same

> amount of current, so my guess is shorted secondary

> windings.

>   If anyone can help me keep from killing all of these poor,

> innocent, obit's, I would greatly appreciate it.

>

> Thanks

>

Matt,

I assume your safety gap is right across the transformer output? You

need to reduce the safety gap spacing - at 1/2" the gap is not going to

break down before your transformer does. You also should add a couple of

air-wound RF chokes (RFC's) in series with your transformer output.

Here's a reprint of a schematic that Mike Hammer had in an earlier

post:

Here is a generic typical diagram for a coil.

TESLA COIL SCHEMATIC                              ---------

|       | T1

---------

O

X1                               C1             O

RFC           | |             O  L2

O---------------||||----------| |-----        O

||O    |                   |    | |    |   L1   O

||O    |                   |           |    O   O

-----O||O    * SG                *           |--->O   O

O||O----*---|Gnd               G1            O   O

-----O||O    *                   *                O   O

120   ||O    |                   |                O   O

Volts ||O    |                   |                O   O

O---------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded center tap (or OBIT)

SG = Safety Gap.

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

The RFC Chokes each should be made from 24 to 28 AWG wire close wound on

2" PVC pipe for a length of 5-6 inches. The chokes will help keep out

the high frequency "hash" coming from the the sparkgap and local wiring.

The safety gap should be adjusted so that is just a bit beyond the point

that it will fire with the rest of the tank circuit disconnected. This

will insure that the transformer does not see voltages significantly

higher that the normal open circuit voltage of the transformers.

BTW, oil burner ignition transformers (OBIT's) seem to  be relatively

more sensitive to overvoltages than a neon - try to get some used neon

transformers if you can. The 60 mil type also seem to be somewhat more

robust than the 30 mil ones.

Safe coilin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Chokes

Date: Tue, 17 Jun 1997 12:32:50 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Tue, 17 Jun 1997 14:32:50 +0200 (SAT)

Subject: Chokes

Hi, guys

I have a transformer of 15000 VAC 30 mA and want to build a choke for my

transformer. (I don't know much about chokes) Can anyone give me info on how

to build it, the reason I'm asking you is that I can't unzip some of the

files on ftp.funet.fi

I really don't want to buy a new transformer if something goes wrong.

Thanks

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1997                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: GE Caps

Cc:

Bcc:

X-Attachments:

At 11:57 AM 6/16/97 GMT, you wrote:

>From: rjb@pchem.chem.umass.edu (Rick Bishop)

>Date: Mon, 16 Jun 1997 07:57:20 -0400

>Subject: GE Caps

>

>Hello,

>

>I've just managed to come across some GE caps, rated 4.8uF 12500VDC. I have a

>few questions about these before I hook them up to my coil.

>

>I'm running a small coil powered by a 12kV 30ma neon sign transformer. I gather

>that if I place two of the caps in series, this will be equivalent to a 2.4uF

>25kV, right? This should be able to stand up to my neon ok? Also, is it safe to

>use such a high capacitance on a small coil? I imagine that it will discharge

>through the gap before it is anywhere near fully charged anyway. Or would it be

>better to hook up more in series to reduce the capacitance (I can get up to 6

>of them, but they are quite large.)

>

>In case it's needed, coil specs are:

>

>12kV 30ma Neon Xmfr

>Primary: 14 turns 12awg copper diameter 10"

>Sec: Approx 950 turns 24awg on 4" treated PVC

>Cap: currently 8 saltwater caps

>Gap: cylinder

>

>Thanks for any help,

>

>Rich

>

Rich,

Unfortunately what you have here is DC filter caps

mean for High voltage DC power supply use and not at all suitable

for a Tesla coil.

These caps will have high loss in RF use and the capacitance is way

way too high.

Your 12kv/30 ma neon is about right for a .0066 mfd cap

and could probably handle up to .008 mfd.

Even if you wired all 6 in series the capacitance would still

be .8 mfd. Thats 100 times too large.

Even if you had a large pole pig transformer its still way too

much capacitance.

Those caps might be useful in other types of high voltage

experiments but not for a Tesla coil

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Ignition TC

Cc:

Bcc:

X-Attachments:

At 05:11 PM 6/16/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Mon, 16 Jun 1997 19:11:44 +0200 (SAT)

>Subject: Ignition TC

>

>Hi,

>

>I was wondering if a person can build a Tesla coil that has a transformer

>made of say two ignition coils. If it is possible and someone knows how to

>build such a Tesla coil please send me info on how. If you also know the

>amps of a 30 kV please let me know.

>

Richard,

While it is possible to build a SMALL coil driven

with automotive ignition coils its performance will pale

in comparison to even the smallest neon transformer.

Its simply a matter of power. Most ignition coils get mighty

warm running any more than 60 watts or so.

By comparison a small 6kv/30ma neon is 180 watts.

My smallest neon is 7500/30 which is 225 watts.

And I've used it to drive some coils to 14 inch

output streamers.

I experimented with ignition coils a couple

years ago and was not at all happy with the results.

I got a couple neons and never looked back.

More power to ya,

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: This thing isn't working!!

Cc:

Bcc:

X-Attachments:

At 01:58 AM 6/17/97 GMT, you wrote:

>From: RSNV02D@prodigy.com ( MATT   MCARTHUR)

>Date: Mon, 16 Jun 1997 20:58:26, -0500

>Subject: This thing isn't working!!

>

>Hello all,

>

>  I have just been signed on to this list and have

>several questions to ask about my first coil.

>I keep melting the transformers.  They are 10KV

>@ 23mA Oil burners and I thougth they were almost

>invincible (key words, were and almost)

>  My current specs are .0065uF salt water caps,

>7" diameter, 6" tall solenoid style primary, 4 static

>gaps @ .025" each, .5" safety gap, and a 3.5" diam.,

>12.25" long secondary wound with 500 turns of #23 wire

>and a 5" sphere on the top.

>  All was working as it was after I put a new tranny in it,

>then it putzd out and hummed. It still drew the same

>amount of current, so my guess is shorted secondary

>windings.

>  If anyone can help me keep from killing all of these poor,

>innocent, obit's, I would greatly appreciate it.

>

>Thanks

>

>   --Matt

Well Matt one of the first hard lessons you

learn early in Tesla coiling is that its hard

on components. Neon transformers and oil burner transformers

are a bit delicate and really dont like the use we are putting them to.

But there are things we can do to delay the inevitable as

long as possible.

First of all how is your circuit wired? Do you have your main cap

in parallel across your transformer? And the spark gap in series

with the primary? If so that is a bad combination.

Any RF that appears across the main cap will also appear across

the transformer secondary since they are in parallel.

Rearrange your circuit so the spark gap is in parallel across

the transformer and the cap in series with primary. In other

words swap the position of the gap and cap.

In this configuration the transformer secondary is shorted by the

energized gap while the heavy stuff is occurring in the coil primary.

Next up is the safety gap. Is your safety gap firing? If you open

it up until it stops firing it is no longer "safe". Adjust your safety

so it fires every few seconds or so. As your system comes closer

to perfect tune you will get less kickbacks and less activity on

the safety gap.

Next item is RF chokes. These need to be from 2 - 10 mh and

placed in the HV lines between the safety gap and main spark gap.

These chokes will cut down on the amount of RF heading back toward

your transformer.

I've used 125 turns of AWG 26 on a large ferrite toroid. This gives

about 2.25 mh and a pair of these chokes work well. Put one in each

High voltage leg. Ferrite rods are also good.

Air core chokes can be made by winding several hundred turns on a

PVC form. A 5 inch long winding of AWG 28 on a piece of 2 inch PVC

pipe will have about 360 turns and a little over 3 mh of inductance.

For stubborn cases and recommended when using air core chokes is a little

series resistance. Try about 1000 ohms at 50 watts or so.

The series resistance will kill the Q of the air core chokes

and prevent them from ringing. Some times air core chokes can ring up

like mini Tesla coils and produce rather high voltages.

The current flow through these resistors is low so the power loss

from them is minimal.

I will include a typical diagram here to go with the above text.

TESLA COIL SCHEMATIC                              ---------

|       | T1

---------

O

X1                               C1              O

RFC           | |             O  L2

O---------------||||----------| |-----        O

||O    |                   |    | |    |   L1   O

||O    |                   |           |    O   O

-----O||O    * SG                *           |--->O   O

O||O----*---|Gnd               G1            O   O

-----O||O    *                   *                O   O

120   ||O    |                   |                O   O

Volts ||O    |                   |                O   O

O---------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded center tap (or OBIT)

SG = Safety Gap.

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

Good luck

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Chokes

Cc:

Bcc:

X-Attachments:

At 12:32 PM 6/17/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Tue, 17 Jun 1997 14:32:50 +0200 (SAT)

>Subject: Chokes

>

>Hi, guys

>

>I have a transformer of 15000 VAC 30 mA and want to build a choke for my

>transformer. (I don't know much about chokes) Can anyone give me info on how

>to build it, the reason I'm asking you is that I can't unzip some of the

>files on ftp.funet.fi

>

>I really don't want to buy a new transformer if something goes wrong.

Richard,

See the post I sent out yesterday to Matt McArthur on his

problem with burning out his oil burner transformers.

The same info applies.

Make sure your spark gap is placed across the HV outputs not

your cap.

Place a close spaced safety gap directly across the neon HV terminals

to prevent over voltage.

Place RF chokes of at least 2 millihenry in the HV lines in between

the main gap and safety gap.

If you can find some large ferrite toroids these make excellent RF chokes.

However the surplus source for these has dried up so you're on your own.

Ferrite rods would work as well.

Air core chokes will work OK as long as you dont put too many turns

on them. If you get lots of turns they will want to act

like little Tesla coils and create hi voltages. Just the thing

you are trying to prevent.

About 350 turns or so of AWG 28 on a piece of 2 inch PVC

will have about 3 mh of inductance. The use of a little series

resistance is recommended with air core chokes to keep them from

ringing.

The most important piece is the safety gap. I have run successfully

without RF chokes using just a safety gap. However without

the chokes the kickbacks across the safety are more numerous

and more violent.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Primary coil

Date: Fri, 20 Jun 1997 09:14:42 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Fri, 20 Jun 1997 11:14:42 +0200 (SAT)

Subject: Primary coil

Hi, guys

Can anyone tell me what size my primary's diameter should be. I don't know

where will a person find the place for maximum energy transfer. In some

books they say that you should rewind and change your diameter and check if

your coil performs better.

I'm asking you guys because my primary coil is made out of wood, otherwise I

will go and make a lot of wood rolls to make my 1/4 inch coper turn

around.(sorry if I'm not speaking clearly but I can't get onto the right

word now, my home language is Afrikaans) I'm speaking about the primary coil

that you wind the coper pipe around.

Does anyone have a tip or formula that I can calculate it.

Has anyone an idea how to build a toroid. It should be of aluminium. I have

created one once but the high voltage goes of from the sharp point. Anyone

got an idea to make it more solid the aluminum foil.

Thanks

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Primary coil

Date: Fri, 20 Jun 1997 17:02:09 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Adam Gramly <richardgramly@sprintmail.com>

Date: Fri, 20 Jun 1997 12:02:09 -0500

Subject: Re: Primary coil

Organization: home

Tesla-2 wrote:

>

> From: Richard <esmit@ilink.nis.za>

> Date: Fri, 20 Jun 1997 11:14:42 +0200 (SAT)

> Subject: Primary coil

>

> Hi, guys

>

> Can anyone tell me what size my primary's diameter should be. I don't know

> where will a person find the place for maximum energy transfer. In some

> books they say that you should rewind and change your diameter and check if

> your coil performs better.

>

> I'm asking you guys because my primary coil is made out of wood, otherwise I

> will go and make a lot of wood rolls to make my 1/4 inch coper turn

> around.(sorry if I'm not speaking clearly but I can't get onto the right

> word now, my home language is Afrikaans) I'm speaking about the primary coil

> that you wind the coper pipe around.

>

> Does anyone have a tip or formula that I can calculate it.

>

> Has anyone an idea how to build a toroid. It should be of aluminium. I have

> created one once but the high voltage goes of from the sharp point. Anyone

> got an idea to make it more solid the aluminum foil.

>

> Thanks

>  ------------------------------------------------------------------------> |R

ichard Smit  | Telephone: +27-525-32634                                |

> |P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

> |Bultfontein   | Age:       17 years old                                 |

> |9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

> |South Africa  | Birthday:  5 October 1979                               |

> ------------------------------------------------------------------------>

> * Send list Replies to: Tesla-2@emachine.com

>

> * To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

>   in the body of your message to: Listserv@Emachine.com

>

> -----

> Processed with Listserv v2.93 for Wildcat v4

I am not sure of the correct formula for the size of a primary coil, but

I do know that it should be about five times the diameter of the

secondary.  What shape is your primary going to be?

You can mace a fairly good toroid by wraping the aluminum foil around a

foam or plastic form.  You could also weld two aluminum salad bowls

together, and sand off the edges.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Toroid plans

Date: Fri, 20 Jun 1997 17:51:11 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Fri, 20 Jun 1997 19:51:11 +0200 (SAT)

Subject: Toroid plans

Hi,

I was wondering if anyone has a plan on how to construct your own torroid.

If anyone of you guys have a toroid that you feel you don't want or want to

get rid of, please contact me I will buy if from you if anyone can help me.

Email me.

Thanks

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: KEVIN BATCHELOR Torroid

Date: Fri, 20 Jun 1997 17:56:30 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Fri, 20 Jun 1997 19:56:30 +0200 (SAT)

Subject: KEVIN BATCHELOR Torroid

If anyone can email this to Kevin, would you please do it, I can't seem to

be able to send it to him, some problem i'm having.

Dear, Kevin

Thanks for your email and info you send me. Where I live in South Africa, in

a very small town we only have one hardware store and it doesn't sell

aluminum ducting. I would be glad if  you can help me, if you have some toroids

that are broken or that you will never use I would be happy if you can send them

for me, I will pay you but as long as it is cheap. The reason why it shouldn't be

a high price is because if I buy something like $23 then it costs me about 4.5 times

that much and then it will be R126, that is a whole lot of money.

I would be thankful if you can help me.

Thanks

Richard

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Primary coil

Cc:

Bcc:

X-Attachments:

At 09:14 AM 6/20/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Fri, 20 Jun 1997 11:14:42 +0200 (SAT)

>Subject: Primary coil

>

>Hi, guys

>

>Can anyone tell me what size my primary's diameter should be. I don't know

>where will a person find the place for maximum energy transfer. In some

>books they say that you should rewind and change your diameter and check if

>your coil performs better.

Depends a great deal on the type of primary you are winding.

If winding a solenoid type primary then its diameter

should be at least twice the diameter of your secondary.

So if winding a primary for a secondary on 4 inch PVC with an OD

of 4.5 inches then you want the primary at least 9 inches diameter.

In this case going to 10 inches wouldnt hurt.

For a flat spiral or saucer shaped primary the inside turn

should be spaced at least 1 inch from the secondary.

That means for our 4.5 inch OD secondary the inner turn

diameter would be 6.5 inches. The turns spiral outward with

a 1/4 inch turn to turn spacing and the final outside diameter

depends on the number of turns. Go for at least 12 turns

and going to 14 wouldnt be a bad idea.

>

>I'm asking you guys because my primary coil is made out of wood, otherwise I

>will go and make a lot of wood rolls to make my 1/4 inch coper turn

>around.(sorry if I'm not speaking clearly but I can't get onto the right

>word now, my home language is Afrikaans) I'm speaking about the primary coil

>that you wind the copper pipe around.

A wood form for your primary is OK. You dont need to make a lot

of different primaries to find the one that works best.

Make one primary with more than enough turns (12-14) and

tap in the number of turns you need to achieve resonance.

Adjust the relative position of the secondary and primary

to vary the coupling. Try to get the secondary down as close

to the primary as possible without getting primary/secondary arcing.

If you do get arcing raise up the secondary a little at a

time until it stops.

>Has anyone an idea how to build a toroid. It should be of aluminium. I have

>created one once but the high voltage goes of from the sharp point. Anyone

>got an idea to make it more solid the aluminum foil.

>

I use flexible aluminum dryer duct. It comes avaiable in various

diameters. For a small coil the 3 or 4 inch inch duct will make

a nice toroid. This stuff is nice because you dont need to cover it

with foil. use it as is. Form it into a ring and tape the two ends together.

Cover the taped joint with aluminum tape or foil. Mount a pair of pie

pans bottom to bottom inside the duct ring. Bolt the pans together

and as they are drawn together they will grip the duct ring and form

a solid unit.

Heres a crude ascii diagram that I stole from somebody.

.                        .

.     .\    Pie Pans    /.     .

\--------------/     Dryer Duct

.     . /--------------\ .     .

.   /                \   .

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Toroid plans

Cc:

Bcc:

X-Attachments:

At 05:51 PM 6/20/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Fri, 20 Jun 1997 19:51:11 +0200 (SAT)

>Subject: Toroid plans

>

>Hi,

>

>I was wondering if anyone has a plan on how to construct your own torroid.

>

>If anyone of you guys have a toroid that you feel you don't want or want to

>get rid of, please contact me I will buy if from you if anyone can help me.

>Email me.

>

>Thanks

The toroids we make are very light weight

and not very strong and not really shippable and it would

cost much more to ship one than its worth.

Dont worry too much about having a toroid if you cant find

some materials locally. The whole idea of Tesla coiling

is improvising equipment out of what you have or can find.

Try putting aluminum foil around a florists styrofoam ring.

Or an inner tube from a garden tractor tire.

Or use a cheap stainless steel salad bowl.

I got good result from a 6 inch diameter styrfoam ball covered

with foil.

Try anything metal. I used a quart paint can once on a small

coil. It works. Use what you got or can find. Be creative.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Spark gap performance

Date: Sun, 22 Jun 1997 14:33:12 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Sun, 22 Jun 1997 16:33:12 +0200 (SAT)

Subject: Spark gap performance

Hi,

A few weeks ago I was changing my sparkgap. The first time I had ran my coil

I only used a spark gap with only two electrodes. My Tesla coil was only

giving about an 2 inches of spark lenth but only when an object came near

it. I changed it (as I described to some of the people) to a sparkgap with

about 15 electrodes in it so that the spacing is a lot smaller between the

nails. My Tesla coil's performance went to about 8 inches sparks and it were

like a flame, the sparks running in a linear direction(see fig.1). Can

anyone tell me why it makes your Tesla coil perform better.

Someone wrote about the Banjo effect, what in the world is that?

FIG.1

/

|------> spark

\

|

/

-----

|     | ------> secondary coil

|     |

|     |

GREAT NEWS: Today I found a sort of swimming pool pipe from my fathers

garage and now I'm going to tie the points together and cover it in foil, so

at last I can make a Torroid.

See yah!

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Spark gap performance

Cc:

Bcc:

X-Attachments:

At 02:33 PM 6/22/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Sun, 22 Jun 1997 16:33:12 +0200 (SAT)

>Subject: Spark gap performance

>

>Hi,

>

>A few weeks ago I was changing my sparkgap. The first time I had ran my coil

>I only used a spark gap with only two electrodes. My Tesla coil was only

>giving about an 2 inches of spark lenth but only when an object came near

>it. I changed it (as I described to some of the people) to a sparkgap with

>about 15 electrodes in it so that the spacing is a lot smaller between the

>nails. My Tesla coil's performance went to about 8 inches sparks and it were

>like a flame, the sparks running in a linear direction(see fig.1). Can

>anyone tell me why it makes your Tesla coil perform better.

First of all using nails for spark gap electrodes would be

very inefficient. Not enough surface area. They will quickly overheat

and cease to function well.

By running many small gaps in series each individual gap has

to handle only a smaller part of the total load. So the total

amount of loss goes down. 15 series gaps are probably a bit

too many for a 15kv neon.

I suggest you make a new gap using sections of copper tubing for the

electrodes. 3/4 inch copper tubing cut into sections 2 or 3 inches

long will make a very good gap. For your 15kv neon use 10 gaps which

will require 11 electrodes. Space the electrodes .025 inch from each other.

This will give .25 inch total amount of gap.

If you build such a gap I bet you will see a big jump in performance.

>

>Someone wrote about the Banjo effect, what in the world is that?

Never heard that reference before. Cant help you there.

>

>GREAT NEWS: Today I found a sort of swimming pool pipe from my fathers

>garage and now I'm going to tie the points together and cover it in foil, so

>at last I can make a Torroid.

>

Be aware that you may get poorer performance from adding a toroid

at first. A toroid will require more power to breakout with a spark

than a smaller spherical terminal. You are not producing much

power with your coil right now. The best way for you to boost

your power level is with a better gap.

If you cant get any spark from your toroid try laying a wire

off the edge of it to provide a point to spark from.

Also a toroid will lower the resonant frequency of your secondary

so make sure you have sufficient turns in your primary coil

to tune to this new lower frequency. Remember your primary

and secondary resonant frequencies must match.

TTYL!

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Transco Transformer Rebuild

Date: Tue, 24 Jun 1997 15:47:04 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Kevin Batchelor <KBATCHELOR@arslimited.com>

Date: Tue, 24 Jun 1997 11:47:04 -0400

Subject: Transco Transformer Rebuild

I am getting ready to unpot and rebuild a Transco 15Kv 60ma

transformer.  Wondered if anyone had any experience with this type and

had any advice to offer.  Additionally wondering about the rebuild and

removal of magnetic shunts.

Any comments would be helpful.  I just can't wait to turn into a "Tar

Baby".

Thanks,

Kevin

KBATCHELOR@ARSLIMITED.COM

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: capacitor

Date: Tue, 24 Jun 1997 16:01:40 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Tue, 24 Jun 1997 18:01:40 +0200 (SAT)

Subject: capacitor

Hi,

I have redesign my capacitor that was made out of glass and foil.

My new cap is made out of paper and foil, both dipped in oil so that the

paper has the same dielectric strength of polyethylene. At last my spark

from the secondary is more intense than ever before but now I get about 6

inches spark and not 18 inches as I should from my 15000VAC 30mA. Can anyone

help me, I don't know what is wrong.

What can cause the intense spark but not any longer sparks.

Thanks

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: capacitor -Reply

Date: Tue, 24 Jun 1997 17:00:11 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Kevin Batchelor <KBATCHELOR@arslimited.com>

Date: Tue, 24 Jun 1997 13:00:11 -0400

Subject: capacitor -Reply

>My new cap is made out of paper and foil, both dipped in oil so that the

>paper has the same dielectric strength of polyethylene. At last my spark

>from the secondary is more intens than ever before but now I get about

>6 inches spark and not 18 inches as I should from my 15000VAC 30mA.

>Can anyone help me, I don't know what is wrong.

Have you tried changing the spark gap?  I believe you were using nails.

Perhaps if you increase your gap your capacitor will charge further

before breaking down.  What is the capacitance of your capacitor?  Did

you change your toroid?  You may also need a larger toroid to build up the

charge before discharging.  If the toroid is small, it will break out sooner

with smaller sparks.

Keep Playing!

Kevin

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: capacitor

Date: Wed, 25 Jun 1997 06:24:08 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Wed, 25 Jun 1997 08:24:08 +0200 (SAT)

Subject: Re: capacitor

Transformer 15kVAC 30mA

Secondary coil: Diameter 4.4 inch

height 24 inches

723 turns of #21 AWG

Frequency about 300kHz

Primary coil: 15 turns

Height 6 inches

Diameter 8.8 inches

Capacitor

Foil size: 5.2 inches X 8 inches

number of foil: 28

Paper size: A4 size

paper thickness: 90 mils

Capacitor .0055 soaked in transformer oil.

Toroid about twice the size of the secondary diameter and 1 inch high.

At 07:11 PM 25-06-97 GMT, you wrote:

>From: Mike Hammer <mhammer@misslink.net>

>Date: Wed, 25 Jun 1997 14:11:56 -0500

>Subject: Re: capacitor

>

>At 04:01 PM 6/24/97 GMT, you wrote:

>>From: Richard <esmit@ilink.nis.za>

>>Date: Tue, 24 Jun 1997 18:01:40 +0200 (SAT)

>>Subject: capacitor

>>

>>Hi,

>>

>>I have redesign my capacitor that was made out of glass and foil.

>>

>>My new cap is made out of paper and foil, both dipped in oil so that the

>>paper has the same dielectric strength of polyethylene. At last my spark

>>from the secondary is more intens than ever before but now I get about 6

>>inches spark and not 18 inches as I should from my 15000VAC 30mA. Can anyone

>>help me, I don't know what is wrong.

>>

>>What can cause the intense spark but not any longer sparks.

>>

>>Thanks

>>

>

>Richard,

>

>This is a pretty normal result of improving your main cap.

>A better main cap will not usually give you much increase

>in spark length but will make your streamers hotter, brighter

>and louder.

>

>Paper may not be a great dielectric for a Tesla cap

>but as you can see its a lot better than glass.

>

>You might post the specs on your cap. I would be interested

>in seeing what you have done.

>

>You didnt mention what the value of your new cap was estimated to be.

>You may get an increase in spark length by using more capacitance

>up to the limit of what your transformer can charge. In your

>case with a 15kv/30ma neon that would be .0053 mfd.

>

>To get longer sparks work on your spark gap. This will give you

>the biggest improvement.

>

>TTYL!

>Mike Hammer

>mhammer@misslink.net

>

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Secondary grounding

Date: Wed, 25 Jun 1997 19:01:35 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Shane Randle <randlec@scnc.addison.k12.mi.us>

Date: Wed, 25 Jun 1997 15:01:35 -0400

Subject: Secondary grounding

I am just beginning to build my secondary coil, and I was wondering how

the coil should be grounded. I've never done this before, and I was just

wondering how you guys do it. Thanks in advance for any help you can

give me.

-Shane M. Randle

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: capacitor

Cc:

Bcc:

X-Attachments:

At 04:01 PM 6/24/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Tue, 24 Jun 1997 18:01:40 +0200 (SAT)

>Subject: capacitor

>

>Hi,

>

>I have redesign my capacitor that was made out of glass and foil.

>

>My new cap is made out of paper and foil, both dipped in oil so that the

>paper has the same dielectric strength of polyethylene. At last my spark

>from the secondary is more intens than ever before but now I get about 6

>inches spark and not 18 inches as I should from my 15000VAC 30mA. Can anyone

>help me, I don't know what is wrong.

>

>What can cause the intenser spark but not any longer sparks.

>

>Thanks

>

Richard,

This is a pretty normal result of improving your main cap.

A better main cap will not usually give you much increase

in spark length but will make your streamers hotter, brighter

and louder.

Paper may not be a great dielectric for a Tesla cap

but as you can see its a lot better than glass.

You might post the specs on your cap. I would be interested

in seeing what you have done.

You didnt mention what the value of your new cap was estimated to be.

You may get an increase in spark length by using more capacitance

up to the limit of what your transformer can charge. In your

case with a 15kv/30ma neon that would be .0053 mfd.

To get longer sparks work on your spark gap. This will give you

the biggest improvement.

TTYL!

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: plastic capacitor

Date: Thu, 26 Jun 1997 15:46:08 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Thu, 26 Jun 1997 17:46:08 +0200 (SAT)

Subject: plastic capacitor

Hi,

I was wondering if anyone can give me some suggestions.

I have build my paper cap as I told ya. I was just wondering if a person can

build a cap out of 20 micron plastic bags and put them in series to handle

the 15 kVAC power. I read it somewhere about Bill Richards that had used zip

locker bags(whatever that is) and put them in series to handle the voltage.

He said it has a very high Q factor. If something has a very high Q factor

your spark will get better and the spark gap will fire faster. Would it work

if I will do the experiment. I'm ready to try anything as long as I get the

best sparks that I can get.

Does anyone know Bill Richards email address?

Thanks

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: plastic capacitor

Date: Thu, 26 Jun 1997 18:37:53 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Engle, Daniel (NJAOST)" <DEngle@NJAOST.ML.com>

Return-Receipt-To: "Engle, Daniel (NJAOST)" <DEngle@NJAOST.ML.com>

Date: Thu, 26 Jun 1997 14:37:53 -0400

Subject: RE: plastic capacitor

I think he meant Zip-Loc bags.  Somewhere along the lines of "yellow and

blue make green!" .  I'm not quite sure I follow you on the cap idea

though.  Could you explain in a little more detail about putting "them

in series to handle the voltage.  Do you mean something like a salt

water capacitor?  Instead of using the glass bottles, use the zip locks?

Curious minds want to know! :-)

Dan Engle

-----Original Message-----

From:   tesla-2@emachine.com [SMTP:tesla-2@emachine.com]

Sent:   Thursday, June 26, 1997 11:46 AM

To:     Engle, Daniel (NJAOST)

Subject:        plastic capacitor

From: Richard <esmit@ilink.nis.za>

Date: Thu, 26 Jun 1997 17:46:08 +0200 (SAT)

Subject: plastic capacitor

Hi,

I was wondering if anyone can give me some suggestions.

I have build my paper cap as I told ya. I was just wondering if

a person can build a cap out of 20 micron plastic bags and put them in series

to handle the 15 kVAC power. I read it somewhere about Bill Richards that

had used zip locker bags(whatever that is) and put them in series to handle

the voltage.

He said it has a very high Q factor. If something has a very

high Q factor your spark will get better and the spark gap will fire faster.

Would it work if I will do the experiment. I'm ready to try anything as long

as I get the best sparks that I can get.

Does anyone know Bill Richards email address?

Thanks

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Alignment of Caps

Date: Fri, 27 Jun 1997 05:53:53 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Fri, 27 Jun 1997 07:53:53 +0200 (SAT)

Subject: Alignment of Caps

Hi,

I only have two questions today that is bugging me.

1. When you want the right voltage for a capacitor you put them in series and

when you want the right capacitance  you put them in parallel. Which of

these to figures is right.

Fig.1

||

|-----||-------| Total .005 uF

|     ||       |

------|              |---------- TOTAL OUTPUT .0053 uF

| ||  || || || |

|-||--||-||-||-| Total .0003 uf

||  || || ||

Fig.2 |

||  || || ||   Total .0053 uF

|-||--||-||-||-|

| ||  || || || |

|              |

---------|              |-------------- TOTAL OUTPUT .01 uF

| ||  || || || |

|-||--||-||-||-| Total .0053 uF

||  || || ||

2. I have read about a Tesla coils spark that should bet thick, but my Tesla

coil is giving thin purple arcs, Is it right if it does so or is it wrong?

Thanks.

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Alignment of Caps

Date: Fri, 27 Jun 1997 14:48:53 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Fri, 27 Jun 1997 07:48:53 -0700

Subject: Re: Alignment of Caps

Organization: Stoneridge Engineering

Richard,

Question 1:

A couple of clarifications. Connecting capacitors is series to increase

breakdown voltage works well only when all the capacitors are of the

same value and type of construction. In this case the voltage that each

one "sees" during the time they're charging AND during the time they're

discharging through the tank circuit is shared fairly equally between

them. Generally, in order to increase the working voltage, groups of

identical capacitors are connected in series. To increase the amount of

capacitance, groups of these groups then need to be connected in

parallel.

1. Series Connected Caps:

For capacitors that are connected in series, the total capacitance is

reduced. The general formula for series-connected capacitors is:

1/Cs = (1/C1) + (1/C2) + (1/C3)...

When C1 = C2 = C3 (etc) = C, this can be simplified to:

Cs = C/N   where N is the number of series-connected capacitors,

each               having capacitance of C

In this case the total voltage across the group will be equally shared

across the individual caps by the same factor:

Vcap = Vtotal/N

2. Parallel-Connected Caps:

Capacitors connected in parallel will be additive:

Cp = C1 + C2 + C3...

When C1 = C2 = C3 (etc) = C, this can be simplified to:

Cp = N*C

In this case, the total current is equally shared by the individual

caps.

3. Series/Parallel Combinations:

Compute the individual capacitances of each series string first, using

the first formula. Then add the capcitances of the series-strings

together using the formula for parallel capacitors to arrive at the

total capacitance.

With the above in mind, let's try to answer your first question. If the

individual total values are correct as yyou've shown them below, then

both figures below are numerically correct. However, figure 2 is more

typically what you want to use to get the right capacitance at the right

breaksown voltage for Tesla Coiling. Figure 1 places the full voltage

stress across the top capacitor instead of sharing it with identical

units.

If all the individual capacitors were the SAME value, then the total

values would be different. You can use the formulas above to find out

what these would be for your caps.

Question 2:

The appearance of the discharge is a function of a lot of things.

Generally, the most important things are frequency and power. Smaller

coils tend to operate at higher frequencies and lower power.

Higher-frequency systems tend to have a more "brushlike" appearance at

the relatively low power levels of most small coils. In order to form a

hot and thick discharges, considerable power must be efficiently

transferred from the secondary coil/toroid to the ionized air. This

means that we've got to provide significant power to the primary tank

circuit, and efficiently transfer it to the secondary side, and we've

got to prevent the secondary from prematurely losing energy via a

properly sized toroid, or dumping energy back into the primary due to a

poorly quenching gap.

Losses stemming from lossy capacitor dielectrics, high RF resistance

primary circuit, a poor RF grounding system, or poorly quenching gaps

will rob you of energy available to the secondary. A toroid that breaks

out too early will cause the discharge to be spread out among many

smaller discharges instead of a single hot discharge. Successful coiling

is really a matter of properly sizing and matching your components so

that they work  well together, and using sound design and construction

practices which minimize losses. Some key areas include:

1. Tank Cap - should be LDPE dielectric, with solidly connected

terminations that can efficiently handle the high tank currents. This is

typically the weakest link in most systems. Good performing high voltage

pulse capacitors are hard to find. They can be made using LDPE sheeting

or film, however, and they MUST be run under oil!!

2. Primary Coil - should be heavy gauge solid wire or copper tubing to

reduce skin-effect losses.

3. Spark Gap - must be able to efficiently handle the power, and should

have sufficient air-flow so that the arc is extinguished (quenched) once

all the primary energy has been transferred to the secondary. This is

typically the 2nd weakest link in most systems.

4. Secondary coil and base ground - must be of the proper size and

aspect ratio for the anticipated power level. Must have the base tied to

a solid RF ground, since substantial RF currents will be coming out of

the base. Grounding systems can be a "hidden" lossy area.

5. Toroid - must be sized to hold off breakout until almost the peak

voltage is reached in order to maximize the ability of the coil to

"punch" a conductive channel throught the air. The whole system must be

tuned so that the primary and "loaded" secondary are set to the same

operating frequency.

6. Input Power Source - The input power source must be capable of

rapidly recharging the tank cap(s between discharges, withstanding high

voltage RF transients, and capable of safely being short-circuited. Neon

sign transformers or ballasted plate or distribution transformers are

generally used. Ignition coils are good only for very small coils.

In the final analysis, Richard, there's really no substitute for raw

power! Even the best efficiently-made small coil can safely process a

limited amount of power, and will never attain the lightning-like, white

hot snapping discharges of a coil running at 5 or 10 times the power

level. Focus on reducing the losses in your smaller system, and learn

what works... generally this will be the "hard way", like the rest of us

learned. Then you can successfully, and (and more wisely) progress to

larger, hotter systems.

Safe coilin' to you, Richard!

-- Bert H --

Tesla-2 wrote:

>

> From: Richard <esmit@ilink.nis.za>

> Date: Fri, 27 Jun 1997 07:53:53 +0200 (SAT)

> Subject: Alignment of Caps

>

> Hi,

>

> I only have two questions today that is bugging me.

>

> 1. When you want the right voltage for a capacitor you put them in serie and

> when you want the right capacitance  you put them in parallel. Which of

> these to figures is right.

>

> Fig.1

>

>                  ||

>            |-----||-------| Total .005 uF

>            |     ||       |

>      ------|              |---------- TOTAL OUTPUT .0053 uF

>            | ||  || || || |

>            |-||--||-||-||-| Total .0003 uf

>              ||  || || ||

>

> Fig.2 |

>              ||  || || ||   Total .0053 uF

>            |-||--||-||-||-|

>            | ||  || || || |

>            |              |

>   ---------|              |-------------- TOTAL OUTPUT .01 uF

>            | ||  || || || |

>            |-||--||-||-||-| Total .0053 uF

>              ||  || || ||

>

> 2. I have read about a Tesla coils spark that should bet thick, but my Tesla

> coil is giving thin purple arcs, Is it right if it does so or is it wrong?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: plastic capacitor

Date: Fri, 27 Jun 1997 17:19:22 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Fri, 27 Jun 1997 19:19:22 +0200 (SAT)

Subject: RE: plastic capacitor

At 06:37 PM 26-06-97 GMT, you wrote:

>From: "Engle, Daniel (NJAOST)" <DEngle@NJAOST.ML.com>

>Return-Receipt-To: "Engle, Daniel (NJAOST)" <DEngle@NJAOST.ML.com>

>Date: Thu, 26 Jun 1997 14:37:53 -0400

>Subject: RE: plastic capacitor

>

>I think he meant Zip-Loc bags.  Somewhere along the lines of "yellow and

>blue make green!" .  I'm not quite sure I follow you on the cap idea

>though.  Could you explain in a little more detail about putting "them

>in series to handle the voltage.  Do you mean something like a salt

>water capacitor?  Instead of using the glass bottles, use the zip locks?

I want to put 6 mils of plastic between each plate which can handle

aproximately 1000 Volts, therefore I want to put them in series of fifteen

packs so that it is at 15000 Volts. Then what I'm going to do is put some in

parralelle to obtain the right capacitance and I will put the whole plastic

cap in transformer oil.

Hope that helps.

Thanks

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: plastic capacitor

Date: Fri, 27 Jun 1997 17:19:36 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Fri, 27 Jun 1997 19:19:36 +0200 (SAT)

Subject: Re: plastic capacitor

Hi, Mike and all,

>Have you made any progress on your spark gap? This is where

>you will get your biggest increase in performance.

You won't believe what happened to me. I was changing the space between my

nails and switched on my coil, found resonance and I nearly got a 12 inch

spark from my toroid. I then thought I'm going to take out a piece of foil

in my cap(that was what I last did to get that long spark), but when I had

put the foil back I couldn't get that length of spark back.

I'm thinking of changing my gap today, but I have been experimenting with

about 6 pieces of water pipe and put them near each other so that there is 1

mm space. That kind of gap doesn't nearly perform so great as my gap of iron

nail.

Any comment?

The reason I think I can't find the long spark anymore is because I think

the nails are rusting and getting smaller of that sparks.

Thanks

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: plastic capacitor

Date: Fri, 27 Jun 1997 18:36:03 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Fri, 27 Jun 1997 14:36:03 -0400

Subject: Re: plastic capacitor

> From: Richard <esmit@ilink.nis.za>

> Date: Fri, 27 Jun 1997 19:19:36 +0200 (SAT)

> Subject: Re: plastic capacitor

> Hi, Mike and all,

> >Have you made any progress on your spark gap? This is where

> >you will get your biggest increase in performance.

> You won't believe what happened to me. I was changing the space between my

> nails and switched on my coil, found resonance and I nearly got a 12 inch

> spark from my torroid. I then thought I'm going to take out a piece of foil

> in my cap(that was what I last did to get that long spark), but when I had

> put the foil back I couldn't get that length of spark back.

This is typical when you open the gap up.  It allows the cap to build to

higher voltages before discharging.

> I'm thinking of changing my gap today, but I have been experimenting with

> about 6 pieces of water pipe and put them near each other so that there is 1

> mm space. That kind of gap doesn't nearly perform so great as my gap of iron

> nail.

> Any comment?

1mm is not enough space, and 5 gaps isn't really the best, I'd go for 8

gaps with a .2" or so spacing.

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: plastic capacitor

Cc:

Bcc:

X-Attachments:

At 03:46 PM 6/26/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Thu, 26 Jun 1997 17:46:08 +0200 (SAT)

>Subject: plastic capacitor

>

>Hi,

>

>I was wondering if anyone can give me some suggestions.

>

>I have build my paper cap as I told ya. I was just wondering if a person can

>build a cap out of 20 micron plastic bags and put them in series to handle

>the 15 kVAC power. I read it somewhere about Bill Richards that had used zip

>locker bags(whatever that is) and put them in series to handle the voltage.

>He said it has a very high Q factor. If something has a very high Q factor

>your spark will get better and the spark gap will fire faster. Would it work

>if I will do the experiment. I'm ready to try anything as long as I get the

>best sparks that I can get.

Yes this would work and your cap would be less lossy

but 20 micron plastic material is very thin. For a 15 KVAC

transformer you need a cap rated nearly 30KV and this requires

from 60-90 mils of polyethylene. Thats .06 to .09 inch.

You would have to stack a huge quantity of 20 micron plastic

to equal that amount.

Zip lock bags are polyethylene bags that are used to contain food items

to be frozen. They are availble in any grocery. However even

the heavy duty ones are only 2 or 3 mils thick.

I would say your paper cap is probably working fairly well.

Much better than your old glass cap. Stick with it for now

and work on improving your other components.

Have you made any progress on your spark gap? This is where

you will get your biggest increase in performance.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Coil

Date: Fri, 27 Jun 1997 19:17:43 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Kent Vander Velden" <graphix@iastate.edu>

Date: Fri, 27 Jun 1997 14:17:43 -0500

Subject: Coil

Mike,

It has been a long time since the last time we spoke.  Wanted you to know

that I had not given up on the idea of building a coil.  I have done some

experimenting during the few hours that I get on the weekends to return

home over the last couple months.  At the moment I have a setup that is

giving me about 8" strong arcs and lights up nearby florescent tubes.

While I was indeed excited to get that, the length of the arcs does not

sound optimal for a 15kv 60ma compared to what others are getting.  Let me

describe the coil a little and perhaps you will have some advice.

The primary coil is ~12 turns of 1/4" copper tubing parallel to the

secondary with an inside diameter of ~5.5".  The primary is self supporting

and does not have a frame to hold it in place.  The gap between turns is

~1/4".  Best output appears to be to happen with connection to the greatest

length, i.e. last turn.

The secondary is ~400 turns of 21 gauge enameled wire on a 3.5" diameter

PVC pipe.  The PVC pipe was varnished before (inside and outside) and after

wrapping.  There is ~2' of extra wire coming out of the top of the coil and

~ 1' out the bottom for spare.  The last two turns are space ~1/2" apart.

The coil was wound on a lathe with  only the enamel as spacing between

turns.

The capacitor is a 1 gallon plastic bucket inside of a 3 gallon plastic

bucket.  The buckets are filled with salt (epson salt) water and have a

thin (<1/4") layer of oil on the surface.  Electrodes into the water are

copper ground wires.  I believe the capacitance meter said it was .001 uf

(may not have this number correct).

The spark gap is a multi series spark gap similar to what you had.  A

3.5" PVC pipe with a series of 1/2" copper tube pieces (six I think)

mounted along the inside.  The distance between each is not precise and

varies between ~1/8" to ~1/32".  The variation may be greater than this but

you should get the idea that the spacing is not exact.  Air is pushed

through the spark gap with a 2hp shop vacuum.

The safety gap is a couple of carriage bolts mounted ~3/4" apart and

mounted on L iron.

The discharge terminal is a brass sphere (large door knob?) with maximum

diameter of ~3.5" with a connecting rod on the bottom.  the connecting rod

is made of brass and is part of the sphere.  The diameter of the connecting

rod is ~1".

The main transformer is a 15kv 60ma neon.

All connections except the RF ground are 15kv rated high voltage wire.

The RF ground wire is 12 gauge stranded.  The RF ground itself is a ~2'

copper tube pounded into the ground with ~2" above ground.  Connections to

the primary include a solider connection and a large batter alligator

clamp.

Originally I had ~ 8 turns of 12 gauge solid copper on a _black_ 6" PVC

drain pipe and everything else the same.  The leakage to the pipe could be

easily seen as the area around some of the turns of the primary would glow

blow.  Grounding was done to the shop's conduit.  With this setup no arcs

would happen into the air without encouragement from a grounded object and

several discharges happened inside of the secondary and from the secondary

to the primary.  The secondary coil had to be raised out of the primary

almost all the way to prevent arcing.

I want to emphasize the arcs that occurred inside of the secondary.  They

actually burned carbonized tracks inside of the tube.  This was first

noticed after removing the cap on top of the secondary to find smoke

rolling out.  I scraped the tracks out best as I could but it still made be

concerned that the secondary was damaged.  I now see an occasional arc

(looks like a hand, i.e. arcs start at one point and radiates to several)

on the surface of the secondary but nothing as severe as before.

The new ground system did not seem to help much but the new primary

helped a great deal.  The original spark gap which was a fixed length spark

similar in construction to the safety gap but used in an air blast

configuration.  The air was supplied by a large air compressor.  This gap

appeared to give stronger performance but has not been tried with the

current setup as my project was moved out of our large shop and into a semi

trailer that does not have supply from the compressor.

I am now working on a new primary that will be angle out from the coil at

a 30 degree angle off of the vertical and have 16 turns.  I hope this will

lower the times the likelihood of a secondary-primary arc, increase power

from the secondary and allow better tuning.  The new primary will be

mounted on six pieces of 1/2" PVC mounted on a board.

The spacing in the spark gap could also be made a bit more consistent.  I

also want to add a cylinder inside of the spark gap to encourage the air to

travel more along the actual gaps instead of down the center between the

actual gaps.

The capacitor could use a lot of improvement most likely as it is

certainly the crudest part of the system.

I have also found that treated plywood does not make for a good insulator

and thus not a good mount for the spark gap or safely gap.  The wood

eventually gets a carbon track formed between the plys of wood and shorts

the system out.  Pretty scary the first time it happens as you think the

neon has just died.  A better mounting material from what I have now would

be nice as well.  Currently I use lids of auto body putty cans but

something with a greater weight and just as good of an isolator would be

great.

Well there you have it.  Mostly made out of parts that I had.  The copper

tubing and salt I bought but the rest was scrap.  This coil was for proof

of concept before investing time or money in material for a better coil.

any suggestions that you can make for improvement would be appreciated.

BTW: The variac has _not_ been ordered yet.

Thanks.

----------

Kent Vander Velden

graphix@iastate.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: plastic capacitor

Cc:

Bcc:

X-Attachments:

At 06:37 PM 6/26/97 GMT, you wrote:

>From: "Engle, Daniel (NJAOST)" <DEngle@NJAOST.ML.com>

>Return-Receipt-To: "Engle, Daniel (NJAOST)" <DEngle@NJAOST.ML.com>

>Date: Thu, 26 Jun 1997 14:37:53 -0400

>Subject: RE: plastic capacitor

>

>I think he meant Zip-Loc bags.  Somewhere along the lines of "yellow and

>blue make green!" .  I'm not quite sure I follow you on the cap idea

>though.  Could you explain in a little more detail about putting "them

>in series to handle the voltage.  Do you mean something like a salt

>water capacitor?  Instead of using the glass bottles, use the zip locks?

>

>

>Curious minds want to know! :-)

>

>Dan Engle

Dan,

There are two methods used to get a high voltage rating for a cap.

One is to use thick layers of material with a high voltage rating.

Unfortunately the best materials for low loss pulse capable caps

(polyethylene and polypropylene) have rather low voltage ratings.

So this requires a thick layer or many thinner layers stacked up

to get the desired rating.

The second method is to wire low voltage caps in series.

In this method you make identical high value caps with thin dielectric

material and wire a sufficient number of them in series to achieve the

desired voltage rating. This is a much preferred method as the stress

is spread out across many dielectric layers.

My first plate cap was made this second way. I made 6 caps

using 2 sheets of 4 mil poly for each dielectric. Each cap

had 24 plates of aluminum foil. The plates measured 6 X 6 inches

with an extended tab for interconnections.

Each cap measured .048 mfd and is good for about 3000 volts AC.

Wiring 6 of them in series yields a total capacity of .008 mfd

and an 18000 volt AC rating.

This cap has worked quite well with 12kv neons at up to 1500 watts.

Best output was 36 inches from a 4 inch diameter secondary.

The idea of using poly bags for cap construction isnt new or novel.

The basic idea is to place of square of foil inside the bag

and stack up a number of bags to get the desired capacitance.

Then wire up enough of these stacks to get the necessary voltage rating.

Lots of work and lots of bags.

I also thought once about using those polypropylene sheet protectors

that go into a 3 ring notebook. They are inexpensive but it

would take quite a few to make a workable cap.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary grounding

Cc:

Bcc:

X-Attachments:

At 07:01 PM 6/25/97 GMT, you wrote:

>From: Shane Randle <randlec@scnc.addison.k12.mi.us>

>Date: Wed, 25 Jun 1997 15:01:35 -0400

>Subject: Secondary grounding

>

>I am just beginning to build my secondary coil, and I was wondering how

>the coil should be grounded. I've never done this before, and I was just

>wondering how you guys do it. Thanks in advance for any help you can

>give me.

>-Shane M. Randle

>

Well like the old saying goes there's lots of ways to skin

that cat.

My preferred method is one popularized by Richard Quick.

In this method a copper or brass plate is epoxied to the outside

wall of the secondary form below the secondary winding.

The bottom end of the secondary winding is then soldered to this plate.

Then your ground lead can be attached to the plate.

I have held ground straps on with a couple turns of tape

and with a large rubber band but now I solder the ground

lead directly to the plate.

I have a ground wire about two foot long coming from the secondary

ground plate terminated in a large ring terminal.

The ground lead from my RF ground system also terminates in the

same size ring terminal. When I set the coil up I bolt together

the two ground leads with a 1/2 inch nut and bolt and washers.

Ground consists of two 4 foot ground rods through the basement floor.

Total ground lead length is 6 feet.

All ground leads are AWG 8.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: plastic capacitor

Cc:

Bcc:

X-Attachments:

At 05:19 PM 6/27/97 GMT, you wrote:

>From: Richard <esmit@ilink.nis.za>

>Date: Fri, 27 Jun 1997 19:19:22 +0200 (SAT)

>Subject: RE: plastic capacitor

>

>At 06:37 PM 26-06-97 GMT, you wrote:

>>From: "Engle, Daniel (NJAOST)" <DEngle@NJAOST.ML.com>

>>Return-Receipt-To: "Engle, Daniel (NJAOST)" <DEngle@NJAOST.ML.com>

>>Date: Thu, 26 Jun 1997 14:37:53 -0400

>>Subject: RE: plastic capacitor

>>

>>I think he meant Zip-Loc bags.  Somewhere along the lines of "yellow and

>>blue make green!" .  I'm not quite sure I follow you on the cap idea

>>though.  Could you explain in a little more detail about putting "them

>>in series to handle the voltage.  Do you mean something like a salt

>>water capacitor?  Instead of using the glass bottles, use the zip locks?

>

>I want to put 6 mils of plastic between each plate which can handle

>aproximately 1000 Volts, therefore I want to put them in series of fifteen

>packs so that it is at 15000 Volts. Then what I'm going to do is put some in

>parallel to obtain the right capacitance and I will put the whole plastic

>cap in transformer oil.

>

>Hope that helps.

>Thanks

My experience is that polyethylene will handle

more voltage than 1000 volts for 6 mils.

Before I built my first plate cap I built a number

of test caps. These caps used aluminum foil and 4 mil

poly with either a single or double layer of 4 mil poly.

I fired these test caps dry with no oil on my 3 inch

coil and in static tests..

Through a number of tests to determine just how far I could

go before blowing these test caps I found that 4 mils of poly

would easily handle 1500 volts AC. I made several caps using 8 mils

of poly and found it would handle 3000 VAC and at 3500 volts it was

still holding.

I ran a couple of these 8 mil caps on my coil at about 3500 volts

and got several inches of spark output. I proceeded to increase the

voltage on each of these in turn to see at what voltage they would blow up.

Each one handled 3500 volts fine. One blew at 3900 and both the others

blew at slightly over 4000.

So I felt confident that my 8 mil poly caps would be good for 3000

volts with some safety margin. That comes to about 375 volts per mil.

Using that figure your 6 mils should be good for 2250 volts with room to

spare.

Remember that these test I made were done with dry caps. No oil.

Adding oil would suppress corona and most likely improve the voltage

rating by excluding air.

TTYL!

To: tesla-2@emachine.com

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Coil

Cc:

Bcc:

X-Attachments:

At 02:17 PM 6/27/97 -0500, you wrote:

>From: "Kent Vander Velden" <graphix@iastate.edu>

>To: <mhammer@misslink.net>

>Cc: <tesla-2@emachine.com>

>Subject: Coil

>Date: Fri, 27 Jun 1997 14:17:43 -0500

>X-MSMail-Priority: Normal

>

>Mike,

>

>  It has been a long time since the last time we spoke.  Wanted you to know

>that I had not given up on the idea of building a coil.  I have done some

>experimenting during the few hours that I get on the weekends to return

>home over the last couple months.  At the moment I have a setup that is

>giving me about 8" strong arcs and lights up nearby florescent tubes.

>While I was indeed excited to get that, the length of the arcs does not

>sound optimal for a 15kv 60ma compared to what others are getting.  Let me

>describe the coil a little and perhaps you will have some advise.

Good to hear from you. I was wondering what became of you and was

considering sending you an email just to say hello.

8 inches is a good start but you should be able to get triple

that amount.

>

>  The primary coil is ~12 turns of 1/4" copper tubing parallel to the

>secondary with an inside diameter of ~5.5".  The primary is self supporting

>and does not have a frame to hold it in place.  The gap between turns is

>~1/4".  Best output appears to be to happen with connection to the greatest

>length, i.e. last turn.

Primary sounds good. Your last statement tells me your primary

resonant frequency is too high. Your secondary is resonant at a lower

frequency than your primary circuit. You either need more primary turns

or a larger main cap. I would guess you need more cap.

>

>The secondary is ~400 turns of 21 gauge enameled wire on a 3.5" diameter

>PVC pipe.  The PVC pipe was varnished before (inside and outside) and after

>wrapping.  There is ~2' of extra wire coming out of the top of the coil and

>~ 1' out the bottom for spare.  The last two turns are space ~1/2" apart.

>The coil was wound on a lathe with  only the enamel as spacing between

>turns.

Ok this is OK. More turns of a smaller guage wire would have

been better for this size secondary. AWG 24 would be great.

How much of that AWG 21 do you have left. Want to trade some

of your 21 for some AWG 24?

>

>The capacitor is a 1 gallon plastic bucket inside of a 3 gallon plastic

>bucket.  The buckets are filled with salt (epson salt) water and have a

>thin (<1/4") layer of oil on the surface.  Electrodes into the water are

>copper ground wires.  I believe the capacitance meter said it was .001 uf

>(may not have this number correct).

Yea here is a problem. .001 mfd is way small. You should be

thinking in terms of .01 mfd.

<snip>

>  I want to emphasize the arcs that occurred inside of the secondary.  They

>actually burned carbonized tracks inside of the tube.  This was first

>noticed after removing the cap on top of the secondary to find smoke

>rolling out.  I scraped the tracks out best as I could but it still made be

>concerned that the secondary was damaged.  I now see an occasional arc

>(looks like a hand, i.e. arcs start at one point and radiates to several)

>on the surface of the secondary but nothing as severe as before.

Oh oh. Internal arcing in the secondary. Did you epoxy

a disk of plexiglass or Lexan inside the form like I showed you?

Is any part of your secondary wiring inside the form?

If you have carbon tracking you may need to make a new secondary.

Try running a drum sander up and down inside the form to scour

away the carbon tracking then reapply a couple coats of polyurethane

varnish.Then epoxy a couple disks of plexiglass or lexan inside the

form to prevent futute arcing.

>

>  The new ground system did not seem to help much but the new primary

>helped a great deal.  The original spark gap which was a fixed length spark

>similar in construction to the safety gap but used in an air blast

>configuration.  The air was supplied by a large air compressor.  This gap

>appeared to give stronger performance but has not been tried with the

>current setup as my project was moved out of our large shop and into a semi

>trailer that does not have supply from the compressor.

>

>  I am now working on a new primary that will be angle out from the coil at

>a 30 degree angle off of the vertical and have 16 turns.  I hope this will

>lower the times the likelihood of a secondary-primary arc, increase power

>from the secondary and allow better tuning.  The new primary will be

>mounted on six pieces of 1/2" PVC mounted on a board.

This sounds good. Some recent tests have suggested that a primary

with a 15 degree slope may work better.

>

>  The spacing in the spark gap could also be made a bit more consistent.  I

>also want to add a cylinder inside of the spark gap to encourage the air to

>travel more along the actual gaps instead of down the center between the

>actual gaps.

Agreed. Plus I think you need a few more electrodes.

You may need as many as 10 for a 15kv transformer. Space them

.025 inch apart.

>

>  The capacitor could use a lot of improvement most likely as it is

>certainly the crudest part of the system.

No arguement there.

Looks like you need to work on your secondary to remove the carbon

tracks and maybe install some plexiglass baffles to prevent future

arcing.

Rework your gap with a few more electrodes and consistant spacing.

New capacitor needed with more capacitance.

Your new proposed primary sounds good and will help.

Keep me posted and feel free to email me anytime

with any further questions.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Coax

Date: Sun, 29 Jun 1997 13:40:41 GMT

Organization: The Energy Machine Information System 504-734-0526

From: aasphalt@win.bright.net (Lynn Anderson)

Date: Sun, 29 Jun 1997 08:40:41 -0500 (CDT)

Subject: Coax

I have located a large coil of aluminum sheathed,1/2" dia. coax. cable.Will

this be appropriate to use for a primary for a 15-60 neon transformer Tesla

coil?If so I shall buy the entire 2000' roll and have a bit left over to

sell at a reasonable price.Thanks!

Get your "Tars and Stripes" from Anderson Asphalt!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Alignment of Caps

Date: Sun, 29 Jun 1997 15:53:24 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sun, 29 Jun 1997 11:53:24 -0400

Subject: Re: Alignment of Caps

> From: Bert Hickman <bert.hickman@aquila.com>

> Date: Fri, 27 Jun 1997 07:48:53 -0700

> Subject: Re: Alignment of Caps

> Organization: Stoneridge Engineering

>

> Richard,

>

> Question 1:

>

Super-Humongous SNIP to conserve bandwidth...

> -- Bert H --

>

Bert,

Another EXCELLENT post! I have gotten into the habit of clipping out

all the pieces of items that appear in the Tesla and Tesla-2 and HV

posts (and their archives) that are particularly interesting to me,

and I keep these in my own little set of archives here in my

computer. So far I have about 120 megs worth of the stuff. It is

always a pleasure to plop some more items by people like Bert Hickman

and Mike Hammer and Malcolm Watts into my little hoard of knowledge.

I am pleased to see more and more individuals like Richard Smit

asking questions and sharing on these lists. Thanks, Bert and Mike,

for your on-going commitment to helping beginners on this Tesla-2

list, and thanks to all you "newbies" and "wannabees" that keep this

list alive with your questions.

I wish that I could contribute more myself this summer, but where I

am at my summer assignment I have not yet been able to get on-line.

I'm still working on it, though. In the meantime I will take

advantage of days like today when I happen to be back in Paterson for

a while, and at least make a quick run through my e-mail a) so that

my poor mail box does not overflow, and b) so that I don't miss

anything interesting.

The really big bummer is that for the summer I will not be able to do

much in the way of experimentation, which is what I love the most

about coiling!

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Coax

Date: Sun, 29 Jun 1997 20:45:54 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 29 Jun 1997 13:45:54 -0700

Subject: Re: Coax

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: aasphalt@win.bright.net (Lynn Anderson)

> Date: Sun, 29 Jun 1997 08:40:41 -0500 (CDT)

> Subject: Coax

>

> I have located a large coil of aluminum sheathed,1/2" dia. coax. cable.Will

> this be appropriate to use for a primary for a 15-60 neon transformer Tesla

> coil?If so I shall buy the entire 2000' roll and have a bit left over to

> sell at a reasonable price.Thanks!

>

>            Get your "Tars and Stripes" from Anderson Asphalt!

>

Lynn,

If this is "hard line" or semi-rigid coac, it would work fine in a

primary circuit, and could handle considerably more than the power from

a 15 KV 60 MA trannie. I assume this also has an outer vinyl/plastic

layer that would need to be removed (at least where you want to tap

it?). Compare it to 0.5" copper tubing for a cost comparison... 2000' is

a LOT of cable - hope you have lots of space! A typical medium sized

primary uses only 60 - 120 feet of material...

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: HV List

Date: Sun, 29 Jun 1997 22:32:09 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Sun, 29 Jun 1997 18:32:09 -0400

Subject: HV List

Tesla-2 wrote:

>

> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> Date: Sun, 29 Jun 1997 11:53:24 -0400

> Subject: Re: Alignment of Caps

>

> > From: Bert Hickman <bert.hickman@aquila.com>

> > Date: Fri, 27 Jun 1997 07:48:53 -0700

> > Subject: Re: Alignment of Caps

> > Organization: Stoneridge Engineering

> >

> > Richard,

> >

> > Question 1:

> >

>

> Super-Humongous SNIP to conserve bandwidth...

>

> > -- Bert H --

> >

>

> Bert,

> Another EXCELLENT post! I have gotten into the habit of clipping out

> all the pieces of items that appear in the Tesla and Tesla-2 and HV

> posts (and their archives) that are particularly interesting to me,

How do I go about getting on the HV list?

Thanks,

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: HV List

Date: Mon, 30 Jun 1997 02:10:23 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 29 Jun 1997 19:10:23 -0700

Subject: Re: HV List

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: msnock@theonramp.net (Micheal Huff)

> Date: Sun, 29 Jun 1997 18:32:09 -0400

> Subject: HV List

>

> Tesla-2 wrote:

> >

> > From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> > Date: Sun, 29 Jun 1997 11:53:24 -0400

> > Subject: Re: Alignment of Caps

> >

> > > From: Bert Hickman <bert.hickman@aquila.com>

> > > Date: Fri, 27 Jun 1997 07:48:53 -0700

> > > Subject: Re: Alignment of Caps

> > > Organization: Stoneridge Engineering

> > >

> > > Richard,

> > >

> > > Question 1:

> > >

> >

> > Super-Humongous SNIP to conserve bandwidth...

> >

> > > -- Bert H --

> > >

> >

> > Bert,

> > Another EXCELLENT post! I have gotten into the habit of clipping out

> > all the pieces of items that appear in the Tesla and Tesla-2 and HV

> > posts (and their archives) that are particularly interesting to me,

>

> How do I go about getting on the HV list?

>

> Thanks,

> Mike

> --

>

Mike,

Send an E-Mail to Steve Roys at hvadmin@anchorage.ab.umd.edu telling him

a little bit about yourself, areas of HV work/investigation you're

doing, and request to be put on the list. That's it!

Safe hi-voltin' to ya!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: NEW TOROID

Date: Mon, 30 Jun 1997 17:15:55 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard <esmit@ilink.nis.za>

Date: Mon, 30 Jun 1997 19:15:55 +0200 (SAT)

Subject: NEW TOROID

Hi,

Well, today I have constructed my new toroid in about 45 minutes.

I will give you my plans if anyone want to make their own torroids cheaply.

I have taken to pie pans and placed them against each other so they look

like this >--<. Then I have taken 8 toilet paper rolls(the cardboard tube

that is holding the paper together. I took them and cut a slit into them so

that they look like this letter C. The paper rolls were bent over the sides

of this > in the picture of this >--<. I took some sort of glue that is like

water and took some strips of aluminum foil about .5 inch in width and 12

inches long. This pieces of foil I glued around the toilet rolls, after that

I put some sticky paper on the sharp edges. In the middle of the two pans I

put a piece of metal from pie pans and connected them in the middle so that

it holds the lose pieces of foil in place. I tested it and I could see there

is much less corona losses on my torroid, so tomorrow I am going to varnish

my toroid to insulate it better and see what the results are.

Try it, seems to work for me.

Happy coiling.

PS! if any of this is unclear, please let me know and I'll draw a picture

for you and give each step.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: insulated Toroid

Date: Mon, 30 Jun 1997 20:06:29 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Mon, 30 Jun 1997 13:06:29 -0700

Subject: insulated Toroid

Has anyone experimented in varnishing portions of the toroid? What

would happen if you varnished the bottom half?

Will this change the XsubC apparent in the secondary? Will it prevent

or supress arcs to the primary? If anyone has experimented in odd

shaped terminals let me know. I'm interested. I saw a post here about

making a toroid out of toilet paper tubes and it struck me as odd. I

made mine out of a styrofoam wreath form. I sprayed it with Elmer's

spray glue and rubbed on triangle shaped strips of Aluminum foil.

after it dried it looked great and has no sharp points or edges.

(inside the form on the inside rim I pressed in a piece of bare copper

wire to make sure all points had a good connection.)

(that wire is the connection to the top of the primary, which is 866

turns of #26 polyimide coated magnet wire. this wire is very

manageable, but quite expensive. I found mine at a surplus store. I

got 2500 feet of it for about 17 dollars US.)

Also- How do I make a saltwater capacitor? I want one in the interim

until my poly rolled cap is finished...

I found clear plastic at Target. It's intended as a furniture cover. I

can get it for about a dollar per linear foot, and the roll is 10 feet

wide. I haven't miked it out yet but it looks about 10 mils thick.

Where do people get the 60 mil stuff? By comparison, I want to know

how thick other sources of rolled plastic are, and where I can get

them.

Thanks in advance, Mike.....

Nathan

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: insulated Toroid -Reply

Date: Mon, 30 Jun 1997 20:27:05 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Kevin Batchelor <KBATCHELOR@arslimited.com>

Date: Mon, 30 Jun 1997 16:27:05 -0400

Subject: insulated Toroid -Reply

Nathan:

Instead of the furniture cover poly...try the paint department at your local

hardware store...try Home Depot.  You can pickup a roll of 6mil for about

$5.  If I recall correctly...mine was either 25' or 50' by 10' wide.  It works

well...you'll just have to use several layers.

If you haven't started your cap...you might want to use a flat plate cap.

Mike tells me that they pulse better and thus perform better in the Tesla

Coil use.  Additionally, you can add or remove plates to change the

capacitance.

Kevin

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: insulated Toroid

Date: Mon, 30 Jun 1997 20:58:52 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Mon, 30 Jun 1997 16:58:52 -0400

Subject: Re: insulated Toroid

Tesla-2 wrote:

>

> From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

> Date: Mon, 30 Jun 1997 13:06:29 -0700

> Subject: insulated Toroid

>

>      Has anyone experimented in varnishing portions of the toroid? What

>      would happen if you varnished the bottom half?

>      Will this change the XsubC apparent in the secondary? Will it prevent

>      or supress arcs to the primary?

If the sparks can go several feet (or inches) in the air then that small

amount of varnish or sealer isn't going to do much to stop it,

especially at high frequencies.

>      If anyone has experimented in odd

>      shaped terminals let me know. I'm interested. I saw a post here about

>      making a toroid out of toilet paper tubes and it struck me as odd. I

>      made mine out of a styrofoam wreath form. I sprayed it with Elmer's

>      spray glue and rubbed on triangle shaped strips of Aluminum foil.

>      after it dried it looked great and has no sharp points or edges.

>      (inside the form on the inside rim I pressed in a piece of bare copper

>      wire to make sure all points had a good connection.)

>      (that wire is the connection to the top of the primary, which is 866

>      turns of #26 polyimide coated magnet wire. this wire is very

>      manageable, but quite expensive. I found mine at a surplus store. I

>      got 2500 feet of it for about 17 dollars US.)

I wouldn't spend that much on about a pound.  Standard enameled wire

works great and I only pay $4 a pound at a local motor repair shop.

>      Also- How do I make a saltwater capacitor? I want one in the interim

>      until my poly rolled cap is finished...

See:  http://www.geocities.com/CapeCanaveral/Lab/5322/bottle.htm

They are very cost effective (but lossy) and run about 700-800pF per

bottle.  An alternative might be some doorknob caps.  You can get 35kV

3000-4000pF caps relatively cheap.

>      I found clear plastic at Target. It's intended as a furniture cover. I

>      can get it for about a dollar per linear foot, and the roll is 10 feet

>      wide. I haven't miked it out yet but it looks about 10 mils thick.

>      Where do people get the 60 mil stuff? By comparison, I want to know

>      how thick other sources of rolled plastic are, and where I can get

>      them.

>

>      Thanks in advance, Mike.....

Your welcome (although I know you mean the other Mike :] ).  10mil

sheets can be stacked for 60mil thickness.  I have used 6 and 4mil

polyetylene drop cloth for caps.

Would you mind running down the specs on your coil?

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: NEW TOROID

Date: Mon, 30 Jun 1997 21:24:17 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Mon, 30 Jun 1997 17:24:17 -0400

Subject: Re: NEW TOROID

>

> From: Richard <esmit@ilink.nis.za>

> Date: Mon, 30 Jun 1997 19:15:55 +0200 (SAT)

> Subject: NEW TOROID

>

> Hi,

>

> Well, today I have constructed my new toroid in about 45 minutes.

>

> I will give you my plans if anyone want to make their own torroids cheaply.

>

> I have taken to pie pans and placed them against each other so they look

> like this >--<. Then I have taken 8 toilet paper rolls(the cardboard tube

> that is holding the paper together. I took them and cut a slit into them so

> that they look like this letter C. The paper rolls were bent over the sides

> of this > in the picture of this >--<. I took some sort of glue that is like

> water and took some strips of aluminum foil about .5 inch in width and 12

> inches long. This pieces of foil I glued around the toilet rolls, after that

> I put some sticky paper on the sharp edges. In the middle of the two pans I

> put a piece of metal from pie pans and connected them in the middle so that

> it holds the lose pieces of foil in place. I tested it and I could see there

> is much less corona losses on my torroid, so tomorrow I am going to varnish

> my toroid to insulate it better and see what the results are.

Good job at finding suitable household trash to do the job.  As far as

the varnishing goes, I hope you are talking about the secondary coil and

not the toroid.  No need or point to put varnish on the output

terminal.  You don't want to insulate your output terminal!

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: insulated Toroid

Date: Mon, 30 Jun 1997 22:52:36 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 30 Jun 1997 18:52:36 -0400

Subject: Re: insulated Toroid

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: insulated Toroid

> Date: Monday, June 30, 1997 4:06 PM

>

> From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

> Date: Mon, 30 Jun 1997 13:06:29 -0700

> Subject: insulated Toroid

>

>      Has anyone experimented in varnishing portions of the toroid?

>      What would happen if you varnished the bottom half?

>      Will this change the XsubC apparent in the secondary? Will it

>      prevent or supress arcs to the primary? If anyone has experimented in

>      odd  shaped terminals let me know. I'm interested. I saw a post

>      here about  making a toroid out of toilet paper tubes and it struck me as

>      odd. I made mine out of a styrofoam wreath form. I sprayed it withElmer's

>      spray glue and rubbed on triangle shaped strips of Aluminum foil.

>      after it dried it looked great and has no sharp points or edges.

>      (inside the form on the inside rim I pressed in a piece of bare copper

>      wire to make sure all points had a good connection.)

>      (that wire is the connection to the top of the primary, which is 866

>      turns of #26 polyimide coated magnet wire. this wire is very

>      manageable, but quite expensive. I found mine at a surplus store. I

>      got 2500 feet of it for about 17 dollars US.)

>

>

>      Also- How do I make a saltwater capacitor? I want one in the interim

>      until my poly rolled cap is finished...

>

>

>      I found clear plastic at Target. It's intended as a furniture cover. I

>      can get it for about a dollar per linear foot, and the roll is 10 feet

>      wide. I haven't miked it out yet but it looks about 10 mils thick.

>      Where do people get the 60 mil stuff? By comparison, I want to know

>      how thick other sources of rolled plastic are, and where I can get

>      them.

>

>

>      Thanks in advance, Mike.....

>

>

>      Nathan

>

Nathan,

Make sure that clear plastic stuff is polyethelene and not vinyl.

Vinyl is too lossy and heats up so fast the stuff will melt on you!!

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: HV List

Cc:

Bcc:

X-Attachments:

At 10:32 PM 6/29/97 GMT, you wrote:

>From: msnock@theonramp.net (Micheal Huff)

>Date: Sun, 29 Jun 1997 18:32:09 -0400

>Subject: HV List

>

>How do I go about getting on the HV list?

>

>Thanks,

>Mike

Thanks for reminding me. I forgot I was supposed

to find that out for you.

To all that are interested in Steve Roys HV list

send a request for info to: hvadmin@anchorage.ab.umd.edu

This list covers general HV interests but not

normally Tesla coils.

TTYL!

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Coax

Cc:

Bcc:

X-Attachments:

At 01:40 PM 6/29/97 GMT, you wrote:

>From: aasphalt@win.bright.net (Lynn Anderson)

>Date: Sun, 29 Jun 1997 08:40:41 -0500 (CDT)

>Subject: Coax

>

>I have located a large coil of aluminum sheathed,1/2" dia. coax. cable.Will

>this be appropriate to use for a primary for a 15-60 neon transformer Tesla

>coil?If so I shall buy the entire 2000' roll and have a bit left over to

>sell at a reasonable price.Thanks!

>

Lynn,

My personal opinion is that unless that coax is dirt

cheap then forget it and go for 1/4 inch copper tubing.

1/4 inch copper will handle your 15/60 neon easily

and be good for several thousand watts.

If you can get the coax very cheap then go ahead and use it.

It will make a very large primary coil that should work well

but would really be overkill for a neon driven system.

My own primary coil is wound with solid AWG 6 copper

and performs very well at up to 1500 watts.

I have run a primary made from solid AWG 12 house wire

up to 720 watts with no problem.

TTYL!