.onlineHelp ENVISION Xprism "Input of Files (Data Objects)"

.section 2 "Input of Files Containing Data Objects"

Xprism can read in any data file supported by Khoros 2.0.  Data files may have

2D or 3D data that may be stored \fIimplicitly\fP or \fIexplicitly\fP.

The size of the data set may be determined by \fIwidth\fP, \fIheight\fP, or

both.  In the following descriptions, when either width or height can be

used to indicate the size of the data set, height has been used.

.section 3 "Implicit 2D data"

For files containing implicit 2D plot information, the value segment is

expected to contain the Y values of the data points. The X coordinates are

implied from the position of the Y coordinates in the value segment.   Thus,

data sets stored in this fashion are restricted to having incremental X values

beginning at 0.

.PSPIC $ENVISION/manual/Envision/E.9.xprism/figures/2d_impl.eps

.section 3 "Explicit 2D data"

Like implicit 2D data, files containing explicit 2D plot information, also

have the Y values stored in the value segment.   The location information,

however, is explicitly given in the location segment.  This makes it possible

to specify X coordinates that are not incremental.

.PSPIC $ENVISION/manual/Envision/E.9.xprism/figures/2d_expl.eps

.section 3 "Implicit 3D data"

For files containing implicit 3D plot information, the value segment is

expected to contain the Z values (elevation) of the data points.  The D

values (imagery), are obtained by normalizing the Z values.  The X and Y

values are implied by the size of the data itself. X is considered to be

incremental integer values beginning at 0 and ranging to the width of the

value segment; Y is considered to be incremental integer values beginning

at 0 and ranging to the height of the value segment.

.PSPIC $ENVISION/manual/Envision/E.9.xprism/figures/3d_impl.eps

.section 3 "Explicit 3D data"

For files containing explicit 3D plot information, the D values (imagery)

are obtained by normalizing the values in the value segment.  Where X, Y, and

Z values are stored is determined by the dimension.

.paragraph

If the \fIdimension of the data is 1\fP, Z values are stored in the location

segment; X and Y values are determined from the width and the height of the

value segment, as in the implicit 3D data case.  The most common application

of this case is for GIS systems, where the elevation data is stored in the

location segment and the D values (the image) are stored in the value segment.

.PSPIC $ENVISION/manual/Envision/E.9.xprism/figures/3d_expl_d1.eps

.paragraph

If the \fIdimension of the data is 2\fP, X and Y values are stored in the

location segment; the Z values are stored in the value segment, and the

D values are obtained by normalizing the Z values.  Note that there can be

no row/column information implied by this storage method;  thus, it cannot

be used for visualization of line plots, mesh plots, surface plots, or any

other plots that require the number of rows to be specified.  It is appropriate

\fIonly\fP for 3D scatter plots.

.PSPIC $ENVISION/manual/Envision/E.9.xprism/figures/3d_expl_d2.eps

.paragraph

If the \fIdimension of the data is 3\fP, X, Y, and Z values are all stored in

the location segment.  As in the other cases, the D values are still found

in the value segment.  Like the previous storage method with dimension = 2,

there is no row/column information implied by this storage method.  Thus,

it can only be used with 3D scatter plots.

.PSPIC $ENVISION/manual/Envision/E.9.xprism/figures/3d_expl_d3.eps