P C - E C A P

Version 2.10

Program Reference Manual

Copyright (c) 1990-92 by Peter Volpa

All Rights Reserved

_______

____|__     |                (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

Peter Volpa

Circuit Systems

418 Church Road

Sicklerville, NJ 08081-1727  USA

Table of Contents

-----------------

Introduction . . . . . . . . . . . . . . . . . . . . .  1

License  . . . . . . . . . . . . . . . . . . . . .  2

Ordering Information . . . . . . . . . . . . . . .  3

(also registering by credit card)

Disclaimer, Trademarks, ASP info . . . . . . . . .  4

System Requirements  . . . . . . . . . . . . . . .  5

Installing PC-ECAP . . . . . . . . . . . . . . . .  6

Quick Start  . . . . . . . . . . . . . . . . . . . . .  7

Miscellaneous program information :

Printer configuration  . . . . . . . . . . . . . .  8

Data format  . . . . . . . . . . . . . . . . . . .  8

Accessing data files in other directories  . . . .  8

Memory requirements vs. maximum circuit nodes  . .  8

Network / Multi-User operation . . . . . . . . . .  9

Getting Started  . . . . . . . . . . . . . . . . . . .  10

Main Menu Descriptions . . . . . . . . . . . . . . . .  10

Select input file  . . . . . . . . . . . . . . . .  10

Edit input file  . . . . . . . . . . . . . . . . .  11

Analyze circuit  . . . . . . . . . . . . . . . . .  15

View output data . . . . . . . . . . . . . . . . .  16

Output to printer  . . . . . . . . . . . . . . . .  16

Change program settings  . . . . . . . . . . . . .  17

(number of sample points, printer type, etc.)

Quit this program  . . . . . . . . . . . . . . . .  18

Analyzing a Circuit Using PC-ECAP  . . . . . . . . . .  19

Input format . . . . . . . . . . . . . . . . . . .  20

Circuit element description  . . . . . . . . . . .  21

Specifying analysis parameters . . . . . . . . . .  24

(group delay, impedance, vswr, return loss)

An example!  . . . . . . . . . . . . . . . . . . .  26

Another example (using the Model Maker)  . . . . .  27

Models . . . . . . . . . . . . . . . . . . . . . . . .  30

References for modeling  . . . . . . . . . . . . .  31

Revision History . . . . . . . . . . . . . . . . . . .  32

Appendix A - Command Line Switches . . . . . . . . . .  33

Appendix B - Error Messages  . . . . . . . . . . . . .  34

Appendix C - PC-ECAP to Lotus 1-2-3 file conversion  .  38

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 1

Introduction

PC-ECAP is an easy to use AC circuit analysis program which

analyzes circuits consisting of resistors, capacitors,

inductors, transformers, transistors (both bipolar and

FET's), operational amplifiers and transconductance

amplifiers.

Totally integrated in one package, it contains a full screen

text editor for preparing analysis input, an electronic

circuit analyzer that calculates your circuit's frequency

and phase response, a high resolution display plotter that

supports all common graphics adapters and a high resolution

plotter compatible with most printers.

Features:

*    Calculates Magnitude, Phase, Group Delay,

Impedance, VSWR and Return Loss.

*    Completely menu driven - very easy to use.

*    Can analyze circuits with up to 90 nodes,

and with as many as 1000 components.

*    Built in full screen editor with on-line help

function.

*    Free input format - common abbreviations such as

P, U, N, M, K, MEG, G are recognized.  For

example if your circuit has a 47pf capacitor, all

you need to enter for its value is 47pf.

*    Supports CGA, EGA, VGA, Hercules, MCGA, AT&T and

even the Monochrome Display Adapter.

*    All calculations are done in 15 digits of double

precision.

*    High resolution plots on IBM/Epson 9 and 24 pin

compatible printers, and HP LaserJet+.

*    Program may be configured to send its output to

LPT1, LPT2, LPT3 or a disk file.

*    Bipolar transistor model maker.  Just enter your

transistor's operating parameters and PC-ECAP will

create a Hybrid-Pi model for your transistor.

*    Will utilize a math coprocessor if available, but

does NOT require one.

*    Built-in network / multi-user support.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 2

License

PC-ECAP is distributed as "shareware".  It is NOT public

domain or free software.

Non-registered users of this software are given the

opportunity to evaluate the complete PC-ECAP software

package for 30 days with no obligation.  If, after the

evaluation period, you find PC-ECAP to be suitable for your

needs, you must register your copy or discontinue using

PC-ECAP.

What we're saying is if you use this program, then you

should pay for your copy.  That way Circuit Systems will be

able to provide updates, support, and generally stay in

business.

PC-ECAP is distributed in two forms: a registered version

and an unregistered version.  The unregistered version is a

fully-functional program that has all the features of the

registered version.  However, the unregistered version

requires the user to go through an initialization procedure

each time PC-ECAP is started.  The initialization procedure

requires the user to enter a random number before PC-ECAP

will run.  The registered version does not have this

initialization procedure.

The REGISTERED version of PC-ECAP is LICENSED TO THE SINGLE

PERSON WHO REGISTERED THE PROGRAM AND MAY NOT BE DISTRIBUTED

IN ANY MANNER.

All users are granted a limited license to copy the

UNREGISTERED version of PC-ECAP for the trial use of others.

This license does NOT include distribution or copying of

this software:

*    In conjunction with any other product or service.

*    In general use within a company or school.

*    For any "disk fee" other than a small (generally

$2.00 to $5.00) copying charge.

*    For distribution in any modified form.  The file

containing this license information MUST be

included along with full documentation and all

the files that make up this software package.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 3

Ordering Information

A PC-ECAP registration licenses you to use the product on a

regular basis.  Registration also entitles you to technical

support, update notices, and enables you to get future

releases of PC-ECAP at a minimal price.

Individual registrations come in two forms.  The first,

registration only, costs $69 (plus $2.00 shipping and

handling).  Upon receipt of your paid registration form, we

will send you the latest registered version of PC-ECAP with

documentation on the disk.  We also offer PC-ECAP with a

printed manual.  For $79 (plus $4.00 shipping and handling)

we will send you the latest registered version of PC-ECAP

along with a professionally printed bound manual.  New

Jersey residents should include 6% sales tax.

Educational institutions may apply a 35% discount to all

prices.  PLEASE include your educational institution

letterhead (with your title) when you order.

Site licensing is also available.  Prices are based on the

estimated number of users.  Please write for more

information.

A PC-ECAP registration form is included in the distribution

package for your convenience.  It is in the file ORDERFRM.

Please take time to fill it out and return it.  If you would

like high-quality software to continue to be made available

on a try-first basis and for a reasonable cost, it is up to

YOU to support such programs.

Please send your registration form, suggestions and any

comments that you might have to:

Peter Volpa         or       For questions only,

Circuit Systems                 no orders.  Call

418 Church Road                  (609) 875-5433

Sicklerville, NJ  08081-1727           12 - 10 PM EST

Registering PC-ECAP by Credit Card

We have established a special arrangement with the Public

(software) Library for those of you who wish to order by

credit card.  For credit card orders only, you can order

from PsL with your Master Card, Visa, AmEx or Discover card

by calling (800) 2424-PsL or (713) 524-6394.  You may also

FAX to (713) 524-6398 or CompuServe to 71355,470 or mail to

PsL, P.O. Box 35705, Houston, TX 77235-5705.  These numbers

are for ordering only.  For information about volume

discounts, site licensing, shipping of product, returns,

latest version number or other technical information, please

write to Circuit Systems or call (609) 875-5433.  All credit

card orders will be charged $4 for Shipping and Handling.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 4

DISCLAIMER

ALTHOUGH WE HAVE EXTENSIVELY TESTED PC-ECAP AND BELIEVE IT

WILL GIVE ACCURATE RESULTS IF USED CORRECTLY, WE SUPPLY THIS

PROGRAM AS IS.  CIRCUIT SYSTEMS DISCLAIMS ALL WARRANTIES,

EXPRESSED OR IMPLIED, THAT THIS PROGRAM IS FIT FOR ANY

PURPOSE.  CIRCUIT SYSTEMS ALSO ASSUMES NO LIABILITY FOR ANY

DAMAGES EITHER DIRECT OR CONSEQUENTIAL, WHICH MAY RESULT

FROM THE USE OF THIS PROGRAM.

Trademarks

MS-DOS and Excel are registered trademarks of Microsoft

Corporation.

PC-DOS, PC/AT, PC/XT, and PS/2 are registered trademarks of

IBM Corporation.

Compaq is a registered trademark of Compaq Computer Corp.

Hercules is a registered trademark of Hercules Computer

Technology.

AT&T is a registered trademark of American Telephone and

Telegraph Company.

Olivetti is a registered trademark of Ing. C. Olivetti.

LaserJet is a registered trademark of Hewlett-Packard

Company.

Lotus and 1-2-3 are registered trademarks of Lotus

Development Corporation.

Quattro Pro is a registered trademark of Borland

International Corp.

AS-EASY-AS is a trademark of TRIUS, Inc.

Association of Shareware Professionals

This program is produced by a member of the Association of

Shareware Professionals (ASP).  ASP wants to make sure that

the shareware principle works for you. If you are unable to

resolve a shareware-related problem with an ASP member by

contacting the member directly, ASP may be able to help. The

ASP Ombudsman can help you resolve a dispute or problem with

an ASP member, but does not provide technical support for

members' products.  Please write to the ASP Ombudsman at 545

Grover Road, Muskegon MI 49442-9427 or send a Compuserve

message via CompuServe Mail to ASP Ombudsman 70007,3536.

The OMB may be contacted by FAX by sending to the ASP FAX

number: (616) 788-2765.  In communication with the OMB

please include a telephone number and/or FAX if available.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 5

System Requirements:

*    IBM PC, XT, AT, PS/2 or compatible.

*    PC or MS-DOS 2.0 or later.

*    One floppy disk drive.

*    384K of system memory.

*    A graphics card is recommended for best

performance.  But if all you have is a Monochrome

Display Adapter, PC-ECAP will even graph its

results on that - though the resolution will be

much lower.

*    An IBM/Epson or HP LaserJet+ compatible printer

if hard copy is desired.  Most printers used with

IBM and IBM compatible computers support one of

these standards.

Attention users of MS-DOS version 3.2

Version 3.20 of MS-DOS contains a bug that can cause

unexpected results in programs that use floating-point math

(eg. PC-ECAP).

You do not need to read any further unless your system has

ALL of the following characteristics:

1.    You use MS-DOS version 3.20.

2.    You boot from a hard-disk drive.

3.    Your system has a math coprocessor (for instance,

an 8087 chip).

4.    You run programs that use floating-point math

(eg. PC-ECAP).

For systems that satisfy all of the preceding conditions,

you may be able to eliminate floating-point math problems by

installing a small patch in DOS.  Please contact Microsoft

for complete details on this.

Since this bug has been fixed in later versions of MS-DOS,

you may also want to consider upgrading your DOS to a more

recent version.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 6

Before You Begin

The very first thing that you should do before going any

further is to make a backup copy of your distribution disk.

To do this just use the DOS command DISKCOPY.  This will

work whether or not you have two floppy disk drives.  It

also doesn't matter if the diskette that you want to copy to

is formatted or not.  DISKCOPY will format it for you.

From the DOS prompt type this:

DISKCOPY A: B:

You will be asked to "Insert source diskette in drive A:"

This should be your original PC-ECAP disk.  Continue to

follow the directions on the screen and you will have your

backup copy.

Installing PC-ECAP

We have included an "Install" program with PC-ECAP to make

installation very easy.  The Install program will not do

anything "funny".  It will not modify your AUTOEXEC.BAT or

your CONFIG.SYS files.  What it will do is create a

subdirectory on your hard drive called "\ECAP".  Then it

will copy all the needed PC-ECAP files from your floppy

drive to that subdirectory.  If you have any PC-ECAP program

files (NOT data files) prior to version 2.0, they will be

deleted.  That's all.  It is strictly for your convenience.

The default "from" drive is the drive where INSTALL.EXE is

located and the default "to" drive is drive C:.  These may

be changed to any other drives as needed.

If you don't have a hard drive, you don't need to proceed

any further.  Skip the rest of this section and use your

backup copy as your working disk.

To use the install program, insert your backup copy of

PC-ECAP into your floppy drive and type:

A:INSTALL

Follow the directions and before you know it, you will have

PC-ECAP on your hard drive.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 7

Quick Start

Although there is no substitute for reading this entire

manual, I know there are those of you who can't wait to try

a new program.  So for those who like to try-first-read-

later, this will give you a taste of what PC-ECAP can do.

Start PC-ECAP by typing:

ECAP

at the DOS prompt.

From the main menu choose the "Select input file" selection.

Select EXAMPLE1.DAT as your input file.  The circuit

described by EXAMPLE1.DAT is a simple RC low-pass filter and

is shown below.

1        1.59K                 2

------/\/\/\/\------*---------

|

|

input                      ___|___  .1uf      output

_______

|

|

0  |

--------------------*----------

Now from the main menu choose "Analyze circuit".  The

analysis will begin and you will see the results as the

program progresses.  When the analysis is complete, the

results will be plotted on your display.  First you will see

a plot of Magnitude vs. Frequency.  If you press any key

other than ESC you will get a plot of Phase vs. Frequency.

Pressing any key (other than ESC) again will show you

Magnitude again.  When you are finished, press ESC to return

to the main menu.

As I said, this is a taste - for the details read on.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 8

Printer Configuration

PC-ECAP supports four printer types.  To properly print on

your printer, you must configure the program for the printer

that you are using.  The "Change program settings" choice of

the main menu has an option to "Pick a printer to use".

Select your printer from the list given.  One of these types

should be a good match for your printer.  Refer to the

"Change program settings" section in the manual for more

details.  Be sure to save your choice.

Users of PC-ECAP versions prior to version 2.0

Before version 2.0, all numbers were represented and stored

in BCD format.  With version 2.0, numbers are now stored in

IEEE binary format.  However, PC-ECAP can read any result

files stored in BCD.  When you access an old file you will

see the message "Converting file format . . .".  This

conversion is done "on the fly", meaning the converted file

is not saved.  To avoid this file conversion every time you

view an old file, simply rerun an analysis.

Accessing PC-ECAP Circuit Files in Other Directories

If you are running DOS 3.0 or later you may easily work with

circuit files in other directories or on other drives.  To

do this you must have the directory that contains the

PC-ECAP program files listed in your PATH.  You should have

a line similar to this in your AUTOEXEC.BAT file:

PATH=C:\ECAP

Yours will not be EXACTLY like this one.  You may have

additional directories listed or the drive letter may be

different.  The important thing is that the PC-ECAP drive

and directory be part of the PATH.  You may want to refer to

your DOS manual if you need more information on this.

Once your PATH is set up, DOS will know where to find

PC-ECAP no matter where you are.  And under DOS 3.0 or

later, PC-ECAP will know where to find any additional files

it needs to run.

Memory Requirements

With version 2.1 of PC-ECAP, you can analyze circuits with

up 90 nodes.  Older versions of the program had a limit of

40 nodes.  Analysis of a 90 node circuit requires that you

have 430K bytes of free memory.  That is, memory left after

you load any memory resident programs.  If you don't have

this much memory available, don't worry.  You will just be

limited to circuits having fewer nodes.  If you have less

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 9

than 430K free, PC-ECAP will give you its estimate of the

largest circuit that you may analyze on program start up.

Keep in mind that this is an estimate and therefore it is

not exact.  Therefore, if you are told that you have enough

memory to analyze circuits with up 57 nodes, and you have a

circuit with 58, give it a try.  It may very well work.

Also please note, that since node numbers start at 0, the

highest possible node number is 89.

Network / Multi-User Operation

Added with version 2.1 of the program, is the ability to run

PC-ECAP on a local area network.  Your network must support

DOS file-sharing (added with DOS version 3.0).  When running

on a network, multiple users may share circuit description

and analysis result files.

PC-ECAP imposes two main restrictions when accessing files

on a network.  If a file is being edited, no one else may

run an analysis on it.  If a circuit is being analyzed, no

one may view or output the results until the analysis is

complete.  In general, individual file access at any one

time is restricted to operations that make sense.  For

example, a circuit whose analysis is only partially complete

can't be viewed.  Of course you may do any operation you

choose if you are the only person using a particular file.

If you attempt to access a file that is already in use,

PC-ECAP will display the message, "File is currently being

used by someone else".  You may wait until that file is

available or you may choose to work on a different file.

You should also be aware that PC-ECAP maintains a single

configuration file.  So if you change some of the program

settings, and then write these changes to disk, the next

person on the network to run the program will get these new

settings.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 10

Getting Started

PC-ECAP is started by typing the following command at the

DOS prompt:

ECAP   [ options ]

The optional command line switches are used to manually set

the video mode.  Since PC-ECAP is usually able to do this

automatically, these options should not be needed by 99% of

computers.  They are covered separately in Appendix A.  If

you have trouble seeing the main menu after the opening

screen, please refer to this appendix.

When PC-ECAP is started, you will first see the main menu.

The program has been designed to be very easy to use, and

the menu system plays an important part in reaching this

goal.  All menu choices may be selected in either one of two

ways.  First, a selection may be made by typing the first

letter of an item.  These are highlighted so they stand out.

Selection may also be made by using the cursor keys (up

arrow, down arrow, home and end) to highlight a choice.

When "ENTER" is pressed the selection is made.  The program

will do its best to logically anticipate your next choice.

For example, after editing a circuit description and

returning to the main menu, the choice "Analyze circuit"

will be highlighted - assuming that after editing, you most

likely will want to analyze your circuit.

MAIN MENU Descriptions

Select input file

This selection will display an alphabetically sorted

list of all the circuit description files in your

current directory.  Use the cursor keys to select a

file to use.  If you see "more . . ." displayed at the

beginning or end of the list, this means that there are

additional files on another page.  You can use the PgUp

or PgDn keys in addition to the up arrow, down arrow,

left arrow, right arrow, home and end keys to view

them.  Once your choice is highlighted just press ENTER

to select it.

PC-ECAP also has a "Speed Search" feature.  To use it,

type the first letter of the file that you want to

select, followed by enough letters to uniquely specify

it.  In most cases all it takes is 2 or 3 letters.

Your desired file will be highlighted.  Press ENTER to

select it.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 11

If you want to select a file that is not listed (for

example - a new circuit description that you want to

input) press F2.  A window will appear asking for the

filename.  Type the filename and press ENTER.  All

filenames should be eight characters or less in length.

If you type in more than eight, only the first eight

will be accepted.  A file extension is optional, if it

is omitted the extension ".DAT" will be appended to the

filename.  If a filename has an extension other than

".DAT", the ".DAT" extension will be substituted in its

place.

You can delete a highlighted file by pressing F3.  When

you do so, a window will appear asking, "Delete File?".

Answer "Yes" to delete.  "No" or ESC will abort the

operation.

If at any time you want to abort the current operation,

pressing the ESC key will return you one level up in the

program.

Edit input file

This starts the built-in full screen editor.  The file

that you want to edit must be selected first.  If no

file is selected, you will be sent to the "Select input

file" screen and then returned to the editor.

The editor is very straight forward to use, just type

each line followed by the ENTER key.  When in the

editor these keys have special meaning:

Right Arrow  - move one character to the right.

Left Arrow   - move one character to the left.

Home         - move to the beginning of the line.

End          - move to the end of the line.

Up Arrow     - move one line up.

Down Arrow   - move one line down.

Ctrl Home    - move to the beginning of the file.

Ctrl End     - move to the end of the file.

PgDn         - move 20 lines down.

PgUp         - move 20 lines up.

Del          - will delete the character directly above

the cursor.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 12

Backspace    - will delete the character to the left of

the cursor.

Ins          - will toggle between the insert and

replace modes.  In the insert mode the

cursor will change to a block cursor and

any characters typed will be inserted

between any characters already on the

screen.  When in replace mode, any

characters typed will type over those on

the screen.

In addition to these special keys, five function keys

have special meaning.  Press them alone or in

combination with the "Alt" key to access the following

features.

F1 Help

One of the features that has been added to make

PC-ECAP easy to use is the on-line help.  When F1

is pressed a window will appear containing useful

information on items such as circuit element

description formats, analysis range and sweep

type, value types and examples.  You can move

through these with the PgUp and PgDn keys.  When

you are ready to continue editing, pressing F1

again or the ESC key will close the window and let

you edit.

F2 Model Maker

The model maker allows you to more accurately

model bipolar transistors.  The transistor model

used is the Hybrid-Pi model.  Despite its

simplicity, this model gives quite good results

for analysis up to about one half of the

transistor's cutoff frequency (Ft).

When the model maker is activated, a window will

appear.  You will first be asked for the node

numbers of the transistor.  Enter these pressing

the ENTER key after each entry for base, emitter,

and collector.  You next have to answer four

questions.  You will be asked (and need to know)

the following:

Transistor DC collector current - hopefully you

will know this.  If not, try your best guess.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 13

Transistor current gain or Beta - since the gain

of bipolar transistors varies with collector

current, the number that you enter here should be

at the collector current specified above for best

results.  Usually it is available in the data

sheet for the transistor.

Transistor Gain-Bandwidth product (Ft) - this also

should be at the collector current entered above

(check the data sheet).

Transistor common base output capacitance - this

is just the base to collector capacitance.

Usually it is listed as Cobo in the transistor

data.  (Which means output capacitance common base

with the emitter open).

After all the information is entered, the window

will close and components for the model will be

inserted into your circuit description after the

current cursor line.

F3 Insert Line

Pressing F3 will insert a blank line above the

current cursor line and move the cursor line and

all lines below it down.

F4 Delete Line

Pressing F4 will delete the current cursor line

and move all lines below it up.  The deleted line

will be placed in the cut buffer and can be pasted

if desired.

F5 Find Component

The Find Component feature is useful to find

circuit elements in your circuit description file.

Just press F5, enter the element to be found, and

the line containing the element will be

highlighted with the cursor positioned there.  If

the element does not exist, a "Component not

found" message will appear.

Alt-F1 Net List

This feature can make it easier to track down

errors in your circuit description.  The Net List

is a node by node list of all the circuit elements

that make up your description.  By comparing the

connections in this list to your circuit's

schematic, you can verify that your circuit is

described properly.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 14

When the Net List is selected, you will be asked

whether you want your list sent to the screen or

to a defined printer.  If you select screen, a

window will appear listing your circuit's nodes

and their attached components.  Use the cursor

keys to move through this list.  To close this

window and return to editing, just press ESC.

Alt-F2 Rename File

You may change the name of a circuit description

file by selecting "Rename File".  A window will

appear asking for your new file name.  Type the

new name and press ENTER.  ESC will close the

window and leave the existing file name unchanged.

When you rename and save a file, the old named

file will still exist.  This is useful if you have

a circuit that is very similar to an existing

circuit description.  You can modify the existing

description and save it under a new name. If you

want to delete the old named file, you can do so

from the "Select input file" menu.

Alt-F3 Paste Line

This option will insert a line saved in the cut

buffer above the current cursor line and move the

cursor line and all lines below it down.

Alt-F4 Copy Line

Will copy the current line into the cut buffer.

Used with the Paste Line feature above.

Alt-F5 Find Next

This will find the next component meeting your

search criteria as specified under "Find

Component".  If you have asked for a unique

component, you won't be able to find a "next".

This option is most useful when you want to find

all occurrences of a part, such as all capacitors

or all resistors, etc.

Leaving The Editor

When you are finished with your editing, press the ESC

key.  If you have made any changes to the file, a

window will pop up and ask, "Save Changes?".

Answer "Y" to save all changes, or "N" to ignore all

changes.  An ESC will cancel the exit, if you have

changed your mind.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 15

Analyze circuit

This choice will begin the actual analysis of the

circuit selected.  If a file containing the circuit

description has not been selected, you will first be

sent to the "Select input file" screen and then

returned to begin analysis.

PC-ECAP will subject your circuit to a 1 volt signal at

the selected input node and then calculate the

magnitude and phase response at the selected output

node.  The input signal is from an ideal AC voltage

source and therefore it can drive any impedance.  Any

output node selected will not be loaded by PC-ECAP.

This analysis is repeated for each of the frequencies

in the specified input range.  The number of

frequencies within the input range that PC-ECAP will

subject your circuit to, is controlled by the "Change

the number of sample points" option under the "Change

program settings" choice of the main menu.  Its use

will be fully explained under that heading.  As PC-ECAP

progresses, it will display magnitude and phase results

on the screen.  The results of the analysis are also

saved in a disk file.  The file will have the same name

as the circuit description file but will have a file

extension of ".OUT".

When the analysis is finished, the results will be

plotted on your display.  For all display adapters

other than VGA, PC-ECAP will use the highest resolution

that your display adapter supports.  VGA adapters will

be run in EGA mode (640 x 350).  The resolution used

will be as follows:

CGA          640 x 200

EGA          640 x 350    enhanced display

640 x 350    monochrome display

640 x 200    color display

Hercules     720 x 348

VGA          640 x 350    monochrome or color

MCGA         640 x 480    monochrome or color

AT&T         640 x 400

MDA          80 x 25

Pressing any key other than ESC will cycle through the

various plots.  ESC will return you to the main menu.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 16

View output data

This selection allows you to view the results of

circuits that have been previously analyzed.  The

results will be listed in tabular form on the display.

You can use the cursor keys to move through them.  The

cursor keys that are active under this selection are

the following:

Up Arrow     - scroll up one line.

Down Arrow   - scroll down one line.

PgUp         - scroll up 19 lines.

PgDn         - scroll down 19 lines.

Home         - display the first 19 lines.

End          - display the last 19 lines.

The analysis results can also be plotted on the display

by pressing the F1 function key.  Plotting works the

same way as described in the "Analyze circuit" section.

Pressing any key except ESC will cycle through the

various plots.  Pressing ESC will return you to the

tabular listing of the data. Press ESC again to return

to the main menu.

Output to printer

PC-ECAP will normally send the results of an

analysis to a printer selected under the "Change

program settings" option.  If desired, the results can

instead be sent to a disk file for printing later.  The

file will have a filename that is the same as the file

selected, but with a file extension of ".PRN".

Partial output listing

This choice will output a listing of the circuit

description at the time of analysis and high resolution

plots of the results.

Full output listing

Choosing this option will output a listing of

the circuit description at the time of analysis, a

tabular listing of the analysis results, and high

resolution plots of these results.

It is necessary to have a printer that is IBM/Epson

compatible or HP LaserJet compatible if printed output

is desired.  Most printers used with IBM compatible

computers support one of these standards, although in a

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 17

few cases it may be necessary to select the proper mode

by means of a switch on the printer.  Be sure that you

have configured PC-ECAP for your particular printer

type.  This is done under the "Change program settings"

option of the main menu.  (see next section for

details).

Change program settings

PC-ECAP allows you to configure certain aspects of its

operation.  The configuration information is saved in a

disk file that is read every time the program is run.

The name of this file is "PC-ECAP.CFG".  If this file

is not present it will be created.  This should only be

necessary the very first time you run PC-ECAP.  The

file will have as defaults: 70 sample points for

analysis, LPT1 selected as the output destination, and

an IBM/Epson 9 pin printer selected as your printer

type.  Any of these may be changed by means of this

menu.

Change number of sample points

When an analysis is performed, the desired frequency

range is divided into a number of sample points.  The

number of points may be set to 35, 70, 140, 280 or 560.

It should be noted that you must obviously make a

choice between the high analysis resolution provided by

selecting 560 sample points and the faster

computational speed offered by selecting 35 points.

The number of points you select really depends on the

circuits that you analyze.  I have found that for most

circuits 70 sample points works quite well.  You should

increase the number of points if you have a circuit

that has abrupt changes in magnitude or phase.  You may

also want to increase the number of points if you

analyze circuits over a very wide frequency range.

Select output destination

The output from PC-ECAP can be sent to a printer

designated as LPT1, LPT2, or LPT3.  Alternately output

may be sent to a disk file for later printing.  For

printed output, select the port that you have your

printer attached to (normally this will be LPT1).

Pick a printer to use

PC-ECAP supports four different printer types.  One of

these types should be a good match for just about any

printer.  The supported printer types are: IBM/Epson 9

pin, HP LaserJet, Epson 24 pin and IBM 24 pin.  Just

choose the type that matches your printer.  Some

printers can emulate more than one type, be sure that

your printer and PC-ECAP are both configured for the

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 18

same printer type.  After you make your selection, it

is important that you "Write changes to disk" (see next

choice).  If you don't save your printer configuration,

PC-ECAP will not be setup properly the next time you

use it.  This only needs to be done once.

Some printers allow you to manually set a particular

font, pitch or mode.  DO NOT DO THIS.  On most printers

this will override any printer commands that your

computer may send, and PC-ECAP's printed output will

not print properly.  If you have a problem with printed

output, insure that your printer is in "Pgm" mode.

When in "program" mode, your printer will follow your

program's commands, and your printed output will appear

as intended.

Write changes to disk

This selection will write any changes in configuration

to the "PC-ECAP.CFG" file.  Select this option to make

the changes you have made carry over to the next

session of PC-ECAP.  If the changes are not written to

disk, they will be in effect only for your current

session.

Return to main menu

Returns you to the main menu.

Quit this program

Ends your current session with PC-ECAP and returns you

to DOS.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 19

Analyzing a Circuit Using PC-ECAP

The first step in analyzing any circuit is to draw the

circuit on a piece of paper.  Next number all the nodes in

the circuit.  A node is any connection of 2 or more

components.  Node numbers must be non-negative integers.

There are two rules that you must follow when numbering

nodes, first ground or the common connection between input

and output must always be designated as node number 0.

Secondly, although the nodes may be numbered in any order,

none may be skipped.

PC-ECAP has a limit of 90 nodes and can handle up to 1000

components.  Some components count as more than one circuit

element.  You can use the following as a guide;

Resistors, Capacitors, Inductors, Fet's and

Transconductance amplifiers each count as one

circuit element.

OpAmps and Bipolar transistors count as two.

Transformers count as four.

So it is possible to have fewer than 1000 components in your

circuit and still exceed this limit.  Although, in practice,

it is highly unlikely that you will reach this limit.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 20

Input Format

The input format of a PC-ECAP circuit description is easy to

learn and remember.  Fields describing a component are of

the free format type and are simply separated by spaces.

Circuit element names should begin with a letter designating

the type of circuit element and may optionally be followed

by up to 14 additional alphanumeric characters.

Values used to describe a circuit element may be integer

(100), floating point (1000.45), either an integer or

floating point number followed by an integer exponent

(1E-3, 4.7E6) or either an integer or floating point number

followed by one of the following scale factors:

P=1E-12   N=1E-9    U=1E-6    M=1E-3    K=1E3

MEG=1E6        G=1E9

Letters immediately following a number that are not scale

factors are ignored, and letters immediately following a

scale factor are ignored.  So 10, 10ohms and 10Hz all

represent the same number, and u, uf and umhos all represent

the same scale factor.  Note that 1000, 1000.0, 1000Hz, 1E3,

1.0E3, 1K and 1KHz all represent the same number.

In addition, some values may also be expressed in "European"

format.  This format has one or more digits followed by a

scale factor that also takes the place of a decimal point,

followed by one or more digits.  For example, 1500 would be

expressed as 1K5.  When values are in this format, these

scale factors are recognized: R, K, M.  `R' is equal to a

factor of 1.  `K' is equal to 1000, as above.  `M' is equal

to a factor of 1E6.  Please note the difference between this

use of `M' and `M' in the non-European format above.  When

`M' is placed at the end of a number it multiplies the

number by 1E-3.  But when `M' is placed between two numbers,

the multiplication factor is 1E6.  Be careful!

Some examples:

6R8 is 6.8,    5K1 is 5100,    4M7 is 4.7 million

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 21

Circuit Element Description

Resistors

RXXXX N1 N2 Value

N1 and N2 are the two element nodes.  Value is the

resistance in ohms.

Examples:

R1 1 2 100

RLOAD 12 15 1K

Capacitors

CXXXX N1 N2 Value

N1 and N2 are the two element nodes.  Value is the

capacitance in farads.

Examples:

C4 8 9 1uf

COSC 2 3 22pf

Inductors

LXXXX N1 N2 Value

N1 and N2 are the two element nodes.  Value is the inductor

value in henries.

Examples:

L3 7 0 10mh

LTANK 4 5 56uh

Transformers

TXXXX +Pri -Pri +Sec -Sec  N

+Pri and -Pri are the nodes for the transformer primary.

+Sec and -Sec are the nodes for the transformer secondary.

N is the turns ratio between primary and secondary (Np/Ns).

The + and - signs denote the phasing of the windings,

e.g. the +Pri node will be in phase with the +Sec node.

Example:

T1 3 0 4 5 10turns

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 22

Bipolar Transistors (both NPN and PNP)

BXXXX NB NE NC Beta Rbe

NB, NE, and NC are the base, emitter and collector nodes

respectively.  Beta is the Transistor current gain.  Rbe is

the equivalent base-emitter resistance (in ohms) used in the

Hybrid-Pi transistor model.

Example:

B1 9 0 11 100 1250

Field Effect Transistors  (both JFETs and MOSFETs)

FXXXX NG NS ND Gm

NG, NS, and ND are the gate, source, and drain nodes

respectively.  Gm is the FET transconductance in mhos

(Amps/Volt).

Example:

F2 3 4 0 2000umhos

Operational Amplifiers

OXXXX N+ N- NO Gain Rout

N+, N-, and NO are the non-inverting input, inverting input

and output nodes respectively.  The opamp output is

referenced to ground (node 0).  Gain is the open loop gain

of the opamp, usually somewhere between 10,000 and 1 million

times.  Rout is the open loop output resistance for the

opamp.  Real opamps usually have an output resistance

between 10 and 150 ohms.

Example:

O1 1 2 2 100k 75

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 23

Transconductance Amplifiers

GXXXX N+ N- NO+ NO- Gm

N+ and N- are the non-inverting input and inverting input.

NO+ and NO- are the non-inverting output and the inverting

output.  Please note that these outputs are floating.  In

other words the actual output is not from either output to

ground, but is developed between the outputs.  Either output

(but not both) may be tied to ground if a single ended

output is desired.  Gm is the amplifier transconductance in

mhos (Amps/Volt).  The transconductance amplifier may be

used in its own right or it may be used to perform a voltage

to current conversion, sometimes useful when simulating

other devices.

Example:

GXXXX 1 2 3 0 100mmhos

A Few Words About Transconductance Amplifiers

Although most people are familiar with bipolar transistors

and operational amplifiers, not as many know about

transconductance amplifiers.  These amplifiers are another

variation of a dependent source.  The common bipolar

transistor is a current controlled current source; A small

current entering the base controls a much larger current

flowing between the collector and emitter.  The opamp is a

voltage controlled voltage source; A very small input

voltage effectively controls a much larger voltage swing at

the output.  With this said, a transconductance amplifier

can be described as a voltage controlled current source; A

small voltage difference between the two inputs controls a

current flowing either into or out of the output.  In our

case this current will flow from one output to the other.

Valid Circuit Elements

All of the lines that make up a PC-ECAP circuit description

file must either begin with one of the previously mentioned

elements, be blank, or be a comment.  To add a comment to

your circuit description, begin the line with a semicolon.

An example comment might be:

; this begins the second filter section

If a line begins with something other than a circuit

element, a blank line, or a comment, PC-ECAP will abort the

analysis and flag the bad line.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 24

Specifying Analysis Parameters

Once your circuit has been described you need to determine

how it will be analyzed.  PC-ECAP will subject your circuit

to a 1 volt signal at the selected input node and then

calculate the response at the selected output node.  The

frequency of the input signal can be swept over the

specified range in either a linear or logarithmic fashion.

The PLOT statement is used to tell PC-ECAP which node of

your circuit will be used for the input, which node will be

used for the output, the frequency range that you want to

analyze, the sweep mode (linear or logarithmic) and the

analysis type.  PLOT should be the last statement in your

description.

This is the format for the PLOT statement:

PLOT NI NO Start-freq. Stop-freq. Plot-mode Plot-type

PLOT may optionally be abbreviated to P.  NI is the

circuit's input node.  NO is the circuit's output node.

Start-frequency is simply the beginning frequency for the

analysis.  Stop-frequency is the upper frequency limit of

the analysis.  Plot-mode specifies whether a linear or a

logarithmic frequency sweep is to be performed.  This may be

abbreviated to 3 letters (LIN or LOG) if desired.

Plot-type may optionally be used to specify the analysis

type.  If it is not specified, magnitude and phase will be

calculated.  To calculate group delay, impedance, VSWR or

return loss, you must provide this parameter.

for:                   Plot-type is:

group delay                   Delay

impedance in magnitude        ZPolar

and phase format

impedance in Real and         ZRectangular

Imaginary format

VSWR                        SWR    <reference impedance>

(voltage standing

wave ratio)

return loss                   RLoss  <reference impedance>

Plot-type may be abbreviated to the letters that are

capitalized.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 25

Impedance, VSWR (voltage standing wave ratio) and return

loss calculations will be done at the input node specified.

This doesn't mean that you can't calculate an output

impedance instead.  If your circuit's output is node 8, and

you are interested in its output impedance, just specify

node 8 as the input node for the PLOT statement.  You can

look at it this way, to determine impedance, PC-ECAP must

apply an excitation signal to your circuit.  This signal is

always applied to the node listed as the input node for your

circuit.

For VSWR and return loss, a reference impedance must also be

specified.  This is the impedance that your circuit is

supposed to be matched to.  This is usually 50 or 75 ohms,

but it may be any positive value.

As an example of what we have been discussing, we might have

a circuit where node 2 is the input and node 5 is the

output.  The lowest frequency that we want to look at is 100

Hz and the highest frequency is 10 KHz.  We are only

interested in magnitude and phase vs frequency.  And we want

to sweep this range logarithmically.

This PLOT statement will specify all this:

PLOT 2 5 100HZ 10KHZ LOG

For another example, we have a circuit with node 1 as its

input.  We are interested in determining the input VSWR vs

frequency.  The input is supposed to match 50 ohms, and

operate from 7.0 MHz to 7.3 MHz.

This is the PLOT statement that will do the job:

PLOT 1 5 7MEG 7.3MEG LIN SWR 50

Please note that for impedance, VSWR or return loss

calculations, the output node specified in the PLOT

statement is unimportant.  It is ignored.

What is Group Delay anyway?

If a circuit is designed to pass all important frequencies

with equal amplitude and have absolutely linear amplitude

characteristics, a waveform may still pass through badly

distorted.  The problem is delay distortion; if all

frequency components do not pass through a circuit with the

same time delay, the output waveform will not look the same

as the input.  Group delay is the rate of phase shift vs

change of frequency, and is defined as:

1    d0

group delay = --- x ---  seconds

360   df

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 26

An Example Please!

It's about time now to look at an example circuit.  We will

analyze a simple RC low-pass filter.  The circuit is shown

below.

1        1.59K                 2

------/\/\/\/\------*---------

|

|

input                      ___|___  .1uf      output

_______

|

|

0  |

--------------------*----------

This circuit has 3 nodes and they have been numbered.

Notice that we have numbered the common node between the

input and the output as node 0.  This is required.

The next thing to do is to describe the circuit to PC-ECAP

so that it will know just what is connected to what and the

component values associated with each part.  The next two

lines will completely describe this circuit:

R 1 2 1.59K

C 2 0 .1UF

The first line says to PC-ECAP that a resistor is connected

between nodes 1 and 2 and has a value of 1.59K (notice that

you don't have to enter 1590 for the resistor value, PC-ECAP

understands what "K" means).

The second line reads "a capacitor is connected between

nodes 2 and 0 and has a value of .1uf".  Now all we have to

tell PC-ECAP is where we want to put the input signal, where

we want to look for the output and what frequency range we

want to cover.  This is done with the PLOT statement.  For

this circuit we want node 1 to be our input and node 2 to be

our output.  Also we want to sweep the input frequency from

100Hz to 10KHz in a logarithmic fashion.  The next line will

tell PC-ECAP all this.

PLOT 1 2 100 10KHZ LOG

On your distribution disk (or in the archive file if you got

this program from a BBS) you will find EXAMPLE1.DAT.  This

file contains the circuit description for this circuit.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 27

If you haven't started PC-ECAP yet do so now.  From the

main menu choose the "Select input file" selection.  Select

EXAMPLE1.DAT as your input file.  Next from the main menu

choose "Edit input file".  The editor will load EXAMPLE1.DAT

and you will see the following:

; Example1.dat - RC low-pass filter with a cutoff frequency

;                 of 1 KHz.

R 1 2 1.59k

C 2 0 .1UF

PLOT 1 2 100 10KHZ LOG

This circuit description tells PC-ECAP all that it needs to

know about this circuit and how you want to analyze it.

Press ESC to get back to the main menu and now choose

"Analyze circuit".  The analysis will begin and you will see

the results as the program progresses.  When the analysis is

complete, the results will be plotted on your display.

First you will see a plot of Magnitude vs. Frequency.  If

you press any key other than ESC you will get a plot of

Phase vs. Frequency.  Pressing any key (other than ESC)

again will show you Magnitude again.  If you have an EGA

with 256K of memory or a Hercules or VGA card, both pages of

graphics memory will be used and the change from Phase to

Magnitude and back will be instantaneous.

Another Example

We have included a few other sample circuits with PC-ECAP.

EXAMPLE2.DAT is a 75 ohm high-pass filter that can be used

to reject television interference.  EXAMPLE3.DAT is a

band-pass filter for use as a modem receive filter.  This

example shows how group delay is calculated.  EXAMPLE5.DAT

is an LC low-pass filer for which input impedance is

calculated.  EXAMPLE6.DAT is an example of the calculation

of return loss.  EXAMPLE7.DAT is an LC band-pass filter for

which VSWR is calculated.  If you'd like, take a look at

them and run an analysis.  You may want to draw them on a

piece of paper first to really see what you are doing.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 28

For our last example we're going to take a look at

EXAMPLE4.DAT.  This circuit is a single stage transistor

amplifier made with a 2N3904 NPN transistor.  We will

demonstrate the use of the "Model Maker" that is part of the

full screen editor.  Here's the circuit:

+10 volts

0  |

\   R4

/   470 ohms

\

/

R3       |

8.2K      |  3

+--\/\/\/\----*---------

|             |         output

|            /

R1        |         | /

1        270 ohms   |   2     |/       2N3904

-------/\/\/\/\---*---------|

|         |\

input              R2   \         | \

1.2K  /           _\/

\             |

/             |

|   0         |

------------------*-------------*---------

As you can see the first thing you need to do is to number

the nodes.  Please take a look at the top of R4.  This end

of the resistor connects to the +10 supply voltage but we

have numbered it as node 0.  This is an important point.

Any node that has a DC voltage connection should always be

connected to ground.  For the sake of an AC analysis, any DC

voltage source is the same thing as ground.

Now that we have numbered the nodes, we can describe the

circuit:

R1 1 2 270

R2 2 0 1.2K

R3 2 3 8.2K

R4 3 0 470

Alright, now we are ready for the NPN transistor.  The

transistor that we are using is a 2N3904.  We could use a

simple one line description for this transistor, but that

wouldn't show what happens to the transistor at high

frequencies.  What we need is a more sophisticated

transistor model.  One that will change its characteristics

as the frequency changes like a real transistor.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 29

PC-ECAP has the capability to create just such a model.  But

first you need to know some information about the transistor

that you want to model.  This is what is required:

DC collector current -

For this circuit about 10 milli-amps flows

from the collector to emitter.

Transistor current gain -

At 10 ma. this is a minimum of 100.  This is

the same as Beta (the current gain at DC).

Transistor Gain-bandwidth product -

For the 2N3904 with a collector current of 10

ma. this is 300 MHz minimum.  This is also

known as Ft.

Common base output capacitance -

This is just the capacitance from the base to

the collector.  For the 2N3904 it is 4 pf

maximum.

Now that we have the required information we can make a

transistor model.  Please note that it doesn't matter for an

AC analysis whether the transistor is NPN or PNP.

From the editor press F2.  This will start the Model Maker

function.  A window will appear and ask first for the node

numbers for base, emitter and collector.  You would just

enter 2, 0, and 3.  Then you will be asked for the

information mentioned above.  After answering all the

questions your model will be created and inserted into your

circuit description on the line following the cursor.

For this circuit we might want to know just how good a

frequency response we have, so let's sweep a frequency range

of 1 KHz to 100 MHz.  And let's do it logarithmically.  With

node 1 as the input and node 3 as the output, this line

should make PC-ECAP do what we want:

P 1 3 1KHZ 100MEG LOG

As you might notice, PLOT can be abbreviated to P.

That's all there is to it.  Run an analysis and see what

kind of amplifier we have.  You should also try changing

some component values to see what happens.  What happens if

you decrease R1?  Or increase it?

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 30

Models

The circuit models that PC-ECAP uses for Transformers,

Bipolar transistors, FET's, Operational Amplifiers, and

Transconductance Amplifiers are ideal models.  If you

desire, you can easily create models that are more complex.

For Bipolar transistors if your need a better model just use

the "Model Maker" feature in the editor.  As previously

discussed, a Hybrid-Pi transistor model will be created and

inserted into your circuit description.

For the transformer, the addition of two inductors will make

a transformer that is quite "real".  You can add an inductor

in parallel with the primary to model the transformer's

magnetizing inductance.  This inductor should be equal to

the transformer's primary inductance.  The transformer's

manufacturer can supply the inductor value for this or you

can measure it yourself (the secondary should be open for

this test).  A second inductor can be added in series with

the primary winding (and the inductor paralleling the

primary).  This inductor will model the transformer's

"leakage" inductance.  This inductance is due to the

magnetic field that does not couple the primary and

secondary windings.  Again, the transformer's manufacturer

can supply this or you can measure it (measure the primary

inductance as before, but this time short out the

secondary).  By adding these two inductors, you will create

a transformer that has a finite bandwidth.  The parallel

inductor affects the low frequencies and the series inductor

will affect the high frequencies.

For FET's, the addition of a capacitor to model the gate to

source capacitance and a second one to model the gate to

drain capacitance will improve accuracy at high frequencies.

You can get the appropriate values from your transistor's

data sheet.  Also the Siliconix Low Power Discretes data

book is an excellent source for more information on this.

For FET's:     Cgs = Ciss - Cgd = Ciss - Crss

Cgd = Crss

Operational Amplifiers today are usually frequency

compensated internally.  Most of the time a method called

"dominant pole compensation" is used.  This method inserts a

pole in the opamp's frequency response at a very low

frequency.  This pole will dominate the response (hence the

name) and roll off at a 20 db/decade rate.  The opamp model

that PC-ECAP uses is not frequency dependent.  The open-loop

gain that you ask for is the gain that you will get for all

frequencies.  It's not that difficult to make an opamp model

that will decrease its gain as frequency increases.  Such a

model is included on your disk as OPAMPMOD.DAT.  Please take

a look at it.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 31

References for Modeling

Although we have tried to give you some useful information

to use for modeling circuit elements, a full treatment of

the subject is beyond the scope of this manual.  For those

of you who are interested, a good college library should

have several books that might help.  Two references that may

be more readily available for many people are:

"National Semiconductor Linear Applications Handbook" from

National Semiconductor Corporation

2900 Semiconductor Drive

Santa Clara, CA 95052-8090

This handbook has an excellent paper entitled "The

Monolithic Operational Amplifier: A Tutorial Study".  This

paper tells you just about everything that you would want to

know about the inner workings of opamps.

A good reference for FET's in general is:

"Siliconix Low Power Discretes Data Book" by

Siliconix Incorporated

2201 Laurelwood Rd.

Santa Clara, CA 95054-1516

They have several interesting application notes on the use

of FET's.

Parting Comments

A considerable amount of time and effort was expended to

develop PC-ECAP.  We sincerely hope that you feel it was all

worth it.  We want you to know that we don't intend to stop

here.  We have more ideas for enhancements that we would

like to add in future releases.  But it is you that will

make it possible.  By registering your copy of PC-ECAP, you

will help make the possible into reality.  Thanks for your

support.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 32

Revision History

Version 2.10 - 11/09/92

Now handles circuits with up to 90 nodes.  Added option to

print only circuit description and graphs.  Negative

inductor values are now accepted.  Changed listing format

for frequencies of 10 GHz and above.  Added network (LAN)

support.  Fixed "border flash" when switching from graphics

to text mode on some CGA cards.  Added table of contents to

manual.

Version 2.01 - 2/03/92

Improved syntax checking on the circuit description files.

More input errors are spotted and flagged.

Version 2.00 - 10/14/91

Added group delay, impedance, VSWR and return loss

calculations.  Now has full support for 24 pin printers.

Now supports HP LaserJet.  Editor has been enhanced.

Analysis routine is now twice as fast as previous versions.

Added AT&T and MCGA support.  Added support for a math

coprocessor.  European format for element values now

accepted.  Improved error checking on circuit description

files.  International time and date on printouts now

supported.  Numbers are now stored in IEEE binary format.

No longer supports "non-compatible" video.  Now requires

384K of system memory.

Version 1.11 - 11/23/90

Added the filename to the headers and plot sheet of the

printout.  Negative resistance values are now accepted.

Fixed bug in hard copy plotting routine - plot would display

spurious points if the plot exceeded the maximum range.

Fixed bug in analysis routine; Occasionally, a circuit with

a very wide range of reactive component values could create

an internal numeric overflow.

Version 1.10 - 9/14/90

Added the option of 35 sample points to the select number of

sample points menu.  Greatly increased the number of plot

ranges available when displaying results.  Magnitude now

spans .1 db/div to 80 db/div and Phase now spans 22.5

degrees/div to 720 degrees/div.

Version 1.02 - 1/30/90

First release widely distributed.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 33

Appendix A - Command Line Switches

PC-ECAP will automatically determine the type of video

display adapter that your computer uses and will set itself

accordingly.  There are a few situations where it is not

possible to accurately determine the adapter type.  For

example, if a monochrome monitor is used with a CGA card,

the program has no way of knowing that.  Also, there are

some advanced EGA cards that will change their display mode

to match a given program.  Since PC-ECAP is trying to change

its mode to match the video adapter, there can be no

guarantee as to what you will actually wind up with.

In order to accommodate these situations (as well as others

that may be unforeseen at this time) PC-ECAP has a series of

optional command line switches that can be used to set the

video display mode.

To start PC-ECAP with a command line option, type this at

the DOS prompt:

ECAP    [ option ]

Where "option" is one of the following:

/H        Hercules Monochrome Graphics card (or

compatible).

/C        Color/Graphics Adapter with color monitor.

/CM       Color/Graphics Adapter with monochrome

monitor.  For use with Compaq monochrome

display or computers having LCD displays.

/E        Enhanced Graphics Adapter with EGA monitor.

/EC       Enhanced Graphics Adapter with CGA monitor.

/EM       Enhanced Graphics Adapter with monochrome

monitor.

/MC       Multi-Color Graphics Array.

/M        Monochrome Display Adapter.

/A        AT&T or Olivetti video adapter with color

monitor.

/AM       AT&T or Olivetti video adapter with

monochrome monitor.  May also be useful for

some Laptop computers.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 34

Appendix B - Error Messages

ABORT -- Bad circuit description

The PLOT statement must be the last line of your

circuit description.  You have entered additional

circuit elements after the PLOT statement.  Move these.

ABORT -- Blinking nodes have been skipped

You have skipped over one or more nodes in your circuit

description.  Node numbers must begin at 0, and be

numbered consecutively.

ABORT -- Blinking nodes have less than two components

Your circuit description has one or more "dangling"

components.  All nodes (other than the input) must have

at least two components attached.

ABORT -- Insufficient memory for operation

PC-ECAP makes extensive use of dynamic memory.  That

is, memory that it allocates only when needed.

Although this makes for very efficient memory usage,

peak memory requirements can not be known up front.

Your only choice here is to do without the feature

that gave you this error or get more RAM memory for

your computer.

ABORT -- Invalid circuit element

You have an unknown circuit element in your circuit

description file.  Valid lines must be either blank,

begin with a semicolon ";" (if a comment), or begin

with one of the letters of a circuit element.

ABORT -- Invalid node number

You have a circuit element that has one or more node

numbers that is not an integer, is negative or is

greater than 89.

ABORT -- Invalid reference impedance

You have asked to compute the VSWR or return loss for a

circuit, but have not specified an impedance to

reference these calculations to.  Or your impedance is

negative or zero.  Must be a positive value.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 35

ABORT -- Too many circuit nodes

You have 1 or more circuit nodes with a number greater

than 89, either because you actually have a circuit

with more than 90 nodes or you have made a typo and

have inadvertently skipped some.

ABORT -- Too many circuit elements

You have exceeded the limit of 1000 circuit elements.

R, C, L, Fet's and Transconductance amplifiers each

count as one circuit element.  OpAmps and Bipolar

transistors count as two.  Transformers count as four.

ABORT -- Unknown sweep type

You have specified a sweep type that is something other

than LINear or LOGarithmic (may be abbreviated to the

letters shown capitalized).

CAUTION! Conflicting transistor parameters supplied

The data that you have entered for your transistor

doesn't make any sense.  For example - you may have

said that your transistor has a very high cutoff

frequency and then specified a huge base to collector

capacitance.  Or you may have specified a very low

collector current and a high cutoff frequency.  Things

that contradict each other.  Recheck your data and

change if necessary.

ECAPHERC.DRV is not found

You have a Hercules or Hercules compatible graphics

card, and the required driver ECAPHERC.DRV is missing.

This driver is needed to display text when PC-ECAP is

in graphics mode.  If you have DOS version 3.0 or

greater, PC-ECAP will look for this file in the same

directory that contains ECAP.EXE.  This may be

different than the directory that contains your circuit

files.  Otherwise, ECAP.EXE and ECAPHERC.DRV should be

in the current directory.

File is currently being used by someone else

You can get this message when you are running PC-ECAP

on a network or multi-user system and you attempt to

access a file that is already in use.  Wait until the

file is available or pick a different file.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 36

Floating point math error -

It should not be possible to get this error.  If it has

occurred, please send us a complete description of what

you did to get it, so that we may look into the

problem.

Input File Size Limit Reached

PC-ECAP allocates a 384 line buffer for its editor.

Your circuit description file has exceeded this amount.

The only way this should happen is if you have filled

your circuit description file with comments or other

nonessential information.  Remove the extra data.

Insufficient disk space

Well, what can we say, your disk is full.  Delete those

files that you really don't need or get a bigger disk.

LPT1, LPT2 or LPT3 is not ready

You requested PC-ECAP to print something but your

printer isn't ready.  Check to see that your printer's

power is turned on.  Also check to see if your

printer's cable is connected.  You will also get this

message if you try to print on a printer that does not

exist.

No files found.  Enter name of new file.

When you tried to select an input file, PC-ECAP didn't

find any that had the extension ".DAT".  If you

actually believe that there should be circuit

description files in you current directory, quit

PC-ECAP and check to see if you are in the correct

directory.

Not enough space for . . .

You are out of memory and PC-ECAP is unable to load.

Remove any TSR programs from memory or get more memory

for your computer.

Please create this file or select another

You have asked for an analysis of a circuit that does

not exist.  Use the text editor to create a circuit

description file for your circuit.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 37

Please run an analysis or select another

You have asked to see the results of a circuit that you

haven't analyzed yet.  Run an analysis.

Unable to delete "filename"

You tried to delete a file that was most likely marked

as Read Only.  Check to see if this is the case and

change the file's attribute if necessary.

You have enough memory to analyze circuits with

up to ## nodes.

If your computer has less than 430K of free memory

available, you will see this message when you start

PC-ECAP.  The program is giving an estimate of the

maximum circuit size that you can analyze based on the

amount of memory in your system.

Your copy of PC-ECAP has been damaged!

Someone or something (another program perhaps) has

changed your copy of ECAP.EXE.  PC-ECAP checks itself

every time it runs and this check has failed.

Reinstall the program.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

page 38

Appendix C

PC-ECAP to Lotus 1-2-3 file conversion

Included with the REGISTERED version of PC-ECAP is the

utility program ECAP2WKS.  This program can be used to

convert PC-ECAP ".OUT" files into Lotus 1-2-3 compatible

spreadsheets.  This can be very useful if you want to do

additional operations on your data, such as normalizing

circuit gain at a particular frequency or plotting several

quantities on the same graph.

These PC-ECAP spreadsheets can be used with just about all

of the popular spreadsheet programs such as Lotus 1-2-3,

Quattro Pro, Microsoft Excel and the excellent shareware

program AS-EASY-AS.

To convert a file just type:

ECAP2WKS  filename.out

Filename.out should of course be the name of the PC-ECAP

file that you want to convert.  If you don't specify a file

extension, ".OUT" will be assumed.  As an option, you can

also specify the name of the converted ".WKS" file.  To do

this, just add its name after "filename.out".  The ".WKS"

file extension is optional:

ECAP2WKS  filename.out   filename.wks

ECAP2WKS will accept wildcards.  For example, to convert all

files beginning with "BAND", such as "BAND_ONE", "BAND_TWO",

etc., type:

ECAP2WKS  band*.out

And finally, you may also specify full drive and path

specifications along with the filenames.  For example:

ECAP2WKS  D:\ecap\lowpass.out  C:\lotus\123files\

will convert "LOWPASS.OUT" to "LOWPASS.WKS" and place it in

the \LOTUS\123FILES subdirectory on drive C:.

ECAP2WKS checks to see if the file specified is really a

PC-ECAP ".OUT" file before attempting a conversion.

PC-ECAP Reference Manual           (c) 1992 Circuit Systems

###############################################