RaWrite 1.3

------------

Disclaimer of Warrenty

----------------------

Users of this software must accept this disclaimer of warranty:  "This

software is supplied AS IS.  Mark Becker disclaims all warranties, expressed

or implied, including, without limitation, the warranties of merchantability

and of fitness for any purpose.  Mark Becker assumes no liability for

damages, direct or consequential, which may result from the use of this

software."

Purpose

-------

Write a disk image file to a floppy disk.

Equipment/Software Requirements

-------------------------------

* PC/XT/AT or 100% compatible with at least 256K of RAM and a floppy disk

drive.

* MS-DOS version 3.21 or greater.

* A formatted diskette.

This program uses well-documented generic low-level DOS and BIOS functions.

It should run on nearly every PC in existance.  PS/2's should be able to run

RaWrite but this has not been tested.

CAVEAT

------

This program will write ANY disk file to a floppy, overwriting any previous

information that may have been present.  If you wish to re-use a diskette

that has been written to by RaWrite then that diskette will probably need to

be reformatted; all MS-DOS specific information will have been erased.

Usage

-----

C> RAWRITE

And follow the prompts.  All arguments are case-insensitive.

If the source and destination drives are the same, RaWrite will ask the user

to swap diskettes as required.  Rawrite allocates a large buffer in RAM to

reduce the number of disk swaps.

RaWrite may be aborted at any time by typing ^C or CTRL-Break.

Errors

------

RaWrite attempts to determine if the diskette is a 1.44M, 1.2M, 720K, or 360K

diskette by reading sectors 18, 15, and 9 in that order.  If the inserted

diskette is not one of the these types, then RaWrite will abort with an error

message.

Errors such as write protect, door open, bad disk, bad sector, etc. cause a

program abort with a short error message.