Calculator comparison

Here it is by public demand... it is ugly, it contains a few

Mac-specific non-ASCII characters, some of the fields are quoted (thanks

too much Excel), and it is tab/return delimited, and

it compares the HP-75C with Math and IO ROMs, the HP-71B with Math and

HP-IL ROMs, the HP-28S, the HP-42S, and the HP-48SX on a feature by

feature basis.  The conclusion reached is that the HP-48SX is the most

powerful HP ever made so far, although it is a bit weak in the binary,

file, string, and time areas.  The review is catagorized into 20 areas.

-----------------------------------------------------------------------

Function	HP-75C	HP-71B	HP-28S	HP-42S	HP-48SX

Binary - ASCII string bit set/test	"FLAG$, FLAG?"

Binary - ASCII string bitwise operations	"AAND$, AOR$, AXOR$"		"AND,OR,XOR,NOT"		"AND,OR,XOR,NOT"

Binary - ASCII string rotate/shift	"AROT$, ASHF$"

Binary - binary arithmetic			+  -  *  /  NEG	"BASE+,BASE-,I"	+  -  *  /  NEG

Binary - bit 1Us complement	BITCMP	BITCMP	NOT	NOT	NOT

Binary - bit AND	BITAND	BITAND	AND	AND	AND

Binary - bit arithmetic shift right			ASR		ASR

Binary - bit OR	"BITIOR, BITEOR"	"BITIOR, BITEOR"	"OR, XOR"	"OR, XOR"	"OR, XOR"

Binary - bit rotate			"RL, RLB, RR, RRB"	ROTXY	"RL, RLB, RR, RRB"

Binary - bit shift			"SL, SLB, SR, SRB"		"SL, SLB, SR, SRB"

Binary - bit test	BIT	BIT		BIT?

Binary - decimal to/from binary string	"DTB$, BTD"

Binary - decimal to/from binary/octal/hex	"BSTR$, BVAL"

Binary - decimal to/from hex string	"DTH$, HTD"	"DTH$, HTD"

Binary - decimal to/from octal string	"DTO$, OTD"

Binary - decimal to/from string in any base	"TOBASE$, TODEC"

Binary - hex string operations	"HAND$, HOR$, HXOR$"

Binary - hex string rotate/shift	"HROT$, HSHF$"

Binary - hex string to/from ASCII string	"ASC$, HEX$"

Binary - manipulate wordsize			"STWS, RCWS"		"STWS, RCWS"

Binary - octal to/from decimal				"->DEC,  ->OCT"

Binary - real to/from binary			"R->B, B->R"		"R->B, B->R"

Binary - set binary base			BIN	BINM	BIN

Binary - set decimal base			DEC	DECM	DEC

Binary - set hex mode			HEX	HEXM	HEX

Binary - set octal mode			OCT	OCTM	OCT

Complex - arithmetic	"CADD,CSUB,I"	+  -  *  /  ^	+  -  *  /  ^	+  -  *  V  y^x	+  -  *  /  ^

Complex - complex results from real (C-1)			always allowed ?	CPXRES	always allowed

Complex - conjugate	CONJ	CONJ	"CONJ, SCONJ"		"CONJ, SCONJ"

Complex - finite Fourier transform	FOUR	FOUR

Complex - hyperbolic functions	"CSINH,CCOSH,I"	"SINH,COSH,TANH"	"SINH,COSH,TANH"	"SINH,COSH,TANH"	"SINH,COSH,TANH"

Complex - hyperbolic inverse functions	"CASINH,CACOSHI"		"ASINH, ACOSH,I"	"ASINH, ACOSH,I"	"ASINH, ACOSH,I"

Complex - logarithms	"CLOG, CEXP"	"LOG, EXP"	"LOG, EXP"	"LOG,LN,EXP"	"LOG, EXP"

Complex - matrix conjugate transpose	CTRN	TRN	TRN		TRN

Complex - matrix determinant	CDET		DET	DET	DET

Complex - matrix inversion	CINV	INV	"INV, SINV"	INV	"INV, SINV"

Complex - matrix solve system of eqns	CSYS	SYS	/	"/, SIMQ"	/

Complex - matrix transpose	CTRN			TRAN

Complex - real & imaginary parts		"REPT, IMPT"	"RE, IM"		"RE, IM"

Complex - real <-> complex			"R->C, C->R"	COMPLEX	"R->C, C->R, ->V2"

Complex - reciprocal	CRECP		INV	1/X	INV

Complex - solve for nth complex roots	CROOT

Complex - solve for roots of polynomial	PROOT	PROOT

Complex - square root	CSQR	SQRT	C	SQRT	C

Complex - trigonometric functions	"CSIN,CCOS,CTAN"	"SIN,COS,TAN"	"SIN,COS,TAN"	"SIN,COS,TAN"	"SIN,COS,TAN"

Complex - trigonometric inverses	"CASIN,CACOS,CATN"		"ASIN,ACOS,ATAN"	"ASIN,ACOS,ATAN"	"ASIN,ACOS,ATAN"

Complex - unit vector		SGN	SIGN	UVEC	SIGN

Control - branch	"GOTO, GOTOX"	GOTO		"GTO, KEYG"

Control - conditional branch	IF THEN ELSE		IF THEN ELSE		IF THEN ELSE

Control - conditional branch (single stmt)			"IFT, IFTE"		"IFT, IFTE"

Control - conditional case statement					CASE THEN ELSE

Control - counter loop			START STEP	"ISG, DSE"	START STEP

Control - definite loop	FOR {EXIT} NEXT	FOR NEXT	FOR NEXT		FOR NEXT

"Control - indefinite loop, test at bottom"			DO UNTIL		DO UNTIL

"Control - indefinite loop, test at top"			WHILE REPEAT		WHILE REPEAT

Control - procedure call with parameters	CALL	CALL	variable name		variable name

Control - purges and execute new file		CHAIN

Control - resume program execution	CONT	CONT	CONT	R/S	CONT

Control - routine delimiters	END	END	<<  >>	"LBL, END"	<<  >>

Control - selects DATA statement		ON RESTORE

Control - selects error handling code	ON/OFF ERROR	ON/OFF ERROR	IFERR	FS? 25	IFERR

Control - selects statement to GOSUB	ON GOSUB	ON GOSUB

Control - selects statement to GOTO	ON GOTO	ON GOTO

Control - subroutine call	GOSUB	GOSUB	variable name	"XEQ, KEYX"	variable name

Control - subroutine call via variable	GOSUBX		var name EVAL		var name EVAL

Control - subroutine return	"RETURN, POP"	"RETURN, POP"		RTN

Control - suspend program		PAUSE	HALT	STOP	HALT

Control - terminate execution of program	END	STOP	"ABORT, KILL"		"ABORT, KILL"

Control - user defined function	DEF FN  END DEF	DEF FN  END DEF			DEFINE

Control - user defined subprogram	SUB	SUB  END SUB	<<  >>		<<  >>

Control - wait for specified time	WAIT	WAIT	WAIT		WAIT

Display - clear all user menus			0 MENU	CLMENU	0 MENU

Display - clear display			CLLCD	"CLLCD, CLA"	CLLCD

Display - declare menu variable				MVAR

Display - display string	DISP	DISP	DISP	"VIEW,AVIEW,PSE"	DISP

Display - display string and get input	INPUT	"INPUT, LINPUT"		"INPUT, PROMPT"	"INPUT, PROMPT"

Display - display string with formatting	DISP USING	DISP USING

Display - freeze a portion of the display	"WINDOW, TEMPLATE$"	WINDOW			FREEZE

Display - get current custom menu					RCLMENU

Display - get current endline string	ENDLINE$

Display - get current window size	WSTART? WEND? WSIZE?

Display - list current display devices	DISPLAY$

Display - list lines of a file	LIST	LIST		LIST

Display - manipulate display time	"DELAY, DELAY?"	DELAY

Display - manipulate display width	"WIDTH, WIDTH?"	WIDTH

Display - manipulate right margin	MARGIN MARGIN?

"Display - notation, all digits"		STD	STD	ALL	STD

"Display - notation, engineering"		ENG	ENG	ENG	ENG

"Display - notation, fix place"		FIX	FIX	FIX	FIX

"Display - notation, scientific"		SCI	SCI	SCI	SCI

Display - pop all levels of nested menus				EXITALL

"Display - radix, comma"			SF 48	"RDX,"	SF -51

"Display - radix, period"			CF 48	RDX.	CF -51

Display - rearrange directory menu order			ORDER		ORDER

Display - set contrast		CONTRAST

Display - set custom menu			MENU	"MENU, VARMENU"	"MENU, TMENU"

Display - set display devices	DISPLAY IS	DISPLAY IS

Display - show stack display			CLMF	CLD	TEXT

Display - tab to column	TAB	TAB

Display - turn display on or off	LCD ON/OFF			AON/AOFF

Files - accessing libraries					"ATTACH, DETACH"

Files - advance data pointer in file	ADVANCE#

Files - archive all of memory					"ARCHIVE,RESTORE"

Files - assign number to named file	ASSIGN#	ASSIGN#

Files - auto line numbering	AUTO	AUTO

Files - change directory					UPDIR

Files - change to root directory			HOME		HOME

Files - clear all ASSIGN# assignments	CLEAR ASSIGN#

Files - convert between file types	TRANSFORM	TRANSFORM

Files - copy files to/from memory	"COPY, MCOPY"	COPY			"STO, RCL"

Files - create data file		CREATE

Files - create directory			CRDIR		CRDIR

Files - current directory path			PATH		PATH

Files - delete data items	DELETE#	DELETE#

Files - delete directory and its contents					PGDIR

Files - delete file	PURGE	PURGE			PURGE

Files - delete lines	DELETE	DELETE		DEL

Files - edit	EDIT	"EDIT, EDTEXT"			EDIT

Files - find string and edit line	FETCH	FETCH

Files - get catalog entry	"CAT$, TCAT$"	CAT$			LIBS

Files - get current data pointer pos	INDEX#

Files - get current line number in file	LINE#

Files - get last line number in file	LASTLN?

Files - get number of items in line	LINELEN#

Files - get number of nth ASSIGN#ed file	CAT#

Files - get number of records in file		FILESZR

Files - get pointer position in current line	ITEM#

Files - initialize disk or tape	INITIALIZE	INITIALIZE

Files - insert an item at data pointer	INSERT#	INSERT#

Files - lists files	CAT  /  CAT ALL	CAT  /  CAT ALL			PVARS

Files - load private file from tape	INSTALL

Files - make file execute-only		PRIVATE

"Files - make file read-only, unpurgeable"		SECURE

"Files - make file read/write, purgeable"		UNSECURE

Files - make magnetic card read-only	PROTECT

Files - make magnetic card read-write	UNPROTECT

Files - merge lines of files together	MERGE	MERGE

"Files - name current file, edit workfile"	NAME	NAME

Files - pack directory space		PACKDIR

Files - pack file system	PACK	PACK

Files - port manipulation		"CLAIM PORT, FREE PORT, SHOW PORT"			"MERGE, FREE"

Files - read from file	"READ, READ#"	"READ, READ#"			RCL

Files - rename file	RENAME	RENAME

Files - renumber line numbers of  file	RENUMBER	RENUMBER

Files - replace a data item in a file	REPLACE#	REPLACE#

Files - reset data pointer	RESTORE#	RESTORE#

Files - search for value in a data file	SEARCH#	SEARCH

Files - seek to a given location in a file	SEEK#

Files - set verify mode for card reader	VERIFY ON/OFF

Files - show name of ASSIGN# file	FILE$

Files - static data	DATA	DATA

Files - write to file	PRINT#	PRINT#			STO

Files - write to textfile with USING format	PRINT# USING	PRINT# USING

Graphics - activates interactive plot			DGTIZ		GRAPH

Graphics - create blank graphics object					BLANK

Graphics - define character set		CHARSET

Graphics - draw arcs (circles)					ARC

Graphics - draw axes			DRAX		DRAX

Graphics - draw box					BOX

Graphics - draw line					"LINE, TLINE"

Graphics - draw stat bar plot					BARPLOT

Graphics - draw stat data			DRW7		DRW7

Graphics - draw stat histogram plot					HISTPLOT

Graphics - draw stat scatter plot			DRW7		SCATRPLOT

Graphics - draw string as bit image		GDISP	->LCD	AGRAPH	"->LCD, ->GRO"

Graphics - erase current picture					"ERASE, RESET"

Graphics - extract graphics images					"REPL, SUB"

Graphics - get display as string		GDISP$	LCD->		LCD->

Graphics - get plot parameters			PPAR

Graphics - label plot					LABEL

Graphics - manipulate plot equation			"STEQ, RCEQ"		"STEQ, RCEQ"

Graphics - pixels to/from user coordinates					"PX->C, C->PX"

Graphics - plot bar chart					BAR

Graphics - plot conic section					CONIC

Graphics - plot function			DRAW		"FUNCTION, DRAW"

Graphics - plot histogram					HISTOGRAM

Graphics - plot in polar form					POLAR

Graphics - plot parametric equations					PARAMETRIC

Graphics - plot scatter chart					SCATTER

Graphics - plot truth area					TRUTH

Graphics - redimension current picture size					PDIM

Graphics - scale plot automatically					AUTO

Graphics - scale stat data			SCL7		SCL7

Graphics - set center of plot display			CENTR		CENTR

Graphics - set default graphics object					PICT

Graphics - set dependent variable					DEPN

Graphics - set independent variable			INDEP		INDEP

Graphics - set intersection of axes			AXES		AXES

Graphics - set pixel to value			PIXEL	PIXEL	"PIXON, PIXOFF"

Graphics - set plot height and width			"*H, *W"		"*H, *W"

"Graphics - set plot minima, maxima"			"PMIN, PMAX"		"PMIN, PMAX"

Graphics - set plot resolution			RES		RES

Graphics - set scale of current picture					SCALE

Graphics - set size of plot					"XRNG, YRNG"

Graphics - superimpose graphics images			"AND,OR,XOR,NOT"		"GXOR, GOR"

Graphics - test pixel value					PIX?

Graphics - view named graphics object					PVIEW

IO - assign loop at power on	AUTOLOOP ON/OFF

IO - assigns device codes	ASSIGN IO	ASSIGN IO			OPENIO

IO - change device code of a device	REASSIGN

IO - check for errors in data transmission	SEND?	STATUS			KERRM

IO - copy files to/from device	COPY CARD	COPY			"SEND, RECV, RECN"

IO - force auto assignment of loop	"ASSIGN LOOP, ASNLOOP$"

IO - get accessory ID as a string	DEVAID$	DEVAID

IO - get address of specified device	DEVADDR	DEVADDR

IO - get contents of specified buffer	BUF$

IO - get device code of a device	DEVNAME$

IO - get device ID as a string	DEVID$	DEVID$

IO - get IO buffer size	"IOSIZE, IOSIZE?"				BUFLEN

IO - get number of devices on loop	ADDRESS

IO - get number of last DDC msg		READDDC

IO - get result of parallel poll	PPOLL

IO - get result of serial poll	"SPOLL, SPOLL$"	SPOLL

IO - get value of interrupt cause byte		READINTR

IO - handle interrupts		"ON INTR, OFF INTR, ENABLE INTR"

IO - list current device codes	"LIST IO, LISTIO$"	LIST IO

IO - lock out local control of devices	LOCAL LOCKOUT	LOCAL LOCKOUT

IO - manipulate HP-IL register	"RIO, WIO"

IO - pass control to another device		PASS CONTROL

IO - receive data from device	ENTER	ENTER			"KGET, SRECV"

IO - reenable ON SRQ after ON SRQ	ENABLE SRQ

IO - reset all devices	CLEAR LOOP	"CLEAR LOOP, RESET HPIL"

IO - reset specific devices	CLEAR	CLEAR

IO - restore communications	RESTORE IO	RESTORE IO

IO - send commands or data 	"SEND, SENDIO"	SEND			"XMIT, SBRK"

IO - send data to specific device	"OUTPUT, ENTIO$"	OUTPUT

IO - send group execute trigger cmd	TRIGGER	TRIGGER

IO - send Kermit command packet					PKT

IO - service request response handling	"ON SRQ, OFF SRQ"	REQUEST

IO - set baudrate					BAUD

IO - set character format	IMAGE				TRANSIO

IO - set checksum type					CKSM

IO - set controller status		CONTROL OFF/ON			"SERVER, FINISH"

IO - set device control mode	"LOCAL, REMOTE"	"LOCAL, REMOTE"

IO - set parity					PARITY

IO - set timeout	TIMEOUT ON/OFF				STIME

IO - suspend communications	OFF IO	OFF IO			CLOSEIO

Keys - activate user key assignments		USER		"KEYASN, LCLBL"	SF -62

Keys - assign active keyboard	KEYBOARD IS

Keys - assign key equivalent	DEF KEY	"KEY, DEF KEY"		ASSIGN	"ASN, STOKEYS"

Keys - clear all key assignments	PURGE KEYS	PURGE KEYS		CLKEYS	DELKEYS

Keys - get current key definition	DEFKEY$	KEYDEF$			RCLKEYS

Keys - get device code of keyboard	KEYBOARD$

Keys - get next buffered key	"WKEY$, SKEY$"	KEY$	KEY	GETKEY	KEY

Keys - post key event	PUT	PUT

Keys - test if a key is currently down	KEY$	KEYDOWN	KEY		KEY

Keys - user defined ENTER					SF -63

Logical - complement	NOT	NOT	NOT	NOT	NOT

Logical - conjunction	AND	AND	AND	AND	AND

"Logical - disjunction, exclusive"	EXOR	EXOR	XOR	XOR	XOR

"Logical - disjunction, inclusive"	OR	OR	OR	OR	OR

"Logical - flag, clear"		CFLAG	CF	CF	CF

"Logical - flag, set"		SFLAG	SF	SF	SF

"Logical - flag, test for clear"			"FC?, FC?C"	"FC?, FC?C"	"FC?, FC?C"

"Logical - flag, test for set"		FLAG	"FS?, FS?C"	"FS?, FS?C"	"FS?, FS?C"

Logical - relational x equal y	=	=	"==, SAME"	"X=Y?, X=/?"	"==, SAME"

Logical - relational x greater than or equal y	>=	>=	3	"X3Y?, X3/?"	3

Logical - relational x greater than y	>	>	>	"X>Y?, X>/?"	>

Logical - relational x less than or equal y	<=	<=	2	"X2Y?, X2/?"	2

Logical - relational x less than y	<	<	<	"X<Y?, X</?"	<

Logical - relational x not equal y	<>	<>	-	"X-Y?, X-/?"	-

Math - arithmetic	+  -  *  /  ^	+  -  *  /  ^	+  -  *  /  ^	+  -  *  V  y^x	+  -  *  /  ^

"Math - coordinates, display in cylindrical"					"CF -15, SF -16"

"Math - coordinates, display in polar/spherical"				POLAR	"SF -15,-16"

"Math - coordinates, display in rectangular"				RECT	"CF -15,-16"

"Math - coordinates, rectangular <-> polar"	"CRTOP, CPTOR"	"POLAR, RECT"	"R->P, P->R"	"->POL, ->REC"	"R->P, P->R"

"Math - exponential e to x, minus 1"		EXPM1	EXPM	E^X-1	EXPM

Math - exponentials	"EXP, 10^X"	"EXP, 10^X"	"EXP, ALOG"	"E^X, 10^X"	"EXP, ALOG"

Math - exponentiation	^	^	^	Y^X	^

Math - hyperbolic functions	"SINH,COSH,TANH"	"SINH,COSH,TANH"	"SINH,COSH,TANH"	"SINH,COSH,TANH"	"SINH,COSH,TANH"

Math - hyperbolic inverse functions	"ASINH, ACOSH,I"	"ASINH, ACOSH,I"	"ASINH, ACOSH,I"	"ASINH, ACOSH,I"	"ASINH, ACOSH,I"

"Math - integral, error bound"	IBOUND	IBOUND	returned	returned	IERR

"Math - integral, last value"	IVALUE	IVALUE

Math - integrate numerically	INTEGRAL	"INTEGRAL, IVAR"	D	"INTEG, PGMINT"	D

Math - inverse	1/x	1/x	INV	1/x	INV

Math - logarithm any base	LOGA

Math - logarithm base 10 (common)	LOG10	"LOG10, LGT"	LOG	LOG	LOG

Math - logarithm base 2	LOG2	LOG2

Math - logarithm base e (natural)	LOG	"LN, LOG"	LN	LN	LN

Math - logarithm base e of 1+X		LOGP1	LNP1	LN1+X	LNP1

Math - negation	-	-	NEG	+/-	NEG

Math - nth root					XROOT

Math - percent		%	%	%	%

Math - percent change			%CH	%CH	%CH

Math - percent of total			%T		%T

Math - solve for roots of a polynomial	PROOT	PROOT

Math - solve for roots of equation	FNROOT	FNROOT	ROOT	"SOLVE, PGMSLV"	ROOT

Math - solve interactively			Solver		Solver

"Math - solve, last root"	FNGUESS	"FGUESS, FVALUE"

Math - square			SQ	X^2	SQ

Math - square root	SQR	"SQR, SQRT"	C	SQRT	C

Math - summation of finite series					7

"Math - trigonometric, angle of a (x,y) pair"	ANGLE	ANGLE	ARG		ARG

"Math - trigonometric, co-functions"	"CSC,SEC,COT"

"Math - trigonometric, degree mode"	OP. A. DEGREES	DEGREES	DEG	DEG	DEG

"Math - trigonometric, degrees <-> radians"	"RAD, DEG"	"RAD, DEG"	"D->R, R->D"	"->RAD, ->DEG"	"D->R, R->D"

"Math - trigonometric, function inverses"	"ASIN,ACOS,ATN"	"ASIN,ACOS,ATN"	"ASIN,ACOS,ATAN"	"ASIN,ACOS,ATAN"	"ASIN,ACOS,ATAN"

"Math - trigonometric, functions"	"SIN,COS,TAN"	"SIN,COS,TAN"	"SIN,COS,TAN"	"SIN,COS,TAN"	"SIN,COS,TAN"

"Math - trigonometric, gradian mode"				GRAD	GRAD

"Math - trigonometric, radian mode"	OP. A. RADIANS	RADIANS	RAD	RAD	RAD

"Math - unit, convert"			CONVERT		CONVERT

"Math - unit, create"					->UNIT

"Math - unit, factoring"					UFACT

"Math - unit, reduce to basic SI units"					UBASE

"Math - unit, value"					UVAL

Matrix - arithmetic	"+,-,*, CMMULT"	"MAT +, -, *"	"+, -, *, /, SQ, NEG"	"+,-,*,/,SQ,C,SIN,I"	"+, -, *, /, SQ, NEG"

Matrix - array to/from stack as elements			"ARRY->, ->ARRY"		"ARRY->, ->ARRY"

"Matrix - bound, lower"	LBND	"LBND, LBOUND"

"Matrix - bound, upper"	UBND	"UBND, UBOUND"	SIZE		SIZE

Matrix - create constant matrix	"CON, ZER"	"CON, ZER"	CON		CON

Matrix - create identity matrix	"IDN, CIDN"	IDN	IDN		IDN

Matrix - create vector of column totals	CSUM

Matrix - create vector of row totals	RSUM			RSUM

Matrix - define lower bounds of arrays	OPTION BASE	OPTION BASE

Matrix - determinant	"DET, DETL"	"DET, DETL"	DET	DET	DET

Matrix - edit matrix				"EDIT, EDITN"	MatrixWriter

Matrix - increment/decrement row/column				I+  I-  J+  J-  INDEX

Matrix - insert/delete current row of matrix				"INSR, DELR"

Matrix - LU decomposition	LUFACT

Matrix - manipulate matrix elements			"GET, PUT"	"STOEL, RCLEL"	"GET, PUT"

Matrix - manipulate matrix pointers			"GETI, PUTI"	"STOIJ, RCLIJ"	"GETI, PUTI"

Matrix - manipulate submatrices				"GETM, PUTM"

Matrix - matrix inversion	INV	INV	INV	INVRT	INV

Matrix - move by one element				arrow keys	arrow keys

"Matrix - norm, column (one)"	CNORM	CNORM	CNRM		CNRM

"Matrix - norm, Frobenius  (Cx*x )"	FNORM	FNORM	ABS	FNRM	ABS

"Matrix - norm, row (infinity)"	RNORM	RNORM	RNRM	RNRM	RNRM

Matrix - residual			RSD		RSD

Matrix - set matrix editor mode				"GROW, WRAP"

Matrix - solve system of equations	SYS	SYS	/	"/, SIMQ"	/

Matrix - swaps specified indexed rows				R<>R

Matrix - transpose	TRN	TRN	TRN	TRANS	TRN

Matrix - undo editing of last matrix element				OLD

Matrix - unit vector				UVEC

Matrix - vector cross product	CROSS		CROSS	CROSS	CROSS

Matrix - vector dot product	DOT	DOT	DOT	DOT	DOT

Matrix - vectors to/from real numbers					"->V2, ->V3, V->"

Memory - allocate array variables	"DIM, REDIM"	DIM	"[  ], RDM"	NEWMAT	"[  ], RDM"

Memory - allocate complex variables		COMPLEX	"(,)"	COMPLEX	"(,)"

Memory - allocate integer variables	INTEGER	INTEGER	#		#

Memory - allocate list variables			{  }		{  }

Memory - allocate real variables	REAL	REAL	implicit	DIM	implicit

Memory - allocate short complex variables		COMPLEX SHORT

Memory - allocate short real variables	SHORT	SHORT

Memory - allocate string variables	DIM	DIM	""" """	ASTO	"""  "", C n ""  """

Memory - available memory in bytes	MEM	MEM	MEM		MEM

Memory - create new object					NEWOB

Memory - delete all programs and data				CLALL

Memory - delete all variables	CLEAR VARS	DESTROY ALL	CLUSR	CLRG	"CLUSR, CLVAR"

Memory - delete program	PURGE	PURGE	PURGE	CLP	PURGE

Memory - delete variable		DESTROY	PURGE	CLV	PURGE

Memory - get hex address of file in RAM		ADDR$

Memory - get list of objects			VARS		"VARS, TVARS"

Memory - get size and checksum of an object					BYTES

Memory - get type of object		TYPE	TYPE	STR|CPX|MAT|REAL?	"TYPE, VTYPE"

Memory - increment/decrement variable					"INCR, DECR"

Memory - manipulate memory		"POKE, PEEK$"	SYSEVAL		SYSEVAL

Memory - manipulate register block				"SIZE, DIM?"

Memory - manipulate variable	LET	LET	"STO, RCL"	"STO, RCL"	"STO, RCL"

Memory - recall arithmetic				"RCL+,RCL-,I"

Memory - register arithmetic			"STO+,STO-,I"	"STO+,STO-,I"	"STO+,STO-,I"

Memory - swaps x with any other register				X<>

NumTheory - absolute value	ABS	ABS	ABS	ABS	ABS

"NumTheory - combinations, permutations"			"COMB, PERM"	"COMB, PERM"	"COMB, PERM"

NumTheory - exponent of argument		EXPONENT	XPON		XPON

NumTheory - factorial	FACT	FACT	FACT	N!	!

NumTheory - fractional part	FP	FP	FP	FP	FP

NumTheory - gamma function	FACT	GAMMA	FACT	GAMMA	!

NumTheory - IEEE class of argument		CLASS

NumTheory - IEEE default values	DEFAULT ON/OFF	DEFAULT EXTEND

NumTheory - IEEE denormalized numbers		yes

"NumTheory - IEEE exception, divide by zero"		"TRAP, FLAG DVZ"		FS? 24  FS?25

"NumTheory - IEEE exception, inexact op"		"TRAP, FLAG INX"

"NumTheory - IEEE exception, invalid op"		"TRAP, FLAG IVL"		REALRES

"NumTheory - IEEE exception, overflow"		"TRAP, FLAG OVF"	"SF 58, SF 59"	FS? 24  FS?25	"SF -21,-22"

"NumTheory - IEEE exception, underflow"		"TRAP, FLAG UNF"	SF 57		SF -20

NumTheory - IEEE infinities		"1INF, PROJ"

NumTheory - IEEE relational inequality		#

NumTheory - IEEE relational unordered		?

NumTheory - IEEE rounding modes		"NEAR,NEG,POS,ZERO"

NumTheory - IEEE signaling & diagnostic NaNs		"NAN, NAN$"

NumTheory - IEEE signed zeros		"-0, +0"

NumTheory - integer division	"\, DIV"	DIV		BASEV

NumTheory - integer part (truncate)	IP	IP	IP	IP	IP

"NumTheory - integer, largest 2 x"	"FLOOR, INT"	"FLOOR, INT"	FLOOR		FLOOR

"NumTheory - integer, smallest 3 x"	CEIL	CEIL	CEIL		CEIL

NumTheory - mantissa of argument			MANT		MANT

NumTheory - maximum/minimum of x and y	"MAX, MIN"	"MAX, MIN"	"MAX, MIN"		"MAX, MIN"

"NumTheory - number, largest machine"	INF	MAXREAL	MAXR		MAXR

"NumTheory - number, nearest machine"		NEIGHBOR

"NumTheory - number, smallest denormalized"		MINREAL

"NumTheory - number, smallest machine"	EPS	EPS	MINR		MINR

NumTheory - power of 10 scaling		SCALE10

NumTheory - round to integer		IROUND

NumTheory - round to nth digit	ROUND		RND	RND	RND

NumTheory - sign of a number	SGN	SGN	SIGN	SIGN	SIGN

NumTheory - truncate at nth digit	TRUNCATE				TRUNC

NumTheory - x-y*Floor(x/y) = modulo	MOD	MOD	MOD	MOD	MOD

NumTheory - x-y*IP(x/y) = remainder	RMD	RMD

NumTheory - x-y*Nearest(x/y) = reduction		RED

NumTheory - 9	PI	PI	9	PI	9

Printing - advances by a blank line			CR	ADV	CR

Printing - get current print line width	PWIDTH?

Printing - get current printer devices	PRINTER$

Printing - print all programs and variables				PRUSR

Printing - print entire display as graphics			PRLCD	PRLCD	PRLCD

Printing - print keystrokes and results			SF 32	TRACE

Printing - print modes			PRMD

Printing - print record of keystrokes				NORM

Printing - print specified program	PLIST	PLIST		PRP

Printing - print stack			"PRST, PRSTC"	PRSTK	"PRST, PRSTC"

Printing - print string	PRINT USING	PRINT USING	PRVAR	PRA	PRVAR

Printing - print variable	PRINT	PRINT	PRVAR	"PRV, PR7"	PRVAR

Printing - prints x			PR1	PRX	PR1

Printing - set endline character sequence	ENDLINE				SF -38

Printing - set line length	PWIDTH	PWIDTH			PRTPAR

Printing - set line spacing (single/double)					Flag -37

Printing - set manual print mode				MAN

Printing - set printer devices	PRINTER IS	PRINTER IS			Flag -34

Printing - set time between lines			Flag 52	DELAY	DELAY

Printing - toggle printing				"PROFF, PRON"

Stack - clear stack			CLEAR	"CLX, CLST"	CLEAR

Stack - counts objects on stack			DEPTH		DEPTH

Stack - drop (roll) stack down			"ROLLD, DROP"	RDN	"ROLLD, DROP"

Stack - drops objects off stack			"DROP2, DROPN"		"DROP2, DROPN"

Stack - duplicates objects on stack			"DUP, DUP2, DUPN"	ENTER	"DUP, DUP2, DUPN"

Stack - get element from stack			"OVER, PICK"		"OVER, PICK"

Stack - return last stack argument	RES	RES	"LAST, LASTARG"	LASTX	"LAST, LASTARG"

Stack - roll stack up			"ROLL, ROT"	R^	"ROLL, ROT"

Stack - swaps x and y registers			SWAP	X<>Y	SWAP

Stats - clear statistics registers		CLSTAT	CL7	CL7	CL7

Stats - correlation coefficient		CORR	CORR	CORR	CORR

"Stats - curve fit model, best"				BEST	BESTFIT

"Stats - curve fit model, exponential"				EXPF	EXPFIT

"Stats - curve fit model, linear"		LR	LR	LINF	LINFIT

"Stats - curve fit model, logarithmic"				LOGF	LOGFIT

"Stats - curve fit model, power"				PWRF	PWRFIT

"Stats - distribution, upper Chi-Square"			UTPC		UTPC

"Stats - distribution, upper Normal"			UTPN		UTPN

"Stats - distribution, upper SnedecorUs F"			UTPF		UTPF

"Stats - distribution, upper StudentUs t"			UTPT		UTPT

Stats - forecast x given y				FCSTX	PREDX

Stats - forecast y given x		PREDV	PREDV	FCSTY	"PREDY, PREDV"

Stats - line of best fit through 7DAT					7LINE

Stats - linear regression			LR	"SLOPE, YINT"	LR

Stats - mean		MEAN	MEAN	MEAN	MEAN

Stats - minimum/maximum absolute value	"MINAB, MAXAB"

Stats - minimum/maximum element	"AMIN, AMAX"		"MIN7, MAX7"		"MIN7, MAX7"

Stats - random number	RND	RND	RAND	RAN	RAND

Stats - set random generator seed	RANDOMIZE	RANDOMIZE	RDZ	SEED	RDZ

Stats - set stat columns			COL7	"ALL7, LIN7"	"XCOL,YCOL,COL7"

Stats - set statistics array		STAT	"STO7, RCL7"	"7REG, 7REG?"	"STO7, RCL7"

Stats - sort stat matrix into bins					BINS

Stats - standard deviation		SDEV	SDEV	SDEV	SDEV

Stats - sum	"SUM, ABSUM"	TOTAL	TOT	SUM	"TOT, 7X, 7X^2, I"

Stats - summation & deletion		"ADD, DROP"	"7+, 7-"	"7+, 7-"	"7+, 7-"

"Stats - variance, covariance"			"VAR, COV"		"VAR, COV"

Stats - weighted mean				WMEAN

Strings - append integer part to string				AIP

Strings - ASCII character to number	NUM	NUM	NUM	ATOX	NUM

Strings - change case of string	"UPRC$, LWRC$"	UPRC$

Strings - concatenation	&	&	+		+

Strings - escape sequences	ESC$

Strings - fill a string with a char	FILL$

Strings - find and replace in a string	REPL$				REPL

Strings - find next char in list	BREAK

Strings - find pos of 1st char not in list	SPAN

Strings - find substring in string w/wildcard	FIND

Strings - get portion of string	"LEFT$, MID$, RIGHT$"

Strings - get position of substring	POS	POS	POS	POSA	POS

Strings - get string of chars in range	ASCII$

Strings - get substring chars a to b	SUB$		SUB		SUB

Strings - length of string	LEN	LEN	SIZE	ALENG	SIZE

Strings - number to ASCII character	CHR$	CHR$	CHR	XTOA	CHR

Strings - number to string	STR$	STR$	->STR	ARCL	->STR

Strings - repeat string n times	RPT$

Strings - reverse string	REV$

Strings - rotate string by char count	ROT$			AROT

Strings - shift string by char count				ASHF

Strings - string to number (value)	VAL	VAL	STR->		STR->

Strings - translate chars in a string	MAP$

Strings - trim specified chars from ends	"LTRIM$, RTRIM$"

Strings - underscoring	"USCR$, NSCR$"

Symbolic - algebraic equations			'a=b'		'a=b'

Symbolic - bind value to local variable					|    (where)

Symbolic - collects terms of an algebraic			COLCT		COLCT

Symbolic - concatenate lists			+		+

Symbolic - decimal approximation -> ratio					"->Q, ->Q9, RATIO"

Symbolic - differentiation			6		6

Symbolic - evaluates an object			EVAL		EVAL

Symbolic - expands an algebraic			EXPAN		EXPAN

Symbolic - expression substitute			"EXSUB, OBSUB"		MATCH

Symbolic - find object in a list			POS		POS

Symbolic - forces numeric evaluation			->NUM		->NUM

Symbolic - get an algebraic subexpression			"EXGET, OBGET"		OBJ->

Symbolic - get unevaluated expressions					"APPLY, QUOTE"

Symbolic - integration			D  (only simple poly)		D

Symbolic - interactive equation editor			FORM		EquationWriter

Symbolic - isolate expression			ISOL		ISOL

Symbolic - list <-> stack			"LIST->, ->LIST"		"LIST->, ->LIST"

Symbolic - manipulate list elements			"PUT,GET,PUTI,I"		"PUT,GET,PUTI,I"

Symbolic - numeric constants			"9,e,i,MAXR,MINR"		"9,e,i,MAXR,MINR"

Symbolic - quadratic formula			QUAD		QUAD

Symbolic - separate sides of an equation					EQ->

Symbolic - show variable			SHOW		SHOW

Symbolic - tag an object with a name					"->TAG, DTAG"

Symbolic - Taylor series approximation			TAYLR		TAYLR

System - automatic run of programs	SHELL ON/OFF

System - beep at a specific frequency	BEEP	BEEP	BEEP	"BEEP, TONE"	BEEP

System - cause a specific error	DO ERROR				DOERR

System - change insert replace mode	ESC-I/R ON OFF

System - comments	"REM, !"	"REM, !"

System - debug flow	TRACE FLOW	TRACE FLOW

System - debug message	USERMSG	MSG$

System - debug off	TRACE OFF	TRACE OFF

System - debug variables	TRACE VARS	TRACE VARS

System - get version of operating system	VER$	VER$	#10d SYSEVAL

"System - last error, clear"					ERR0

"System - last error, msg string"		ERRM$	ERRM		ERRM

"System - last error, number"	ERRN	ERRN	ERRN		ERRN

System - line number of last error	ERRL	ERRL

System - manipulate system flags	STATUS	FLAG	"STOF, RCLF"		"STOF, RCLF"

System - set password	LOCK	LOCK

System - shutdown	BYE	"OFF, BYE"		OFF	OFF

System - sound on/off	BEEP ON/OFF	BEEP ON/OFF	SF 51 for quiet	"QUIET, CF 26"	"SF -57, SF -57"

System - standby (continuous on)	STANDBY ON/OFF	STANDBY		"ON, SF 44"

System - startup sequence		STARTUP		SF 11

Time - acknowledge past-due alarms					"ACK, ACKALL"

Time - alarms on/off	ALARM ON/OFF

Time - clear time adjustment	RESET	RESET CLOCK

Time - clock continuously displayed					SF -40

Time - convert date & time to string					TSTR

Time - date & time as numbers	"DATE, TIME"	"DATE, TIME"			"DATE, TIME"

Time - date & time as strings	"DATE$, TIME$"	"DATE$, TIME$"

Time - date arithmetic					"DATE+, DDAYS"

"Time - decimal hours <-> hours,minutes,sec."			"HMS->, ->HMS"	"HMS->, ->HR"	"HMS->, ->HMS"

Time - delete appointment					DELALARM

Time - find appointment					FINDALARM

Time - get appointment					RCLALARM

Time - get current clock adjust factor	ADJUST$	AF

"Time - hour,minute,second arithmetic"			"HMS+, HMS-"	"HMS+, HMS-"	"HMS+, HMS-"

Time - manipulate specific timers	ON|OFF TIMER#	ON|OFF TIMER#

Time - return current timer interval	TIMER?

Time - set adjust factor for clock	ADJUST	ADJUST			CLKADJ

Time - set appointment	APPT				STOALARM

Time - set system date & time	SET	"SETDATE, SETTIME"			"->DATE, ->TIME"

Time - set time as exact	EXACT	EXACT

Time - system ticks (1/8192 of a sec)					TICKS

Binary	15	5	14	11	14

Complex	17	13	18	16	18

Control	16	17	16	8	18

Display	14	14	12	16	15

Files	39	32	3	1	15

Graphics	0	3	18	2	42

IO	31	24	0	0	14

Keys	8	7	2	4	7

Logical	10	13	14	14	14

Math	26	27	28	27	38

Matrix	18	14	17	21	20

Memory	9	15	15	14	18

Number Theory	18	35	19	14	20

Printing	8	5	9	13	11

Stack	1	1	9	6	9

Stats	5	11	19	19	26

Strings	23	8	8	8	9

Symbolic	0	0	19	0	24

System	17	16	6	5	8

Time	11	8	2	2	14

TOTAL:	286	268	248	201	354

Dan Allen