I neglected to say in the ROM bug listing how to tell the ROM revision.

Here's how:

1. Turn the calculator on (press ON and then release).

2. Press ON again, and while holding it down, hit the "D"

key (the fourth white key in the top row) and then release

the ON key.  You should get three vertical lines in the

display, one down the middle.

3. Hit the backspace key (it says DROP and CLR over it).

You should get some meaningless hexadecimal in the first

line of the display.  (Mine says 705D9:1B8D...).

4. Press and hold down the EVAL key.  It should say:

Version HP48-*

Copyright HP 1989

where the * is an uppercase letter (A, B, etc.) indicating

the ROM version.

5. Release the EVAL key.  Hold down ON and hit "C" (the third

white key in the top row.  This is a general reset

operation that returns the calculator to normal from the

diagnostic stuff (which is where ON-D puts you).

I'd be very interested if anyone gets anything but revision A.

Mark Adler

madler@hamlet.caltech.edu