From Usenet:

17 Sep 91 17:09 Steve Harper: Beware WorldPort?

Unfortunately, providing a modem solution for the HP 95LX

has not been as simple as we had hoped.  Here is a repeat

of my posting of several months ago as well as some new

information:

Most portable modems do not have an on/off switch; rather

they rely upon the DTR line from the terminal device to

turn them on or off.  Normally, DTR will be low when the

serial port is unused (modem off) and will go high when

the serial comm program is run (modem on).  This conserves

the modem battery (a precious resource) as much as possible.

For various reasons (power, size, cost), the HP 95 only has

a "three-wire" (TX, RX, GND) serial interface.  There is no

DTR line to turn the modem on.  One way to force the modem

to turn on is to use the modem ac adapter.  ANY OF THE

WORLDPORT MODEMS WILL WORK WITH THE HP 95 PROVIDED THE

MODEM IS ON AC POWER.  This work-around is hardly a

complete solution, however.

Fortunately, the Worldport modems have a mode which changes

the power-on signal from DTR high to RX low (at the time we

started looking at modems for the HP 95, the Worldports were

the only ones with this feature).  That is, the modem will

turn on when the RX line goes low.  This seemed ideal for the

HP 95 since the RX line is held at ground normally (modem off)

and is driven low when an application which uses the serial

port is run (modem on), such as COMM.

This happens to work just fine for the Wordport 1200, but for

the 2400 and 2400MNP, when data is sent on the RX line, the

modem power tries to turn off and on with the data bits, rather

than remaining on.  As you can imagine, this didn't work (this

mode was really not originally designed for this purpose).  But,

not to worry, they were more than happy to make a minor change

in the design to fix that problem.  Hence, the business about

"only modems after serial number such-and-such being able to

work on battery power" came into being.

Information we have received just the other day says that the

2400 has indeed been changed and works as advertized.  The 1200

never really had the problem, so it always worked.  However,

it appears that Touchbase/USRobotics never put the change into

production for the 2400MNP.  So, THE WORLDPORT 2400MNP WILL NOT

WORK ON BATTERY POWER WITH THE HP 95, REGARDLESS OF THE SERIAL

NUMBER.  Their customer support was apparently indicating to

some people that by putting the 2400MNP into "constant speed"

mode, it would work.  This is not true.  It appears to work

until you connect to the phone line.  At that point, the added

current drain drags the power supply down and it either hangs

up or transmits garbage.

In addition to the power-on mode setting (dip switch number 3

ON), there is one additional setting which must be used to get

the modems to work with the HP 95.  All modems monitor the DTR

line as an indication that there is a computer or terminal

there to receive the data.  If DTR is disconnected or low, they

assume the terminal isn't there or has failed, and will usually

hang up and stop listening for commands.  All modems have a

mode which tells them to ignore the DTR line, but it is generally

not the default, so the user must set this mode.

For the 1200, dip switch 4 must be ON to activate this mode (the

manual is totally confused, don't believe it).  For the 2400 and

the 2400MNP, the user should send the modem command AT&D&W<cr>.

This only needs to be done once: it sets the DTR ignore mode and

writes that mode into the EEPROM so it will always come up in

that mode thereafter.  THE DTR IGNORE MODE MUST BE SET IN ANY

MODEM IN ORDER TO WORK WITH THE HP 95, REGARDLESS WHETHER THE

MODEM IS ON BATTERY OR AC POWER.

So, to summarize:

1. In order to work with the HP 95, all modems must be set to

DTR IGNORE mode.

2. All the Worldport modems will work with the HP 95, provided

the modem is on ac power.

3. The Worldport 1200 (any serial number) and the Worldport 2400

(serial number 50000 and up) will work on battery power with the

HP 95 if the RX power on mode is set.

4. The Worldport 2400MNP will not work on battery power with the

HP 95.

Hope this helps....Steve