PRO AUDIOSPECTRUM'S

MOST COMMONLY-ASKED

QUESTIONS AND ANSWERS

January 25, 1992

THE PRO AUDIOSPECTRUM PACKAGE CONTAINS:

Media Vision Pro AudioSpectrum Audio Adapter Board

Pro AudioSpectrum User's Guide

Pro AudioSpectrum Disk 1 & Disk 2 v1.1 on

5 1/4" High Density and 3 1/2" Double Density

The bundled software consist of:

Installation Software and Diagnostic

AudioMate & Utilities

SP Spectrum MIDI Sequencer

ABBREVIATIONS:

PAS       = Media Vision Pro AudioSpectrum

TB        = Media Vision Thunder Board

AdLib     = Adlib sound card

SB        = Creative Labs Sound Blaster sound card

MPU-401   = Roland's proprietary MIDI interface protocol

BBS       = Bulletin Board System

MWINDOWS  = Windows 3.00a with Multimedia Extensions 1.0

RMA       = Return Material Authorization number

Generic CONFIG.SYS:

DEVICE = C:\MWINDOWS\HIMEM.SYS

FILES = 30

BUFFERS = 10

LASTDRIVE = X       (Refer to Question 27 below)

DEVICE = C:\DOS\SETVER.EXE    (Refer to Question 25 below)

DEVICE = C:\PROAUDIO\TSLCDR.SYS /D:MVCD001 /R

DEVICE = C:\PROAUDIO\MVSOUND.SYS D:X Q:X V:X (Refer to Question 2

below)

DEVICE = C:\MWINDOWS\SMARTDRV.SYS 2048 512

Generic AUTOEXEC.BAT:

@ECHO OFF

PATH = C:\;C:\DOS;C:\MWINDOWS;C:\PROAUDIO

PROMPT $P$G

C:\PROAUDIO\MSCDEX.EXE /D:MVCD001 /M:12 /L:X /V

(X is the drive letter assigned to the CD-ROM drive)

The current version for TSLCDR.SYS is v1.31e and for MSCDEX.EXE is v2.2.

If you have an older version, please contact Media Vision Technical Support

for updated drivers.  When calling, please provide us with the version

number which you currently have.

SOUND TYPES:

Digital Audio     = Recorded sounds, usually speech and sound effects.

These sounds are played back by the Digital-To-

Analog Converter (DAC) on the PAS.  Designated by

the ".WAV" extension.

Synthesized Audio = FM sounds, usually music but can also be used to

produce speech and sound effects.  These sounds

are played back by the FM Synthesizers on the PAS.

Designated by the ".MID" extension.

CD Audio          = Sounds produced by the CD-ROM drive which can be

heard when connecting the headphones directly to

the CD-ROM drive.  Also known as Red Book Audio.

PC Speaker        = Crude music tones, speech and sound effects

produced by the System Timer Chip on the

motherboard.  These sounds are played back by the

internal PC Speaker.

INTERNAL CD-ROM DRIVES SUPPORTED BY PAS:

Chinon              CDS-431.  Please contact Chinon at 310-533-0274 to get

the 40-pin to 50-pin converter.

Hitachi             CDR-3650

NEC                 CDR-80, CDR-82, CDR-83

Sony                CDU-541

Texel               DM-3020, DM-3021

Toshiba             XM-3201B

EXTERNAL CD-ROM DRIVES SUPPORTED BY PAS:

Apple               CD-SC (Sony CDU-8001)

CD Technologies     Porta-Drive

Chinon              CDX-431

Denon               DRD-253

Hitachi             CDR-1553S, CDR-1650, CDR-1750S

IBM                 3510

Laser Magnetics     CM212, CM214, CM231

NEC                 CDR-35, CDR-36, CDR-72, CDR-73, CDR-77

Panasonic           CR-501-S

Pioneer             DRM-600 6-disc changer

Sony                CDU-6211

Texel               DM-3010, DM-3120, DM-5020, DM-5021, DM-5120, DM-7120A,

DM-7120B

Toshiba             XM-3201A, XM-3301, XM-5100A

If your CD-ROM drive is not on this list, it will NOT work with the PAS.

If it is on this list and you would like to use the Pro AudioSpectrum's

SCSI interface to control the CD-ROM drive, please contact Media Vision

Sales at 1-800-845-5870 to order an Internal SCSI Cable/Driver Kit for

$39.00 or an External SCSI Cable/Driver Kit for $79.00.

PRO AUDIOSPECTRUM USES THE FOLLOWING:

DMA 1 at default.  Selectable between 1, 2 and 3.

IRQ 7 at default.  Selectable between 3, 5, 6 and 7.

And all of the following I/O port addresses which are fixed:

Speaker             042h      Speaker Timer Count*

043h      Speaker Timer Control*

061h      Speaker Control*

Joystick*           201h      Joystick Status

FM Synthesizer      388h      Left FM Synthesizer Address

389h      Left FM Synthesizer Data

38Ah      Right FM Synthesizer Address

38Bh      Right FM Synthesizer Data

Interrupt Ports     B89h      Interrupt Control

B8Ah      Audio Filter Control

B8Bh      Interrupt Control Register

PCM Circuitry       F88h      PCM Data

F89h      Reserved

F8Ah      Cross Channel Control

1388h     Sample Rate Timer

1389h     Sample Buffer Count

138Ah     Local Speaker Timer Count

138Bh     Local Speaker Timer Control

SCSI Interface      1F88-1F8B SCSI Register

3F88-3F8B SCSI Register

5F88-5F8B SCSI Data Register

*Located on PC system board

NOTE:  If you questions regarding Windows 3.00a with Multimedia Extensions

1.0, please refer to the MPC Upgrade Kit's Most Commonly-Asked Questions

and Answers.

QUESTIONS & ANSWERS:

*******************************INSTALLATION********************************

1.   Q:   Do I have to change any switches or jumpers before I install the

PAS?

A:   The default settings will work in most computers.  Only try with

different DMA and IRQ settings if you experience any problems.

Whenever changing the settings on the PAS, make sure you make the

same changes in the MVSOUND.SYS statement, in CONFIG.SYS.

2.   Q:   What is the MVSOUND.SYS driver for?

A:   You must have "DEVICE = C:\PROAUDIO\MVSOUND.SYS D:X Q:X V:X" in

the CONFIG.SYS file.  The first two X's correspond to the DMA

channel and IRQ settings.  The third X in "V:X" represents the

initial volume level, a value between 0 and 40, where 40 is the

maximum volume level.  This driver resets the mixers and the

volume settings on the PAS.  Without this driver, none of your

Adlib games will play sounds.

3.   Q:   I have tried with all settings and PASTEST.EXE still fails.

A:   First, PASTEST.EXE may need to be run twice in a row.  Second,

reduce the system to a generic CONFIG.SYS and AUTOEXEC.BAT to

eliminate possible software conflicts.  Third, remove all

unnecessary peripheral boards (scanner, tape controller, network

cards, etc.) in the system to eliminate possible hardware

conflicts.  Fourth, in most computers, it will accurately report

if there are conflicts; however, in some 486/33 computers, it may

not be accurate.  If you have already tried everything and

PASTEST still fails, go on and run AudioLnk.

4.   Q:   After I have installed the PAS and connected the PC Speaker cable

to the PAS, I do not hear any PC Speaker sounds in my 486/33.

A:   Some 486/33 computers do not allow the PAS to implement PC

Speaker sounds.  Therefore, leave the PC Speaker cable on the

motherboard so that these sounds will be produced by the

motherboard and played through the internal PC Speaker.

*********************************AUDIOLNK**********************************

5.   Q:   a.   When I run AudioLnk to play MIDI, it does not produce any

sound.

b.   When I run AudioLnk to play WAV, it does not produce any

sound.

c.   When I run AudioLnk to play WAV, it plays the same sound

over and over like a broken record.

A:   a.   If MIDI does not play, it is a possible IRQ conflict.

b.   If WAV does not play, it is a possible DMA conflict.

c.   If WAV plays like a broken record, it is a possible IRQ

conflict.

First, you must have the sound drivers (TAPIMV/VMP/AUDIOMAT)

loaded prior to running AudioLnk.  Refer to the AudioMate Section

in the PAS User's Guide.  Second, select a different DMA/IRQ and

try again.  Whenever changing the settings on the PAS, make sure

you make the same changes in the MVSOUND.SYS statement, in

CONFIG.SYS.  Third, reduce the computer's bus (I/O CLK) speed to

10MHz or less and try again.

6.   Q:   When I run AudioLnk to play CD-Audio, the CD-ROM drive's light

does not come on and I do not hear any sound.

A:   You may need to press F3 twice in a row.

7.   Q:   There is a lot of noise in the microphone recordings.

A:   First, make sure that the microphone has 600-ohm impedance (or

higher).  Second, mount the microphone on a stand instead of

holding in your hand.  Third, use headphones instead of external

speakers/amplifiers to eliminate the possibility of feedback

which can cause distortion or noise.

8.   Q:   Running PAS.EXE to adjust the mixer and volume settings before

going into AudioLnk does not make any difference at all.

A:   First, AudioLnk ignores the settings made in PAS.EXE.  Second,

once inside AudioLnk you can adjust the mixer and volume settings

by pressing F4 to activate the Recording Menu and then F9 to

access the Mixer Control Panel.

********************************SP SPECTRUM********************************

9.   Q:   When I bring up Sp Spectrum (by running SEQ.BAT), TAPIMV.COM

reports that it cannot find the interrupt.

A:   Edit the SEQ.BAT to add the "/IRQ:X" switch to the end of the

TAPIMV.COM statement, where X is the IRQ setting, and try again.

10.  Q:   When I run Sp Spectrum, I do not hear any sound when it is

supposed to be playing.

A:   First, make sure the port in Sp Spectrum is set to 2 to use the

FM synthesizer chips on the PAS.  Port 1 is for playing through

an external MIDI equipment.  Second, select a different IRQ and

try again.  Whenever changing the settings on the PAS, make sure

you make the same changes in the MVSOUND.SYS statement, in

CONFIG.SYS.  Third, reduce the computer's bus (I/O CLK) speed to

10MHz or less and try again.

11.  Q:   Page 18 of the Sp Spectrum Section in the PAS User's Guide refers

to the Tutorial file which I cannot find.

A:   The current version of the PAS disks has this file.  Please send

in your PAS disks to the attention of Customer Support.  Include

your name, address, phone number and a note to request for an

update.

********************MPU-401 AND SOUND BLASTER EMULATION********************

12.  Q:   a.   Does the PAS support MPU-401?

b.   Does the PAS support SB?

A:   a.   No.  The MIDI Interface chip on the PAS is the Yamaha 3802

MIDI controller and not the Roland MPU-401 MIDI Interface

chip.

b.   No.  The method used on the PAS to record and play back is

different on the existing SB card.

Note:     Media Vision has developed an emulation program which

enables the PAS to emulate the MPU-401 in "DUMB mode" and

the SB to work with most SB games.  Please note that this

program requires a 386SX or better and will work in DOS and

Windows 3.0.  This emulation program is called SB2MV.ZIP and

is located in the ZIP files area of this BBS.

*********************************GAMES*************************************

13.  Q:   My PAS is set to DMA 3 and IRQ 5.  I hear music but I don't hear

any sounds when running Sierra games that support the PAS.

A:   The PAS driver in your Sierra games require that the PAS is set

for DMA 1.  Some Sierra games also require IRQ 7.  Sierra is

working on updating the PAS driver to allow DMA 1/2/3 and IRQ

3/5/6/7 to be selected.  Please contact Sierra for the update.

14.  Q:   I have games/software that provide drivers for the SB and AdLib.

Which driver should I select?

A:   The PAS is 100 percent AdLib compatible and will work with any

package that provides driver for AdLib.  If the package does not

support digital audio, then the AdLib driver or SB driver can be

selected.  Otherwise, if the package does support digital audio

with the SB driver, please refer to the "MPU-401 and Sound

Blaster Emulation" section above.

15.  Q:   Does the PAS make my game sound better than my AdLib board?

A:   PAS.EXE is a utility that allows you to adjust the mixer, volume

settings, bass, treble, loudness, enhanced stereo (will make a

mono signal simulate stereo), and realsound (to improve PC

speaker sounds).

16.  Q:   Sometimes when I play a game that supports AdLib, I only hear

sound out of one speaker.

A:   There are two ways to run PAS.EXE.  The first way is to run "PAS

RESET" to reset the mixer.  The second way is "PAS *" to bring up

the Mixer Control Panel on screen.  Press F1 to get help, then F4

to reset the mixer.

17.  Q:   The joystick does not work at all or does not work properly when

using the PAS's joystick port.

A:   First, some computers have a built-in joystick port on the

motherboard.  Other computers have a multi-I/O board that has a

joystick port.  Make sure that only one joystick port is enabled

and the other one is disabled.  Second, if the computer is too

fast that may also cause a problem.  Reduce the computer's BUS

(I/O CLK) speed and try again.  Refer to your computer user's

guide for instructions on how to adjust the BUS (I/O CLK) speed.

Third, contact the game manufacturer to ask for a joystick driver

that works on fast machines.

*****************************SCSI INTERFACE********************************

18.  Q:   Which CD-ROM drives can I use with the PAS?

A:   See CD-ROM Drives Support List above.  The PAS has a SCSI

interface which only allows it to control SCSI devices.  Please

contact Media Vision Sales for the SCSI Cable/Driver Kit

(Internal/External).

19.  Q:   Do I have to use the PAS SCSI interface to control my CD-ROM

drive which has its own controller?

A:   If your CD-ROM drive is SCSI, you can use its own controller or

the PAS SCSI interface.

20.  Q:   Can I use the PAS to control other SCSI devices?

A:   Yes.  Trantor Systems is currently selling SCSI drivers for the

PAS for a variety of SCSI devices (hard disks, removable media,

optical, etc.).  Please contact Trantor Systems for the PAS

driver set at (510) 770-1400.  The price for the driver set is

$59.00.

21.  Q:   Will the SCSI interface on the PAS conflict with another SCSI

controller if they are both installed in the same computer?

A:   They should work fine together.  Make sure that the drive ID's

for the SCSI devices are different.

************************CD-ROM DRIVE INSTALLATION**************************

22.  Q:   I get the error "No SCSI host adapter found".

A:   First, the SCSI CD-ROM driver is board dependent.  If you would

like to use the PAS to control the CD-ROM drive, you need to use

our TSLCDR.SYS driver; otherwise, if you would like to use

another SCSI controller (such as Adaptec or Future Domain) to

control the CD-ROM drive, you need to use its driver (contact the

SCSI controller manufacturer for the driver).  Second, check the

internal, flat ribbon SCSI cable to make sure it is properly

connected to the PAS.  Third, make sure the red painted edge of

the cable is closest to the 4-pin connector on the PAS at J6.

Fourth, make sure the terminator is connected securely to the

back of the CD-ROM drive.  Try both with and without the

terminator.  Fifth, reduce the system to a generic CONFIG.SYS and

AUTOEXEC.BAT.  Sixth, reduce the computer's bus (I/O CLK) speed

to 10MHz or less.  Seventh, if there is a co-resident Thunder

Board, make sure Switch 4 (FM synthesizer switch) on the TB is

set of OFF.

23.  Q:   I get the error "No devices respond on this SCSI host adapter".

A:   First, check the internal, flat ribbon SCSI cable to make sure it

is properly connected to the back of the CD-ROM drive.  Second,

make sure the terminator is connected securely to the back of the

CD-ROM drive.  Try both with and without the terminator.  Third,

make sure you have inserted the computer's power plug into the

CD-ROM drive connector and pushed it firmly into place.

24.  Q:   I get the error "Device driver not found: MVCD001.  No valid CD-

ROM selected".

A:   First, if you are using DR DOS 6.0, it is necessary to change the

device name for both the TSLCDR.SYS and MSCDEX.EXE statements

from "MVCD001" to "TSLCD".  Second, make sure that the device

names in both of these statements match.

25.  Q:   MSCDEX.EXE issues the error "Incorrect DOS version".

A:   If you are using MS DOS 5.0, MSCDEX.EXE v2.2 requires that

CONFIG.SYS has the statement "DEVICE = C:\DOS\SETVER.EXE".  We

will make an announcement in the BBS Newsletter when the new

version of MSCDEX.EXE is available, which will not require

SETVER.EXE when using MS DOS 5.0.

26.  Q:   I cannot use the CD-ROM drive.

A:   First, refer to the questions listed above.  Second, make sure

the MSCDEX.EXE statement precedes any menu, DOS Shell, PCTOOLs,

or any desktop manager statements in AUTOEXEC.BAT to ensure that

the driver has been properly loaded.

27.  Q:   I get the error "Invalid drive specification" when I type the

drive letter assigned to the CD-ROM drive.

A:   First, make sure you have "LASTDRIVE = X" in the CONFIG.SYS,

where X is the next logical drive letter after the letter

assigned to the CD-ROM drive in the MSCDEX.EXE statement.

Second, make sure that the TSLCDR.SYS driver statement precedes

any driver statement that creates a RAM drive such as

RAMDRIVE.SYS.  Third, check if the TSLCDR.SYS and MSCDEX.EXE

drivers have been loaded up properly.  If not, refer to the

questions above.

28.  Q:   I get the error "CDR 101: Drive not ready".

A:   First, check the internal, flat ribbon SCSI cable to make sure it

is properly connected.  Second, make sure the terminator is

connected securely to the back of the CD-ROM drive.  Try both

with and without the terminator.

29.  Q:   a.   I get the error "CDR 103: CD-ROM not High-Sierra or ISO-9660

Format Reading Drive X.  Abort, Retry or Ignore?"

b.   My CD-ROM drive fails at random.

A:   First, check if the LASTDRIVE statement and TSLCDR.SYS driver are

loaded before a hard disk driver and/or a memory manager.  If so,

load the hard disk driver and/or memory manager first.  Second,

refer to the question above.

******************************MISCELLANEOUS*******************************

30.  Q:   Do you have a software development toolkit for the PAS?

A:   Please contact Media Vision Sales at 1-800-845-5870 to order the

Pro AudioSpectrum Developer's Toolkit for $79.95.  This toolkit

contains a 3 1/2" disk which contains drivers, utilities, source

codes for both DOS and MWINDOWS and a manual which contains a

software reference and a hardware reference.

31.  Q:   I have questions regarding MWINDOWS.

A:   Please refer to the MPC Upgrade Kit's Most Commonly-Asked

Questions and Answers.

Before returning a defective product for repair, please call to request an

RMA number first.  Please contact Media Vision Technical Support at 1-800-

638-2807 if you have any questions or would like an update to this list.

Media Vision Sales can be reached at 1-800-845-5870.

pasq&a.doc