10/10/95

Documentation for the image file chip_a4.jpg

Copyright notice:

Permission is granted to copy, redistribute, etc., the images

in this file, provided that no profit is made from doing so,

and that the original author/photographer, Chip Atkinson, is

acknowledged.

Description:

The top image shows my coil in operation.  The sparks are

emanating from the left side, with one spark hitting the

strike rail above the primary winding.  Note the faint

corona on the strike rail. The bright vertical

streak at the bottom of the screen is a jet from the spark

gap.  This gap consists of one gap with an air blast (~20psi)

to blow out the spark.

The bottom image shows the same coil in operation, with

a few sparks emanating from the toroid, and the majority of the

sparks emanating from the top turn of the winding.

This image shows several interesting coil features.   The first

and most obvious is the sparking from the secondary winding to

the strike rail.  The sparking is facilitated by the height

of the toroid.  You can see that the toroid in the bottom image

is considerably higher than in the image above.

Another feature to note is the corona.  Although faint, one can

see that it extends in a cone with its apex at the top turn of

the secondary.

The specifics of the coil are:

120v in

15000v, 150 ma out to primary

0.0177uF rolled cap, 16 turn primary coil,

6" starting diameter from the center,

16 turns, 3/8" Cu tubing spaced 3/8" apart,

30 degree banking.  Tapped at 12th turn.

950 turns 24ga. magnet wire, close wound.

4" diameter pvc drain pipe, capped on both

ends.  7"x22" toroid, about 5" from top of

secondary coil.  Secondary coil is sealed

with Behr's Build 50 Super Gloss decopauge

(sp?) stuff.  (Two part, epoxy based)

Max sparks so far: 39"

Photography:  ASA 400, f11, auto shutter speed.

Pentax ME Super.  28mm f2.8 lens.  Mid day, garage

window covered. (Hence the light leakage around the

garage door).