BIPOLE2.GIF

This 800x600 scan shows a rough looking Tesla workstation set up

with a bi-polar coil. The rolling table has shelving space below

for fourteen homemade poly-AL-oil capacitors, though only four

capacitors occupy the shelf in this image. Later a small shelf

was added to the side of the table for the spark gaps. This table

is equipped with a massive grounding system and has a built in

counterpoise. Below the table top there is a heavy brass plate.

The brass plate has a short length of one inch ground strap

soldered to it. This ground strap may be passed through a slot in

the table top to make a direct connection with the base of the

Tesla resonator. The heavy brass plate is grounded via four

parallel lengths of #6 cable, one going down each leg of the

table, and connecting to 1/8-inch alumnium plate screwed down

below the capacitor shelf, about one inch from the floor. This

aluminum plate acts as a small counterpoise and gives the

illusion to the coil that true ground is directly beneath it.

The counterpoise is connected with several sections of ground

strap to an electrically isolated hydraulic car lift control just

outside the garage door. This ground is better than the one used

by Tesla in Colorado Springs.

On top of the table to the right sits a cylinder spark gap. Just

left of the gap there is a rubber inner tube and pieces of

plastic and wood for winding temporary primary coils used for

testing and experimentation prior to construction of permanent

coil forms. Solid polyethylene core coax (RG-213) comes in from

the bottom left of this image to bring high voltage from the

power supply up to the cylinder spark gap.

Plastic cord is hung from the rafters to support a five turn

helical primary coil wound from 3/8 inch copper tubing on a

wooden coil form lined with plastic insulation.

The secondary coil is a long skinny "candlestick" design from the

late 1960s or early 1970s. The coil is wound on a plexiglas tube

five inches in diameter using about 650 turns of PVC jacketed

solid core bell wire that wound at about 18 turns per inch. The

actual winding was 36 inches long giving this coil an aspect

ratio of 7.2, and an approx. resonate frequency of 530 kHz.

This coil was a disaster when fired in a 1/4 wave configuration.

The design is inefficent and the coil coupled very poorly to

every primary I tried with it. Having a high resonate frequency

it would not accept much power, and when overdriven the resonate

frequency of the coil would split, with several 1/4 wave voltage

peaks appearing in the winding and sparks jumping out from

everywhere but the discharge terminal.

The coil finally did find use as a half-wave coil. I wrapped the

coil up in a couple of rolls of plastic food wrap to help

insulate, then set it on it's side across two saw horses. The

resultant bi-polar coil proved much more efficient, and opened

the door to wide range of investigations with RF transmission

lines, extra coils, and bi-phase coil systems.

System was designed and built by Richard T. Quick II

10028 Manchester Rd., Suite 253, Glendale, MO  63122  USA