This is a rare scan of Robert Golka's Magnifier coil operated

under the name "Project Tesla".

This is most certainly a Magnifier as you can clearly see the RF

transmission line glowing with corona as it takes current from the

driver system (the primary and secondary coils are out of view to

the left) to the base of the extra coil in the photo center.

This is a photo of the free standing extra coil. The extra coil was

wound on a hollow core made of eleven vertical 4 inch diameter

sections of PVC pipes. The vertical members were supported laterally

by a length of flexible PVC pipe wound in a spiral manner and con-

nected to the vertical supports. The dimensions of the extra coil

were as follows:

Diameter - 8.0 feet, Height - 7.5 feet, Wire Size - 6 AWG, Turns 127

Turn-to-Turn Spacing - 1.00 inches

The resonant frequency of the extra coil was 94.3 kHz. Due to the

building construction, the resonate frequency of the coil changed

with it's position relative to the walls, floor, and ceiling. Fre-

quencies from 73 kHz to 94.7 kHz were obtained by moving the coil.

Project Tesla used two spark gaps, a rotary break and a stationary

quenched gap. The rotary break wheel was 18 inches in diameter with

20 teeth, and looked much like a saw wheel. Horizontal tungsten

electrodes were placed opposite each other on either side of the break

wheel. The electrodes were gapped at 0.125 inches. The break wheel was

driven at 7110 RPM by a 40 hp, 3555 RPM motor. The rotor mechanical

drive used an insulating belt and 2:1 step up pulleys. The entire

rotary assembly was shielded in a 1/4 inch thick steel housing. The

quench gap was placed in series with the rotary break and consisted of

a tungsten electrode separated from a metal base plate by a distance

of 0.125 to .500 inch. The quench gap assembly is housed in a chamber

made from a large ceramic insulator of the type used in power sub-

stations. The quench gap housing was sealed and the gap was pressur-

ized at between 20-60 psi with Sulfur hexafluoride gas (SF6).

The coil was supplied with power from a military surplus 150KW high-

voltage generator. The input voltage to the coil system was 44 KVAC

RMS. Primary capacitance ranged from .177 - .2 microfarads. The primary

inductance was 33 microHenries, the secondary inductance was 9 milli-

Henries, the secondary resonate frequency was 64 kHz. Primary/secondary

coefficient of coupling was .60. The maximum spark length was 105 feet.

Project Tesla was started by Robert Golka in an abandoned hangar rented

from the U.S. Air Force for the sum of $1.00 per year. This hangar near

Wendover Utah is the same location where the first atom bomb dropped

on Japan was pre-assembled prior to it's transport to the island

of Tinian for it's fateful flight. Later, Project Tesla was moved to

the "Oxide Building", Climax, Colorado where the extra coil was fitted

with a 160 foot high retractable mast used to duplicate Tesla experi-

ments in Earth resonance.

Richard Quick